| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0080 |
| --- | --- |

| Title: | HDR coding results for FruitStall and Oblivion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bill Mandel 100 Universal City Plaza Los Angeles, CA 91608 USA Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | bill.mandel@nbcuni.com chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Universal Pictures, Harmonic Inc., Movielabs Inc. |  |  |

_____________________________

# Abstract

Various HEVC HM encoding parameters tuned for the High Dynamic Range (HDR) test content clips “FruitStall” (submitted by Sony at the previous JCT meeting) and Universal Picture’s “Oblivion” are compared in this study. The “PQ” non-linear curve, now documented as SMPTE ST.2084, serves as the EOTF. Both YDzDx (SMPTE ST.2085) and YCC (ITU-R Rec. 2020) are tested at 12-bit precision. Results show that the RD behaviour of both color spaces are very similar.

# Experimental Setup

The steps are nearly identical to the tests conducted in the previous meeting [1][2], with the addition this time of a D65 correction applied to YDzDx and a new test clip FruitStall [3].

## Fruit Stall

In the experiments ran for this study, this new FruitStall clip [3] was converted to 12-bit YDzDx PQ [4] in the similar steps performed for “Oblviion.”

The source file is provided in a format designated for the AdHoc-18 [5] study experiments: a concatenated RGB planar 1920x1080p 24 frame/sec sequence cropped from a larger 4K source. Each 16-bit linear RGB sample in the sequence was exported from the original raw Bayer images recorded from the Sony F65 camera that captured the test clip. Recent RExt HM encoder releases support direct input of the RGB sequence, with a minor reorder of component planes via the ---InputColourSpaceConvert=RGBtoGBR command.

# Results

| Title | Profile | Encoder & settings | Color matrix | EOTF |
| --- | --- | --- | --- | --- |
| FruitStall 1080p | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx | PQ |
| FruitStall | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx with D65 | PQ |
| FruitStall | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | YCC Rec. 2020 | PQ |

Table 1 Summary of conducted experiments

# References

[1] B. Mandel, C. Fogg, J. Helman, “High dynamic range video coding results,” JCTVC-O0101

[2] P. Yin, T. Lu, “Cross-check Report for JCTVC-O0101 on High Dynamic Range Video Coding Results,” JCTVC-O0366.

[3] K. Sharman, N. Saunders, J. Gamei, “AHG5 and AHG18: 16-bit Test Sequences for Range Extensions”, JCTVC-O0069

[4] S. Miller, M. Nezamabadi, S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes”,  SMPTE Journal, Vol. 122, No. 4, May-June 2013.

[5] , “AHG-18”, JCTVC-Pxxxx., 16th Meeting: San José, US, 9–17 Jan. 2014.

# Patent rights declaration(s)

Movielabs, Universal, and Harmomic do not have any current or pending patent rights relating to the technology described in this contribution.