| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0081 |
| --- | --- |

| Title: | Comments on chroma_filter_hint SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA | Email: | chadfogg@gmail.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

# Abstract

This contribution suggests a few clarifications on chroma_filter_hint SEI semantics that is intended for indicating chroma subsampling filters in cascaded encoding within studio exchange: (1) the intended precision of the operation; (2) intended use of the SEI. It is also requested to consider whether the syntax should have the ability to signal an explicit phase offset for the chroma channels so that MPEG-1 and JPEG-style horizontal chroma subsample siting can be unambiguously conveyed. It is also recommended to carry over the key note from the SMPTE RP.2050 spec. into the chroma_filter_hint SEI semantics section clarifying that “..there is no benefit to using the filter sets defined in this document in an application when the output is not re-encoded.”  It may be also useful to add an a note that the SEI is not intended to support device-independent output chroma resampling should JCT wish to inform readers of the possibility that decoders are unlikely to implement a programmable chroma resampling engine, especially in the light that display processors may combine the chroma 4:2:0 upsampling step with other scaling, and that HDMI 2.0 et al that are now capable of passing through 4:2:0 from the HEVC decoded picture buffers directly to an external display processor.

# Introduction

The chroma_filter_hint SEI (payloadType = 137) adopted at the November 2013 JCT meeting in Geneva [1]  is a much appreciated means to allow the bitstream to carry indications of the chroma sampling filter applied in 4:2:0 and 4:2:2 conversion prior to encoding so that cascaded encode-decode chains in professional studio facilities can minimize cumulate chroma errors. While the syntax signals the explicitly defined SMPTE RP 2050-1:2012 [2] case, it may leave out a few use other cases that fit the basic FIR paradigm intended to be covered by the SEI semantics.

# Suggested changes and clarifications

## Filter tap and operational precision

The Exp-Golumb se(u) leading zero method of inferring precision may lead to a mis-match with BitDepth.  What is the precision intended by the elements involved in the p1[] equation?  (copied from the spec. in section 2.2.2 below). Should a note be added that the precision of vert_filter_coef and hor_filter_coef is the same as BitDepth?  Otherwise, implementers might attempt to normalize the coefficient after parsing.

## Initial phase offset in the color plane.

Rather than rely on filter design that achieves an offset by symmetry or extra zero’s, it may be better to explicity inform the reader that the chroma planes (Cb,Cr) are intended to have a specific phase offset or subsample location that is not co-sited with luma (Y).  The chroma_sample_loc_info_present_flag only applies to vertical alignment, and interlace content (i.e. when: general_interlace_source_flag==1).

### Syntax

[ From section D.2.27 Chroma sampling filter hint SEI message syntax (JCTVC-O1005_v4.doc) [3] ]

| chroma_resampling_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| ver_chroma_filter_idc | u(8) |
| hor_chroma_filter_idc | u(8) |
| ver_filtering_process_flag | u(1) |
| if( ver_chroma_filter_idc = = 1 ¦ ¦ hor_chroma_filter_idc = = 1 ) { |  |
| target_format_idc | ue(v) |
| perfect_reconstruction_flag | u(1) |
| if( ver_chroma_filter_idc = = 1 ) { |  |
| ver_cosited_flag | u(1) |
| num_vertical_filters | ue(v) |
| for( i = 0; i < num_vertical_filters; i++) { |  |
| ver_tap_length_minus1[ i ] | ue(v) |
| for( j = 0; j <= ver_tap_length_minus1[ i ]; j++) |  |
| ver_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( hor_chroma_filter_idc = = 1 ) { |  |
| hor_cosited_flag | u(1) |
| num_horizontal_filters | ue(v) |
| for( i=0; i < num_horizontal_filters; i++) { |  |
| hor_tap_length_minus1[ i ] | ue(v) |
| for( j=0; j <= hor_tap_length_minus1[ i ]; j++) |  |
| hor_filter_coeff[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Semantics

[ The following is copied from From D.3.27 Chroma resampling filter hint SEI message semantics [3] ]

The chroma resampling filter hint SEI message identifies the coefficient values of one or more chroma resampling filters that are suggested for post-processing use with the output decoded pictures. When the coefficients of a chroma resampling filter at both the encoder and the decoder side are designed based on the coefficients signalled in the chroma resampling filter hint SEI message, degradation of the color difference signal is expected to be minimized.

The chroma resampling filter hint SEI message shall not be present in a CVS that has chroma_loc_info_present_flag equal to 0 or chroma_format_idc equal to 0 or 3. [Ed. Why not 3 with target_format_idc equal to 1 or 2?]

ver_chroma_filter_idc specifies indicated chroma resampling filter coefficients in the vertical direction, as specified in Table D-1, for the output decoded pictures and possibly for input pictures at an encoder that takes the output decoded pictures as an input.

hor_chroma_filter_idc specifies indicated chroma resampling filter coefficients in the horizontal direction, as specified in Table D-3, for the output decoded pictures and possibly for input pictures at an encoder that takes the output decoded pictures as an input.

ver_filtering_process_flag specifies how the chroma resampling filter in the vertical direction is indicated to be performed. If ver_filtering_process_flag is equal to 1, chroma resampling filter is indicated to be applied on a field basis (i.e. only chroma samples of the same parity are used for filtering). Otherwise, the chroma resampling filter is indicated to be applied on a frame basis.

target_format_idc specifies the output format of chroma resampling relative to that of the luma sampling as specified for values of chroma_format_idc in subclause Error: Reference source not found. The value of target_format_idc shall be in the range of 1 to 3, inclusive. [Ed. Should explicitly constrain to "sensible" combinations of chroma_format_idc and target_format_idc and horizontal-versus-vertical filter signalling.]

perfect_reconstruction_flag equal to 1 indicates that the chroma resampling filter coefficients satisfy the perfect reconstruction condition, which means that upsampled results are identical when downsampling and upsampling are repeated using the indicated filters. perfect_reconstruction_flag equal to 0 indicates that the chroma resampling filter coefficients may not satisfy the perfect reconstruction condition. [Ed. Is integer rounding error being neglected when determining the value of this flag? Also, isn't perfect reconstruction an inherent property of the filter tap values that the receiver of the SEI message can identify from the indicated filter tap values themselves without this flag? If so, why indicate it? Even if not, why indicate it? Does the flag cause the decoder to do something different than it otherwise would have, or is it just a way of "advertising" the filter?]

NOTE – The chroma resampling filter coefficients signalled by ver_chroma_filter_idc equal to 2 or hor_chroma_filter_idc equal to 2 satisfy perfect reconstruction condition.

ver_cosited_flag equal to 1 indicates that the chroma plane is intended to be vertically co-sited with the luma plane. ver_costied_flag equal to 0 indicates that the chroma plane is not intended to be vertically co-sited with the luma plane.

num_vertical_filters specifies the number of filters indicated for chroma resampling in the vertical direction. The value of num_vertical_filters shall correspond to a value specified in Table D-1.

ver_tap_length_minus1[ i ] plus 1 specifies the length of the i-th filter in the vertical direction. The value of ver_tap_length_minus1[ i ] shall be in the range of 0 to 31, inclusive.

ver_filter_coeff[ i ][ j ] specifies the value of the j-th coefficient of the i-th filter in the vertical direction. The value of ver_filter_coeff[ i ][ j ] shall be in the range of −231 to 231 − 1, inclusive.

hor_cosited_flag equal to 1 indicates that the chroma plane is intended to be horizontally co-sited with the luma plane. ver_costied_flag equal to 0 indicates that the chroma plane is not intended to be horizontally co-sited with the luma plane.

num_horizontal_filters specifies the number of filters indicated for chroma resampling in the horizontal direction. The value of num_horizontal_filters shall correspond to a value specified in Table D-3.

hor_tap_length_minus1[ i ] plus 1 specifies the length of the i-th filter in the horizontal direction. The value of hor_tap_length_minus1[ i ] shall be in the range of 0 to 31, inclusive.

hor_filter_coeff[ i ][ j ] specifies the value of the j-th coefficient of the i-th filter in the horizontal direction. The value of hor_filter_coeff[ i ][ j ] shall be in the range of −231 to 231 − 1, inclusive.

Table D-1 specifies the coefficients of the chroma resampling filter in the vertical direction Fv. When ver_chroma_filter_idc is not equal to 0, the usage of coefficients of the chroma resampling filter in the vertical direction Fv is specified in Table D-2.

Table D-3 specifies the coefficients of the chroma resampling filter in the horizontal direction. When hor_chroma_filter_idc is not equal to 0, the usage of coefficients of the chroma resampling filter in the horizontal direction Fh is specified in Table D-4.

In Tables D-2 and D-4, "Chroma sample type" refers to chroma_sample_loc_type_top_field or chroma_sample_loc_type_bottom_field, as applicable.

Table D-1 – Chroma filter coefficients in the vertical direction

| ver_chroma_filter_idc | ver_filtering_process_flag | Vertical chroma resampling filter coefficients Fv | Informative Remarks |
| --- | --- | --- | --- |
| 0 | – | Unspecified | Chroma filter coefficients are unknown or determined by the application. |
| 1 | – | Fv[ i ][ j ] = ver_filter_coeff[ i ][ j ] i = 0 .. num_vertical_filters − 1 j = 0 .. ver_tap_length_minus1[ i ] | Filter coefficients are explicitly listed in the chroma resampling filter hint SEI messasge |
| 2 | 0 | Fv[ 0 ][ ] = { −3, −19, 34, 500, 500, 34, −19, −3 } Fv[ 1 ][ ] = { 19, 103, 1037, −135 } | Corresponds to the filter defined in SMPTE RP 2050-1:2012 See also SMPTE EG2050-2:2012 for information on a possible implementation of the filter defined in SMPTE RP2050-1:2012 |
|  | 1 | Fv[ 0 ][ ] = { −8, −26, 115, 586, 409, −48, −4, 0 } Fv[ 1 ][ ] = { 24, −41, 1169, −128 } Fv[ 2 ][ ] = { −76, 783, 330, −13 } |  |
| 3..255 |  | Reserved | For future use by ITU-T ¦ ISO/IEC |

Table D-2 – Usage of chroma filter in the vertical direction

| Chroma sample type | ver_filtering_process_flag | num_vertical_filters (when applicable) | Usage |
| --- | --- | --- | --- |
| 0, 1 | 0 | 2 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling Fv[ 1 ][ ] is used as FUv[ 0 ][ ] for upsampling Flipped Fv[ 1 ][ ] is used as FUv[ 1 ][ ] for upsampling |
|  | 1 | 3 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling a top field. Flipped Fv[ 0 ][ ] is used as FDv[ ] for downsampling a bottom field. Fv[ 1 ][ ] is used as FUv[ 0 ][ ] for upsampling a top field. Fv[ 2 ][ ] is used as FUv[ 1 ][ ] for upsampling a top field. Flipped Fv[ 2 ][ ] is used as FUv[ 0 ][ ] for upsampling a bottom field. Flipped Fv[ 1 ][ ] is used as FUv[ 1 ][ ] for upsampling a bottom field. |
| 2, 3 | 0 | 3 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling Fv[ 1 ][ ] is used as FUv[ 0 ][ ] for upsampling Fv[ 2 ][ ] is used as FUv[ 1 ][ ] for upsampling |
|  | 1 | 5 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling a top field. Fv[ 1 ][ ] is used as FDv[ ] for downsampling a bottom field. Fv[ 2 ][ ] is used as FUv[ 0 ][ ] for upsampling a top field. Fv[ 3 ][ ] is used as FUv[ 1 ][ ] for upsampling a top field. Fv[ 4 ][ ] is used as FUv[ 0 ][ ] for upsampling a bottom field. Flipped Fv[ 4 ][ ] is used as FUv[ 1 ][ ] for upsampling a bottom field. |
| 4, 5 | 0 | 3 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling Fv[ 1 ][ ] is used as FUv[ 0 ][ ] for upsampling Fv[ 2 ][ ] is used as FUv[ 1 ][ ] for upsampling |
|  | 1 | 5 | Fv[ 0 ][ ] is used as FDv[ ] for downsampling a top field. Fv[ 1 ][ ] is used as FDv[ ] for downsampling a bottom field. Fv[ 2 ][ ] is used as FUv[ 0 ][ ] for upsampling a top field. Flipped Fv[ 2 ][ ] is used as FUv[ 1 ][ ] for upsampling a top field. Fv[ 3 ][ ] is used as FUv[ 0 ][ ] for upsampling a bottom field. Fv[ 4 ][ ] is used as FUv[ 1 ][ ] for upsampling a bottom field. |

Table D-3 – Chroma filter coefficients in the horizontal direction

| hor_chroma_filter_idc | Horizontal chroma resampling filter | Informative Remarks |
| --- | --- | --- |
| 0 | Unspecified | Chroma filter is unknown or is determined by the application |
| 1 | Fh[ i ][ j ] = hor_filter_coeff[ i ][ j ] i = 0 .. num_horizontal_filters − 1 j = 0 .. hor_tap_length_minus1[ i ] | Filter coefficients are explicitly listed in the chroma resampling filter hint SEI messasge |
| 2 | Fh[ 0 ][ ] = { −1, 2, 6, 2, −1 } Fh[ 1 ][ ] = { 1 } Fh[ 2 ][ ] = { 1, 1 } | Corresponds to the 5/3 filter specified in ITU-T Rec. T.800 ¦ ISO/IEC15444-1 |
| 3..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

Table D-4 – Usage of chroma filter in the horizontal direction

| Chroma sample type | num_horizontal_filters (when applicable) | Usage |
| --- | --- | --- |
| 0, 2, 4 | 3 | Fh[ 0 ][ ] is used as FDh[ ] for downsampling Fh[ 1 ][ ] is used as FUh[ 0 ][ ] for upsampling Fh[ 2 ][ ] is used as FUh[ 1 ][ ] for upsampling |
| 1, 3, 5 | 2 | Fh[ 0 ][ ] is used as FDh[ ] for downsampling Fh[ 1 ][ ] is used as FUh[ 0 ][ ] for upsampling Flipped Fh[ 1 ][ ] is used as FUh[ 1 ][ ] for upsampling |

The chroma upsampling filtering process in the vertical direction is modelled as follows:

for( iPhase = 0; iPhase < 2; iPhase++ ) {
	bUv[ iPhase ][ 0 ] = −( lenUv[ iPhase ] − 1 ) / 2
	bUv[ iPhase ][ 1 ] = lenUv[ iPhase ] / 2
}
for( u = 0; u < W0; u++ ) {
	for( v = 0; v < H1; v++ ) {
		sum = 0
		iPhase = v % 2
		for( j = bUv[ iPhase ][ 0 ]; j  <=  bUv[ iPhase ][ 1 ]; j++ ) {
			sum += p0[ u ][ Clip3( 0, ( v  >>  1 ) + j, H0 − 1 ) ] * FUv[ iPhase ][ j − bUv[ iPhase ][ 0 ] ]
		}
		p1[ u ][ v ] = ( sum + ( divUv[ iPhase ]  >>  1 ) ) / divUv[ iPhase ]
	}
}					(D-1)

where W0 is the width of the cropped output picture in units of chroma samples, H0 is the height of the cropped output picture before vertical chroma upsampling in units of chroma samples, H1 is the height of the output picture after vertical chroma upsampling in units of chroma samples, lenUv[ k ] is the length of filter Uv[ k ][ ], divUv[ k ] is the summation of all coefficients of filter Uv[ k ][ ], p0[ ][ ] is the array of chroma samples in a picture before vertical chroma upsampling, and p1[ ][ ] is the array of chroma samples in a picture after vertical chroma upsampling. [Ed. Show how to compute W0, H0, H0, H1, lenUv and divUv using syntax elements and variables defined in the standard.]

When ver_filtering_process_flag is equal to 1, the chroma resampling filtering process in the vertical direction is applied to each field. [Ed. Clarify relationship to qq

The chroma upsampling filtering process in the horizontal direction is modelled as follows:

for( iPhase = 0; iPhase < 2; iPhase++ )
	bUh[ iPhase ][ 0 ] = −( lenUh[ iPhase ] − 1 ) / 2
	bUh[ iPhase ][ 1 ] = lenUh[ iPhase ] / 2
}
for( v = 0; v < H0; v++ ) {
	for( u = 0; u < W1; u++ ) {
		sum = 0
		iPhase = u % 2
		for( i = bUh[ iPhase ][ 0 ]; i  <=  bUh[ iPhase ][ 1 ]; i++ )
			sum += p0[ Clip3( 0, ( u  >>  1 ) + i, H0 − 1 ) ][ v ] * FUh[ iPhase ][ i − bUh[ iPhase ][ 0 ] ]
		p1[ u ][ v ] = ( sum + ( divUh[ iPhase ]  >>  1 ) ) / divUh[ iPhase ]
	}
}					(D-2)

where H0 is the height of a picture in units of chroma samples, W0 is the width of the cropped output picture before horizontal chroma upsampling in units of chroma samples, W1 is the width of the output picture after horizontal chroma upsampling in units of chroma samples, lenUh[ k ] is the length of filter Uh[ k ][ ], divUh[ k ] is the summation of all coefficients of filter Uh[ k ][ ], p0[ ][ ] is the array of chroma samples in a picture before horizontal chroma upsampling, and p1[ ][ ] is the array of chroma samples in a picture after horizontal chroma upsampling.

[Ed. Downsampling is also described in this SEI message. Where is the corresponding description of the downsampling process?]

# References

[1] K. Kazui, T. Chujoh, “Proposed text of Chroma sampling filter hint SEI”, JCTVC-O0079

[2] Society of Motion Pictures and Television Engineers, “4:2:2 / 4:2:0 Format Conversion Minimizing Color Difference Signal Degradation in Concatenated Operations — Filtering” RP 2050-1:2012

[3] [6] D. Flynn, J. Sole, G.J. Sullivan, T. Suzuki, “HEVC Range Extension Draft 5”, JCTVC-O1005, 17 December 2013.

# Patent rights declaration(s)

Harmonic does not have any current or pending patent rights relating to the technology described in this contribution.