| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0082 |
| --- | --- |

| Title: | How to use pic_struct |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg, Osnat Bar-Nir, P. Haskell 4300 N. 1st Street San Jose, CA 95134 USA Aaron Wells, Didier Le Gall 3101 Jay Street Santa Clara, CA 95054 USA | Email: | chadfogg@gmail.com Osnat.Bar-Nir@harmonicinc.com Paul.Haskell@harmonicinc.com awells@ambarella.com dlegall@ambarella.com |
| Source: | Harmonic Inc. and Ambarella Inc. |  |  |

_____________________________

# Abstract

The interlace-aware subset of HEVC high-level syntax is summarized in this informative input document. These elements are used as metadata to convey properties of interlace-scan or mixed progressive/interlace-scan content being passed through an HEVC codec chain. Previous proposals that, along with syntax, provided some background and use cases for some of these elements adopted into the specification have not been updated to reflect the latest HEVC draft specification language, nor do they account for new elements that were later added to the specification. This document identifies all the interlace-aware elements in one description, and provides use cases intended for broadcasting and other operations. As an aide to readers who are more familiar with earlier standards, tables of corresponding states between HEVC and AVC, and HEVC and MPEG-2 are provided. This document shows how editing and display post-processing would treat the decoded video sequence in accordance with these interlace-aware HEVC elements.

# Introduction

At the request of several proposals [1][2][3], HEVC adapted pic_struct from AVC to convey the intended output meaning of each coded picture within a bitstream, while not altering function in the video coding layer (VCL). The area closest to VCL impacted by pic_struct or other interlace-aware elements is the output behavior model in the Hypothetical Reference Decoder (Annex C). Decoders are free to follow other models, and often will do so if the display device or exchange format that follows decoding operates with different presentation assumptions, such as frame rate and scan type, from the model or target display assumptions made by the encoder that are reflected in the bitstream. The interlace-aware HEVC elements described in this document essentially address how interlace-aware pre-processing of a video sequence was applied prior to HEVC coding, and by implication, how post processing should correspondingly treat the pictures output by the decoder. Editing of both bitstream-domain and decoded-domain pictures may also use the interlace-aware HEVC elements to make more informed splicing decisions.

## The processing chain concept

[diagram:  pre-processing  encode  decode  output  post-processing  HDMI  display ]

## Elements inside HEVC

In HEVC, the following elements can be utilized to regulate interlace or mixed progressive/interlace video.

### Profile_tier_level (syntax: 7.3.3; semantics: 7.4.4)

A few flags were added at the sequence level (sps_parameter_set) as a form of soft profiling to assure equipment that a bitstream in the early stages of parsing will not require any interlace-aware post-processing for display.

| general_progressive_source_flag | general_interlaced_source_flag | Source scan type of all pictures within the Coded Video Stream (CVS) |
| --- | --- | --- |
| 0 | 0 | Unknown or unspecified |
| 0 | 1 | Interlace content only |
| 1 | 0 | Progressive only |
| 1 | 1 | Use source_scan_type in each picture to determine interlace or progressive |

### pic_timing SEI (syntax: D.2.3; semantics: D.3.3)

Pic_struct:  in essense, conveys the meaning of the coded picture in terms of both how it was captured and how it is intended to be output after decoding.

Source_scan_type: this indicates that the picture was captured by an interlace-scan camera or progressive scan camera (or source like film that had been later progressively scanned). A post processing device could apply different filters based on this variable that could change on a picture based in mixed content, such as an interlace advertising segment inserted in the middle of a movie broadcast. This variable will most likely have trust when the bitstream is created by a workflow that has explicit knowledge of the source material, such as 24 fps file source.

Duplicate_flag: the intent of this flag is to allow field sequences to mark which pictures have been duplicated in order to pad gaps in splicing, or maintain fixed output picture rate for some application, such as DVB. This flag has no analogy in AVC or MPEG-2. Encoders for those standards had to synthezie repeated fields without being able to inform decoders that there were coded synthesized/ repeated field pictures.  Only frame sequences in AVC and MPEG-2 that followed the PAFF (Picture Adaptive Field/Frame) convention could make use of the HRD’s built-in low-bitrate-oveehead repeat output process to synthesize non-coded fields via pic_struct 5 and 6. Synthesized pictures marked with duplicate_flag=1 can be made to be very compact thanks to the skipped CTU mode.

HEVC spec. added an overdue normative constraint clarifying that bitstreams alternate among one of three pic_struct settings: {0,7,8}, {1,2,9,10,11,12}, or {3,4,5,6}.

| Pic_struct set | Source scan type of all pictures within the Coded Video Stream (CVS) |
| --- | --- |
| 0,7,8 | Bitstream intended for progressive-scan display target, such as a 720p50 or 720p60 broadcast with repeated frames (e.g. 24 frames/sec movie conveyed in a 60p video feed) first introduced in MPEG-2. A bitstream whose output frames have complete unique corresponding coded frames would set pic_struct to 0 or likely set general_progressive_source_flag=1 && general_interlace_source_flag=0 |
| 1,2,9,10,11,12 | A field sequence with either all interlace scan content or mixed interlace and progressive scan content (PsF) . |
| 3,4,5,6 | The traditional interlace transport of mixed interlace and progressive (field repeat via 3:2 pulldown) mode first introduced in MPEG-2. |

### Chroma resampling filter hint SEI

Chroma resampling filter hint SEI (semantics: D.3.27): this recent SEI will be useful in production equipment where multiple re-encodes (cascades) can avoid large cumulative chroma errors if each generation of  4:2:2 video passed through 4:2:0 (encoder) equipment adheres to a chroma down-sampling and up-sampling formula.

### Time code

Time code SEI  (section semantics: D.3.28). time code has traditionally not altered codec behavior, but some industry pre or post-processing practies rely upon it. There is also a correlation between pic_struct and the time code information. A recent proposal [4] copied this feature from AVC into HEVC.

### Vui_parameters (syntax: Annex E.1., semantics: E.2.)

field_seq_flag: tells the post-processing device that the sequence being passed through the HEVC codec is a sequence of field pictures (all of which are half the height of a frame) or is a sequence of (full-height) frame pictures. This element is probably the biggest clue that interlace content is present, though for latency or other minimalist or common denominator motivations, progressive content may also be multiplexed as a series of fields rather than frames (the so called Progressive Field Sequence convention or “PsF” that was popular with D series digital video tape recorders and SDI connection plants).

chroma_loc_info_present_flag: copied from AVC, this VUI element is meant to convey the vertical chroma subsample location in 4:2:2 and both horizontal and vertical sub-sample location in 4:2:0 for each field. A de-interlacer and scaler may take note of this element to better preserve chroma appearance along edges.

fixed_pic_rate_within_cvs_flag: broadcast standards such as DVB, along with some storage medium formats such as Blu-Ray, require fixed frame rate operation in coded bitstreams. The intent of this mode is to provide regular, unambiguous output of pictures to be conveyed over connectors to display device, essentially recreating the timing of the video feed into the encoder (prior to any redundant picture removal for coding efficiency purposes). Other transports such as HDMI and SDI also require fixed picture rate model operation. Bitstreams however may be coded with variable instantaneous frame rates in practice, but the output model (where decoded picture buffers are sent out of the decoder to the display device) pads any skipped pictures or timing gaps created by 3:2 pulldown to produce a smooth constant rate picture delievery. Decoders and display devices are free to follow other models, including variable display rates,  but the metadata and the model that it is intended to drive can be useful in mapping the coded pictures to the alternate model preferred by the display device. Many flat panels attempt to recover the true 24 fps film content from decoded output video that has been padded through instructions given by pic_struct, or implied by Presentation Time Stamp, essentially dropping the repeated pictures by the output process back to the coded picture stream.  The time granularities of fixed picture is set by vui_num_units_in_tick and vui_time_scale.  See section 5.5.3 of the DVB spec. [5]

## Elements outside HEVC

MPEG-2 Systems layer (transport stream) has a few elements that could alter interlace-aware post-processing. The Presentation Time Stamp (PTS) has values that reflect the HRD output model which repeats output frames (pic_struct=7,8) or fields (pic_struct=5,6).

The following metadata often get translated to HEVC (with or without loss of some meaning), or in other cases are tunneled through the bitstream (user_data) without impating other elements in HEVC.

Metadata from external formats that are passed through Vertical Ancillary Codes, Vertical Blanking Interval, file headers (in file-based worklows) and HDMI.

# Corresponding elements tables

| Output picture pattern | AVC |  |  | HEVC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Output order | Pic_struct | NumClockTS | ct_type (0=prog, 1=interlace) | Pic_struct | NumClockTS | Source_scan _type (1:prog, 0:interlaced) |
| Progressive frame | 0 | 1 | (0) | 0 | 1 | 1 |
| Top field (progressive) | 1 | 1 | 0 | 1 | 1 | 1 |
| Top field (interlaced) | 1 | 1 | 1 | 1 | 1 | 0 |
| Bottom field (progressive) | 2 | 1 | 0 | 2 | 1 | 1 |
| Bottom field (interlaced) | 2 | 1 | 1 | 2 | 1 | 0 |
| Top, bottom (progressive) | 3 | 2 | 0 | 3 | 2 | 1 |
| Top, bottom (interlaced) | 3 | 2 | 1 | 3 | 2 | 0 |
| Bottom, Top (progressive) | 4 | 2 | 0 | 4 | 2 | 1 |
| Bottom, top (interlaced) | 4 | 2 | 1 | 4 | 2 | 0 |
| Top, bottom, top | 5 | 3 | (0) | 5 | 3 | 1 |
| Bottom, top, bottom | 6 | 3 | (0) | 6 | 3 | 1 |
| Double frame output | 7 | 2 | (0) | 7 | 2 | 1 |
| Triple frame output | 8 | 3 | (0) | 8 | 3 | 1 |

| Output picture pattern | MPEG-2 |  |  |  |  | HEVC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output order | Top_field_first | Repeatt first fld | Pict_struct | Prog_frame | Prog_sequence | Pic_struct | NumClockTS | Source_scan _type (1:prog, 0:interlaced) |  |
| Progressive frame | - | - | 11 | 1 | 0,1 | 0 | 1 | 1 |  |
| Top field (progressive) | - | - | 01 | 1 | 0 | 1 | 1 | 1 |  |
| Top field (interlaced) | - | - | 01 | 0 | 0 | 1 | 1 | 0 |  |
| Bottom field (progressive) | - | - | 10 | 1 | 0 | 2 | 1 | 1 |  |
| Bottom field (interlaced) | - | - | 10 | 0 | 0 | 2 | 1 | 0 |  |
| Top, bottom (progressive) | 1 | 0 | 11 | 1 | 0 | 3 | 2 | 1 |  |
| Top, bottom (interlaced) | 1 | 0 | 11 | 0 | 0 | 3 | 2 | 0 |  |
| Bottom, Top (progressive) | 0 | 0 | 11 | 1 | 0 | 4 | 2 | 1 |  |
| Bottom, top (interlaced) | 0 | 0 | 11 | 0 | 0 | 4 | 2 | 0 |  |
| Top, bottom, top | 1 | 1 | 11 | 1 | 0 | 5 | 3 | 1 |  |
| Bottom, top, bottom | 0 | 1 | 11 | 1 | 0 | 6 | 3 | 1 |  |
| Double frame output | 0 | 1 | 11 | 1 | 1 | 7 | 2 | 1 |  |
| Triple frame output | 1 | 1 | 11 | 1 | 1 | 8 | 3 | 1 |  |

# References

[1] K. Sugimoto, “High level syntax to support interlace format”, JCTVC-G540, 7th Meeting: Geneva, CH, 21-30 November, 2011

[2] O. Bar-Nir, C. Fogg, “Proposal to add pic_struct information to HEVC with simulation results”, JCTVC-K0165, 11th Meeting: Shanghai, CN, 10–19 Oct. 2012.

[3] W. Wan, B. Heng, W. Ahmad, “AHG9: Comments on Coding and Display formats”, JCTVC-K0146, 11th Meeting: Shanghai, CN, 10–19 Oct. 2012.

[4] time code SEI

[5] ETSI TS 101 154, “Digital Video Broadcasting (DVB): Specification for the use of Video and Audio Coding in Broadcasting Application based on the MPEG-2 Transport Stream” (2012).

# Patent rights declaration(s)

Ambarella and Harmonic do not have any current or pending patent rights relating to the technology described in this contribution.