| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0083r1 |
| --- | --- |

| Title: | Pic_struct coding results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA | Email: | chadfogg@gmail.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

# Abstract

In this study, deinterlaced 1080p 60 frame/sec video coded with unmodified HEVC main profile is compared to Sequence Adaptive Field/Frame (SAFF) and recent Picture Adaptive Field/Frame (PAFF) proposals coded as 1080i 30 frame/sec. The SAFF results were reported by the MPEG adhoc group m30737, while the PAFF results were reported in m32351. The sequences tested are the set selected by the same interlaced study AdHoc Group between the July 2013 Vienna and October 2013 Geneva MPEG meetings. The deinterlacer applied in the 1080p60 progressive encodings does not modify the original interlace lines: it only interpolates the missing in-between lines. Relative PSNR vs. bitrate plots suggest both deinterlaced progressive and interlace-coding methods have similar objective performance. PAFF luma (Y) BD-Rate reductions measured were: CheerleadersHD2 (-0.51%), CheerleadersHD4 (-1.45%), Musical (-3.50%), Mad Fashion (-21.77%), Basketball (-5.05%), and Tennis (+47.06%). Compared to the SAFF anchors, deinterlaced 1080p60 measured luma BD-Rates reductions of: CheerleadersHD2 (-4.40%), Cheerleaders HD4 (-8.83%),  Macy’s Parade Musical (-14.03%), and Mad Fashion (-36.79%), Basketball (-12.75%), Tennis (+31.61%).  Due to perhaps smooth spatial content, and low motion that resembles a 30p sequence for most frames in the sequence starting from the beginning, and the inevitable smaller intra coded field picture at the beginning of the GOP: Tennis 60p was the only sequence in the selected adhoc set to measure an increase (loss of efficiency) in terms of Y-BD-Rate over both the SAFF anchor and PAFF results. However, the total file size of Tennis at QP=32, for example, is 1/3 the size of the other motion sequences in the study which are more typical of television broadcast sports content, thus this sequence is likely a special corner case dominated by the coded size of the larger progressive intra pictures (compared to field intra pictures) and perhaps other atypical factors.

# Introduction

The 1080p 60 frame/sec sequenced were deinterlaced from the process described in [1] (similar to [2]), then coded with an unmodified HM 12.0 HEVC reference encoder according to the Random Access High Efficiency Main (8-bit) common conditions [3].

This information document updates results given at the previous JCT meeting with minor corrections such as: (1) encoding 8-bit Main instead of 10-bit Main; (2) a correct frame start offset into the test sequence that had previously been too early by one frame; (3) cropping each frame to 1080 instead of coding all 1088 lines in the source clip.

# Comparison

The tables below shows each set of respective gains of deinterlaced progressive-coded 60p and PAFF [5] over the same SAFF anchors [4].

|  | m32351 [7] PAFF gains over SAFF [4] |  |  | Deinterlaced 60p gains over SAFF anchors |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD Y | BD U | BD V | BD Y | BD U | BD V |
| Tennis | -9.4% | -5.7% | -4.1% | 31.61% | -17.85% | -14.68% |
| Basketball | -5.9% | -23.1% | -21.1% | -12.75% | -62.25% | -63.16% |
| CheersHD2 | -2.5% | -10.2% | -10.1% | -4.40% | -46.30% | -40.00% |
| CheersHD4 | -5.9% | -15.1% | -15.5% | -8.83% | -37.18% | -37.09% |
| MadHD1 | -15.9% | -28.4% | -28.5% | -36.79% | -48.38% | -48.83% |
| MusicHD1 | -9.8% | -3.2% | -4.6% | -14.03% | -32.69% | -30.73% |

Figures on the next page are copied from the accompany spreadsheet (JCTVC-P0083_r1.xls) comparing deinterlace 60p vs. the same SAFF anchors [4] and PAFF spreadsheet results [5].

The spreadsheet also contains charts comparing 60p deinterlaced, SAFF and PAFF for each sequence separately.  Only the sequence “Tennis” achieved any gain for 30i (SAFF, PAFF) over deinterlaced, progressively coded 60p, perhaps due to the soft content and low motion characteristic where the 60p higher overhead of a larger first coded intra frame in each GOP, plus sample rate and picture height would perhaps be a significant factor greater factor, more so than any aliasing or motion which are largely absent in the Tennis sequence.

# Observations

For definition purposes: the observed fields are the source field that were input into the deinterlacer (some times called “original”), while the interpolated fields were synthesized by the deinterlacer. When measuring the PSNR of the reconstructed interpolated fields separately from the reconstructed observed fields, it was noted that the interpolated fields measured lower PSNR than the observed fields, approximately -6dB for Y intra-frames on Basketball. The observed and interpolated fields converge as Qp increases (-1dB at QP=37), suggesting that as detail is “washed out” by the HEVC quantization process, the interpolated lines more closely resemble the observed lines.

(a clearer graph of the above is shown in the first tab of the accompanying spreadsheet)

(a clearer graph of the above is shown in the first tab of the accompanying spreadsheet)

# References

[1] K. Slavin, C. Fogg, “AVC and HEVC coding efficiency of deinterlaced sequences,” JCTVC-K0158, October 2012.

[2] A. Zineb, J. Vieron, P. Larbier, J.-M. Thiesse, “HEVC encoding of deinterlaced sequences: a preliminary study,” JCTVC-K0353, October 2012.

[3] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-L1100, Geneva, January 2013.

[4] A. Hinds, “AHG on Study of interlace coding in HEVC”, m30737, October 2013.

[5] G. Barroux, K. Kazui, A. Nakagawa, “Proposal of interlace coding tools for HEVC”, m32351, Jan. 2014

# Patent rights declaration(s)

Harmonic does not have any current or pending patent rights relating to the technology described in this contribution.