| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0086 |
| --- | --- |

| Title: | A proposal on level specification of Main 4:2:2 profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyumi Miyazawa Shun-ichi Sekiguchi Hiroharu Sakate 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

JCTVC-N0312 and JCTVC-O0288 proposed some modifications of the specification of Main 4:2:2 profiles and levels. According to the discussion at the last meeting, Main 4:2:2 profiles shall be turned off RExt-specific tools same as 4:2:0 related profiles. On the other hand, level specification of Main 4:2:2 profiles was not discussed sufficiently. For the profile/level discussion in San Jose meeting, this contribution proposes the modification of level-related definitions on Main 4:2:2 profiles as addressed in JCTVC-N0312 and JCTVC-O0288 again.

# Introduction

In JCTVC-N0312[1], we proposed 4:2:2 profile of Range Extensions (RExt), which extends Main 10 profile without any additional coding tools. Then, in JCTVC-O0288[2], we investigated the coding performance of the RExt-specific coding tools for natural YUV4:2:2 10 bit video sources and proposed the restriction of those tools for Main 4:2:2 profile based on the investigation results. From the result of RExt profiles discussion, we concluded that those RExt tools shall be turned off in all non-4:4:4 profiles. On level specification, we also proposed to modify the definition of maximum bitrate in JCTVC-N0312 and JCTVC-O0288, but this topic has not discussed yet and is to be discussed in San Jose meeting at January 2014. This contribution proposes the level-related modifications originally proposed in JCTVC-N0312 and JCTVC-O0288.

# Proposed level-related modifications to Main 4:2:2 profile

On level definition of Main 4:2:2, we propose to modify the maximum bitrate and CPB size in order to accommodate professional-domain requirement on wide-range bitrate support. The proposed values of CpbBrNalFactor and CpbBrVclFactor are shown as follows. Those parameters on Main 4:4:4 case are also changed to align with 4:2:2 specifications.

Table A-3– Specification of CpbBrVclFactor, CpbBrNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbBrVclFactor | CpbBrNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1000 | 1100 | 2.250 | 1.0 |
| Main 4:2:2:10 | 1600 | 1920 | 2.500 | 0.5 |
| Main 4:2:2 12 | 1600 | 1920 | 3.000 | 0.5 |
| Main 4:4:4 10 | 1600 | 1920 | 3.750 | 0.5 |
| Main 4:4:4 12 | 1600 | 1920 | 4.500 | 0.5 |

# Conclusions

This contribution has proposed a modification to the level specification of Main 4:2:2 profiles, which already proposed in JCTVC-N0312 and JCTVC-O0288. We recommend adopting the proposed modification into new RExt draft text.

# References

- S. Sekiguchi, A. Minezawa, H. Sakate and K. Kazui, “A proposal on HEVC 4:2:2 profile,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0312, 14th Meeting: Vienna, AT, July, 2013.

- A. Minezawa, S. Sekiguchi and H. Sakate, “Proposal on Main 4:2:2 10 profile,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O0288, 15th Meeting: Geneva, CH, October, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).