| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0087_r2 |
| --- | --- |

| Title: | Non-RCE3: On padding/unavailable samples pre-set for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution proposes adaptive pre-set derivation schemes for unavailable samples of intra block copy. The proposed schemes derive a dominant pixel value of current CU or CTU by referring to the neighbouring CU or CTU boundary for each component. It is confirmed that the proposed scheme with CU-level adaptivity achieves up to 3.2% and 1.3% bitrate savings relative to RCE3 anchor for lossy and lossless configurations, respectively. With CTU-level adaptivity, the coding performance of the proposed scheme shows up to 2.9% and 1.1% BD-rate improvement for lossy and lossless configurations, respectively.

Moreover, we evaluate the modified schemes using the largest run-length of pixel value in order to reduce memory size. It has been confirmed that the modified scheme with CU-level and CTU-level adaptivity has almost the same coding performance as that of histogram based scheme.

# Introduction

At the last meeting, Range Extensions core experiment 3 (RCE3) for intra block copy was established [1]. One of the main studies in the RCE3 is to relax vector restriction for unavailable region boundary. This contribution proposes alternative techniques on this subject.

# Proposed schemes

RCE3 subtest C.1 originally proposed in JCTVC-O0074 [2] introduces pre-set to all unavailable samples on intra block copy process with default value (mid grey). Although this scheme is the simplest one among 4 proposals in the RCE3 test C, it sometimes causes discontinuous distortion at block boundary by setting the pre-set indicating mid grey. To solve this problem, we propose an adaptive pre-set derivation scheme depicted in Figure 1. This scheme counts the number of samples in the neighboring samples of left side boundary of current CU for each pixel value and selects the most occurring value as the pre-set used in current CU for each component.

Figure 1  The overview of the proposed scheme

We also evaluated CTU-level pre-set derivation scheme as shown in Figure 2. This scheme derives pre-set value for each CTU and the same pre-set is used in all CU contained in the CTU, which can be implemented with lower computations than the CU-level scheme.

Figure 2  The overview of the CTU-base derivation scheme

# Simulation results

The performance evaluation of the proposed schemes has been conducted. All proposed schemes are implemented on top of RCE3 anchor (HM12.1+RExt5.1). The experimental conditions are subject to RCE3 test configurations[1]. Table 1 and Table 2 show coding performance of the CU-level adaptive scheme relative to the anchor. According to the result, this scheme achieves 0.0-3.2% and 0.0-1.2% gains for lossy and lossless configurations, respectively. As shown in Table 3 and Table 4, the coding results of the CTU-level adaptive scheme shows 0.0-2.9% and 0.0-1.1% BD-rate improvement for lossy and lossless configurations, respectively.

Table 1  Coding performance of CU-level proposed scheme for lossy case

Table 2  Coding performance of CU-level proposed scheme for lossless case

Table 3  Coding performance of CTU-level proposed scheme for lossy case

Table 4  Coding performance of CTU-level proposed scheme for lossless case

# Run-length approach

We additionally evaluate the modified schemes using run-length of pixel value with CU-level and CTU-level adaptivity. In the modified schemes, pre-set is set to the largest run-length of pixel value for each CU or CTU. As shown an example in Figure 3, the largest run-length of pixel value indicating that the length is equal to 7 is selected as the pre-set of the current CU. The modified schemes do not require additional memory of a counter for each pixel value required in the histogram based scheme in Section 2.

Figure 3  An example of calculation of run length of pixel value

From Table 5 to Table 8 show BD-rate performance of the modified scheme with CU-level and CTU-level adaptivity relative to RCE3 anchor. It has been confirmed that the modified scheme with CU-level and CTU-level adaptivity has almost the same coding performance as that of histogram based scheme.

Table 5  Coding performance of CU-level modified scheme for lossy case

Table 6  Coding performance of CU-level modified scheme for lossless case

Table 7  Coding performance of CTU-level modified scheme for lossy case

Table 8  Coding performance of CTU-level modified scheme for lossless case

# Conclusions

In this contribution, we have proposed adaptive pre-set derivation schemes for unavailable samples of intra block copy. The proposed schemes set the most occurring value on one pixel left line as the pre-set for each CU or CTU. According to our simulation results, the CU-level adaptive derivation scheme achieves coding gain up to 3.2% and 1.2% for lossy and lossless configurations, respectively. It has also been found that the proposed scheme should be effective even when CTU-level derivation approach is taken. Moreover, we tested the modified schemes using the largest run-length of pixel value without a counter for each pixel value. It has been confirmed that the modified scheme with CU-level and CTU-level adaptivity has almost the same coding performance as that of histogram based scheme. We recommend to adopt the proposed schemes to RExt draft text or to evaluate both schemes in CE.

# References

- J. Sole, E. Alshina, D.-K. Kwon and W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O1123, 15th Meeting: Geneva, CH, October, 2013.

- E. Alshina, A. Alshin and S. Lee, “AhG5: Intra block copy within one LCU,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O0074, 15th Meeting: Geneva, CH, October, 2013.

- C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz, T. Lin and S. Wang, “AhG5: Intra block copying with padding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O0157, 15th Meeting: Geneva, CH, October, 2013.

- S. Lee, E. Alshina and C. Kim, “AHG5: Extension of intra block copy,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O0112, 15th Meeting: Geneva, CH, October, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).