| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0089 |
| --- | --- |

| Title: | Non-RCE3: On unavailable sample preset for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Park Sunil Lee Chanyul Kim | Email: | cy1205.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution a kind of intra prediction method is proposed to relax the search range restriction for intra block copy. In first step, the most frequently appeared color is selected as the background color of current CU. In second step, the color of each unavailable sample is set to the background color. Compared with the RCE3 anchor, average (-0.6%/-0.9%/-0.9%) BD-rate reductions are observed for CLASS F, RGB SC, YUV SC sequences with AI Main-Tier configuration.

# Introduction

Intra block copy works well for screen images. In most cases, screen images can be divided into two components : foreground pattern and background color. Hence for screen content coding background color is most preferable as the default color of unpredictable samples. In this contribution we propose a most frequently appeared color among neighboring samples as the background color.

# Proposed Method

The most frequently appeared color can be derived from the left neighboring samples, above neighboring samples, or all neighboring samples. The first option is the same with the method in [1]. If there are several candidates, then the candidate of the smallest index is used. In current reference software (HM12.1+RExt5.1) the displacement vector is determined in the luminance channel. If the displacement vector is determined, then the same process is performed in chrominance channels if the vector exists on the overlapped region which contains unavailable samples.

# Experimental Results

The proposed method is tested with RCE3 CTC [2] whose software version is HM12.1+RExt5.1. The coding performance of the following results is slightly different from that of [1]. One possible reason is the difference of search ranges. Note that the search range should be extended to check the coding performance but there is no guidance on the search range extension. Another possible reason is the rule to decide one candidate among several candidates whose sample count is the same.

## Left and Above

- Lossless

- Lossy

## Left

- Lossless

- Lossy

## Above

- Lossless

- Lossy

# References

- A. Minezawa, K. Miyazawa, S. Sekiguchi, and T. Murakami, “Non-RCE3: On padding/unavailable samples pre-set for intra block copy,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0087, 16th Meeting: San Jose, US, January, 2014.

- J. Sole, E. Alshina, D.-K. Kwon and W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O1123, 15th Meeting: Geneva, CH, October, 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).