| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0093 |
| --- | --- |

| Title: | Non-RCE4: Cross-CU major color index prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, peter.chuang, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In RCE4 Test1 (JCTVC-P0107), major color index prediction is performed within each CU. In this proposal, it is proposed to allow major color index prediction across CUs. To obtain major color index prediction at boundaries of a current CU, pre-deblocked samples on the last column of the left CU and the last row of the above CU are converted into major color indices through the major color table of the current CU. It is shown that the proposed method achieves 0.6%, 0.5%, and 0.3% BD-rate savings compared with RCE4 Test1 for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

In RCE4 Test1 [1], major color indices of samples in the current CU can be predicted from their left or above neighboring samples. For samples at the left or above boundary of the current CU, the left or above neighboring samples belong to a neighboring CU, and their indices are set to 0 for predicting the indices of the samples on the first column or row in the current CU. This may degrade coding efficiency.

# Proposed method

We propose to allow major color index prediction across CUs as follows. The samples on the last column of the left CU and the last row of the above CU are involved in predicting the indices of the samples on the first column and the first row in the current CU. Since the major color tables from neighboring CUs can be different from that of the current CU, it does not make sense to predict the indices of the current CU directly from the indices of the neighboring CUs. Therefore, the pre-deblocked samples from the neighboring CUs are converted through the major color table of the current CU into major color index prediction values. In this way, cross-CU major color index prediction can be done properly.

# Experimental results

The proposed method is implemented on top of RCE4 Test1. The tests are conducted under the common test conditions in RCE4 [2]. Table 1 shows the results of the proposed method using HM-12.1+RExt-5.1 [3] as the anchor. Table 2 shows the results of RCE4 Test1 using the same anchor as Table 1.

For comparison, we change the anchor of Table 1 to RCE4 Test1, and the results are shown in Table 3. Compared with RCE4 Test1, 0.6%, 0.5%, and 0.3% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Conclusion

In this contribution, it is proposed to use pre-deblocked samples of neighboring left and above CUs to help to predict the major color indices of samples at the left and above boundaries of the current CU. Compared with RCE4 Test1, it is reported to achieve 0.6%, 0.5%, and 0.3% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0107, Jan. 2014.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Table 1.  Results of the proposed method; anchor: HM-12.1+RExt-5.1

Table 2.  Results of RCE4 Test1; anchor: HM-12.1+RExt-5.1

Table 3.  Results of the proposed method; anchor: RCE4 Test1