| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0095 |
| --- | --- |

| Title: | Non-RCE4: Removal of syntax redundancy in RCE4 Test2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, peter.chuang, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In RCE4 Test2 (JCTVC-P0xxx), run coding can be used to represent major color indices of multiple samples in a CU. In the run coding, a mode flag to indicate copy-above or send-index is first signaled, followed by a major color index if the send-index mode is selected, and then followed by a run value to describe the number of multiple samples. In the copy-above mode, indices of the above line are copied for the multiple samples. In the send-index mode, the multiple samples share the explicitly signaled index. In this proposal, it is proposed to prohibit the copy-above mode and save the mode flag when the previous group of samples selects the copy-above mode. Experimental results show that, compared with RCE4 Test2, 0.3%, 0.3%, and 0.4% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

In RCE4 Test2 [1], run coding can be used to represent major color indices of multiple samples in a CU. There are two run modes: copy-above and send-index. A mode flag is explicitly signaled to select one of the two modes. If the send-index mode is selected, a major color index is next signaled. After that, a run value is signaled to indicate the number of multiple samples for both modes. During the reconstruction process, if the copy-above mode is selected, indices of the above line are copied for the multiple samples; otherwise, the multiple samples share the explicitly signaled index.

The syntax design mentioned above has redundancy when consecutive groups of samples select the copy-above mode. For example, a group of M samples coded with the copy-above mode followed by a group of N samples coded with the copy-above mode is essentially equivalent to a group of M+N samples coded with the copy-above mode.

# Proposed method

The syntax redundancy is removed as follows. If the copy-above mode is chosen by the previous group of samples, the current group of samples cannot choose the copy-above mode and can save the mode flag.

# Experimental results

The proposed method is implemented on top of RCE4 Test2. The tests are conducted under the common test conditions in RCE4 [2]. Table 1 shows the results of the proposed method using HM-12.1+RExt-5.1 [3] as the anchor. Table 2 shows the results of RCE4 Test2 using the same anchor as Table 1.

For comparison, we change the anchor of Table 1 to RCE4 Test2, and the results are shown in Table 3. It is shown that, compared with RCE4 Test2, 0.3%, 0.3%, and 0.4% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Conclusion

In this proposal, a method is proposed to remove the syntax redundancy in RCE4 Test2. Compared with RCE4 Test2, it is reported to achieve 0.3%, 0.3%, and 0.4% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Guo, M. Karczewicz, J. Sole, R. Joshi, “xxxx,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0xxx, Jan. 2014.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Table 1.  Results of the proposed method; anchor: HM-12.1+RExt-5.1 (short-length)

Table 2.  Results of RCE4 Test2; anchor: HM-12.1+RExt-5.1 (short-length)

Table 3.  Results of the proposed method; anchor: RCE4 Test2 (short-length)