| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0096 |
| --- | --- |

| Title: | Non-RCE4: Major color table propagation through non-palette CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang1, PoLin Lai2, Yu-Chen Sun1, Yi-Wen Chen1, Yu-Wen Huang1, Shan Lu2, Shawmin Lei1 1 No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 2 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, polin.lai, yc.sun, yiwen.chen, yuwen.huang, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In RCE4 Test1 (JCTVC-P0xxx) and RCE4 Test2 (JCTVC-P0xxx), major color tables of neighboring CUs can be used to predict the major color table of the current CU. If a neighboring CU is not coded in palette mode, all elements of the major color table from the neighboring CU are set to 0. In this contribution, major color table propagation through non-palette CUs is proposed. For a non-palette CU, the major color table of its left CU is used as its major color table for predicting future CUs. When RCE4 Test1 is the anchor and implementation basis, the proposed method reportedly shows 2.7%, 2.6%, and 2.4% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. When RCE4 Test2 is the anchor and implementation basis, the BD-rate savings are 1.4%, 1.1%, and 1.2%. When the proposed method is combined with the major color table sharing in JCTVC-P0xxx using RCE4 Test1 as the anchor and implementation basis, the BD-rate savings are 4.6%, 4.4%, and 4.0%.

# Introduction

Two palette coding methods were proposed at the 15th JCT-VC meeting and are studied in RCE4 as Test1 [1] and Test2 [2], where major color tables of neighboring CUs can be used to predict the major color table of the current CU. In RCE4 Test1, each major color (i.e., the n-th major color table element) of the current CU can decide whether to use prediction and choose a predictor from the corresponding major color (i.e., the n-th major color table element) of the left or above CU in case of prediction. In RCE4 Test2, each major color of the left CU can decide whether to predict a major color of the current CU. In both methods, however, if a neighboring CU is not coded in palette mode, the major color table of the neighboring CU is set to all zero, and the prior major color table cannot be propagated to following CUs. To improve the coding efficiency, major color table propagation through non-palette CUs is proposed.

# Proposed method

For a non-palette CU, the major color table of its left CU is used as its major color table. This major color table can be a reference major color table for following CUs, and the prior major color table can be propagated through non-palette CUs.

In the implementation on top of RCE4 Test1, a non-normative encoder-only modification is also applied. In addition to testing the major color table derived from the current CU samples directly, the encoder will also test using the major color table from the left or above CU as the major color table of the current CU.

# Simulation results

Experiments were conducted based on RCE4 Test1, RCE4 Test2, and the major color table sharing in JCTVC-P0xxx [3], where the major color table of the left or above CU can be entirely copied as the major color table of the current CU. All the results were simulated according to the common test conditions defined in JCTVC-O1124 [4].

## On top of RCE4 Test1

Lossy coding results and lossless coding results of the proposed method using RCE4 Test1 as the anchor and implementation basis are shown in Table 1, where 2.7%, 2.6%, and 2.4% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

## On top of RCE4 Test2

Lossy coding results and lossless coding results of the proposed method using RCE4 Test2 as the anchor and implementation basis are shown in Table 2, where 1.4%, 1.1%, and 1.2% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

## Combination with the major color table sharing in JCTVC-P0xxx

Lossy coding results of the proposed method combined with the major color table sharing in JCTVC-P0xxx using RCE4 Test1 as the anchor and implementation basis are shown in Table 3, where 4.6%, 4.4%, and 4.0% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. When the anchor is changed to the major color table sharing in JCTVC-P0xxx, the corresponding BD-rate savings are 2.5%, 2.5%, 2.5%, as shown in Table 4.

# Conclusions

In this contribution, a major color table propagation method is proposed. For a non-palette CU, the major color table of its left CU is used as its major color table. This major color table can be a reference major color table for following CUs. It is reported to achieve additional 2.7%, 2.6%, and 2.4% BD-rate savings on top of RCE4 Test1 for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. The corresponding additional BD-rate savings on top of RCE4 Test2 are 1.4%, 1.4%, and 1.4%.

# References

- X. Guo, B. Li, J. Xu, Y. Lu, S. Li, F. Wu, “xxxx,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0xxx, Jan. 2014.

- L. Guo, M. Karczewicz, J. Sole, R. Joshi, “xxxx,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0xxx, Jan. 2014.

- P. Lai, S. Liu, T.-D. Chuang, Y.-W. Huang, S. Lei, “Non-RCE4: Color table merge from above and left,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0xxx, Jan. 2014.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed method; anchor: RCE4 Test1 (short-length)

Table 2.  Results of the proposed method; anchor: RCE4 Test2 (short-length)

Table 3.  Lossy coding results of the proposed method and the major color table sharing in JCTVC-P0xxx; anchor: RCE4 Test1 (short-length)

Table 4.  Lossy coding results of the proposed method and the major color table sharing in JCTVC-P0xxx; anchor: the major color table sharing in JCTVC-P0xxx (short-length)