| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0097 |
| --- | --- |

| Title: | AHG5 and AHG8: Extended Cross-Component Decorrelation for Animated Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Khairat T. Nguyen D. Marpe Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49 30 31002 230 ali.khairat@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute |  |  |

_____________________________

# Abstract

The Cross-Component Decorrelation (CCD) approach specified in the current the RExt Draft allows the prediction from luma to chroma transform blocks only. Adding the possibility to predict from the transform blocks of the first chroma component for the second chroma component would inherit the same complexity. This document proposes to add the mentioned flexibility and shows that for a specific set of content, where other tools show no improvement, the additional source for CCD can improve the compression efficiency.

# Adaptive CCD from Cb to Cr

Simulation results for several screen content approach show huge improvement in terms of compression efficiency for quite simple screen content. Especially, when the sequence consists of simple lines and plain background, very high BD-rate numbers are reported. However, for animated screen content where the scene consists of a lot of gradients, the performance is often smaller or negligible.

The CCD approach adopted in the latest RExt Draft allows the prediction from luma to chroma transform blocks. An improvement of about -2.6 % (AI), -1.0 % (RA), and -0.4 % (LB) were reported for the animated Y´CbCr screen content class under AHG8 test condition and of about -1.6 % (AI), -1.0 % (RA), and -0.4 % (LB) for the VenueVu sequence, which is an animated sequence, under AHG5 test condition. By adding the additional possibility to predict from the first chroma component, an additional improvement of about -0.9 % (AI), -0.8 % (RA), and -0.45  (LB) for the animated Y´CbCr class under AHG8 test condition and of about -1.69 %(I), -1.20 % RA), and -0.87 % (B) for VenueVu under AHG5 test condition are obtained.

# Adaptive Signaling of Prediction Parameters

Currently, the RExt Draft specifies a fixed binarization scheme for the CCD gradient parameter α. The scheme is designed under the constraint of natural Y´CbCr content. Particularly, it is assumed that the distribution of α is concentrated around the value 0. However, for R´G´B´ content and some specific kind of screen content, the distribution of α is different, e.g., it is concentrated around the unquantized α value of 1. In order to overcome this drawback, this document also isoposes to signal the possible range of α in the PPS and derive the constraints on the existing binarization scheme from the PPS parameters. In this way, only one single path in the parsing process is required.

For that purposes, two syntax elements are transmitted in the PPS specifying the lower range and the upper range of α.

# Simulation Results

# on – Cb to Cr

# b to Cr

# AHG5 Test Condition – α can be 2 or 3

# AHG8 Test Condition – α can be 2 or 3

# adaptive Cb to Cr

# Cb to Cr

or detailed results, please refer to the Excel fileed with this contribution.

# Conclusion

As can be seen from the results, by setting α appropriately, performance improvement can be achieved, e.g., for R´G´B´ content. Furthermore, the performance for both flexibility are additive. The results demonstrate clearly the benefit from making the CCD approach more flexible.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).