| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0101 |
| --- | --- |

| Title: | Non-RCE4: Removal of syntax redundancy in RCE4 Test1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Chen Sun, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yc.sun, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In RCE4 Test1 (JCTVC-P0107), two different codewords are used to indicate predicting the major color index of a sample from its left or above. However, the current codeword design results in redundancy when the left index and the above index are the same. In this proposal, the syntax redundancy is removed by assigning only one codeword for the two prediction directions when the two neighboring indices are the same. It is reported to achieve 0.6%, 0.5%, and 0.5% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

In RCE4 Test1 [1], two major color index prediction modes, copy above and copy left, are allowed in the normal line mode. If a line is coded in the normal line mode, for each sample in the line, one of the codewords is signaled to indicate the selected prediction mode, as shown in Table 1. However, if the index of the left sample and that of the above pixel are the same, it is unnecessary to use two different codewords to distinguish them, and removing this syntax redundancy should improve coding efficiency.

Table 1.  Current codeword design of major color index prediction modes

| Mode | Codeword |
| --- | --- |
| Copy above | 1 |
| Copy left | 00 |
| No prediction | 01 |

# Proposed method

We propose to use only one codeword to represent the two prediction directions when the left index (L) and the above index (A) are the same. As Table 2 shows, if L differs from A, the original codewords are applied; otherwise, new codewords are applied.

Table 2.  Proposed codeword design of major color index prediction modes

| Mode | Codeword |  |
| --- | --- | --- |
|  | L != A | L == A |
| Copy above | 1 | 1 |
| Copy left | 00 | 1 |
| No prediction | 01 | 0 |

# Experimental results

The proposed method is implemented on top of RCE4 Test1. The experiments follow the common test conditions in RCE4 [2]. When the anchor is HM-12.1+RExt-5.1 [3], the results are shown in Table 3.

For comparison, we also present the results of using RCE4 Test1 as the anchor, as shown in Table 4. It can be seen that 0.6%, 0.5%, and 0.5% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier and LB-Main-Tier, respectively.

# Conclusion

This proposal removes the syntax redundancy in RCE4 Test1 that uses two different codewords to represent two prediction directions. The method is as follows. When the two prediction directions result in the same prediction of the major color index, only one codeword is used. Experimental results show that, compared with RCE4 Test1, 0.6%, 0.5%, and 0.5% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0107, Jan. 2014.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Table 3.  Results of the proposed method; anchor: HM-12.1+RExt-5.1

Table 4.  Results of the proposed method; anchor: RCE4 Test1