| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0105 |
| --- | --- |

| Title: | RCE3: Cross-verification of subtest C.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the subtest C.2 from RCE3. In this method, unavailable samples are copied horizontally from the left block. The coding performance is independently evaluated by Samsung and the results match those provided by the proponent.

# Introduction

This contribution reports the cross-verification results of the subtest C.2 from RCE3 on the padding-based generation of unavailable samples in intra block copy [1]. In this method, unavailable samples are copied horizontally from the left block. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE3 [2].

# Test results

The cross-verification results of the subtest C.2 from RCE3 are summarized in Table 1 and 2, and they match those provided by the proponent.

Table 1. Test results of RCE3 subtest C.2 in lossy coding configuration

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -0.4% | -0.3% | -0.2% | -0.3% | -0.2% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.3% | -0.2% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.0% | -1.1% | -1.1% | -1.0% | -1.1% | -0.8% | -1.1% |
| YCbCr 4:4:4 SC (Optoinal) | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% | -1.1% | -0.7% |
| Enc Time[%] | 106% | 106% | 106% | 105% | 106% | 105% | 105% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 99% | 99% |

Table 2. Test results of RCE3 subtest C.2 in lossless coding configuration

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | -0.5% | -0.5% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.3% | -0.2% | -0.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.3% | -0.3% | -0.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.7% | -0.7% | -0.6% |
| YCbCr 4:4:4 SC (Optoinal) | -1.2% | -1.2% | -1.3% |
| Enc Time[%] | 99% | 98% | 98% |
| Dec Time[%] | 98% | 95% | 95% |

# Conclusion

This contribution reports the cross-verification results of the subtest C.2 from RCE3 on the padding-based generation of unavailable samples in intra block copy. In this method, unavailable samples are copied horizontally from the left block. The coding performance is independently evaluated by Samsung and the results match those provided by the proponent.

# References

- C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz, T. Lin, S. Wang, “AhG5: Intra block copying with padding,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0157, Oct. 2013.

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1123, Oct. 2013.