| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0106 |
| --- | --- |

| Title: | AHG5: Recommended profiling of range extension coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Elena Alshina Chanyul Kim Ken McCann | Email: | sunil.lee@samsung.com elena_a.alshina@samsung.com dionism@samsung.com ken@zetacast.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides recommendations on the profiling of range extension coding tools for 4:4:4 profiles. It is suggested that the decision on profiling for the HEVC range extensions should follow a similar process to that used for HEVC v1, i.e. it should be based on an assessment of the trade-off between coding gain and complexity of the additional range extension coding tools, evaluated under the common test conditions of HEVC range extensions.

This contribution provides such an assessment. It is asserted that the results indicate that cross component de-correlation is the only additional coding tool in the draft RExt text that provides a beneficial trade-off between performance and complexity with generic video content. It is therefore proposed that the Main 4:4:4 profiles should include cross component de-correlation, but not the other coding tools added by the draft text, which are asserted to provide a beneficial performance/complexity trade-off only with screen content.

It is also suggested that Screen Content (SC) 4:4:4 Profiles, including SC coding tools, should be developed in coordination with the call for proposals for coding of screen content and medical visual content planned for January 2014. It is suggested that all of the coding tools in the draft RExt text should be included in the initial test model for this new activity.

# Introduction

At the 14th JCT-VC meeting in Vienna, five profiles of the HEVC range extensions were defined: Main 12, Main 4:2:2 10/12, and Main 4:4:4 10/12. At the 15th JCT-VC meeting in Geneva, the profiling of range extension coding tools was discussed and it was agreed to exclude the coding tools effective only for screen content coding (SCC) from Main 12 and Main 4:2:2 10/12. However, the consensus on the proposed changes to Main 4:4:4 10/12 could not be reached, since sufficient time was not allocated to discuss all input documents related to the profiles of HEVC range extensions.

In addition to the design improvements to properly support high bit-depth and extended colour format, the range extension coding tools which have been adopted so far include:

- implicit residual DPCM (RDPCM) [1][2]

- explicit RDPCM [3]

- residual rotation [4]

- single context model for significance map coding in transform-skipped (TS) block [5]

- intra block copy [6]

- fast adaptation method for the Rice parameter derivation in the binarization of coeff_abs_level_remaining [7]

- cross component de-correlation [8]

In this contribution, the coding gain and complexity of these additional coding tools are evaluated under the common test condition of HEVC range extensions, and a profiling of the coding tools for 4:4:4 profiles is recommended based on the test results.

# Test results in AHG5 CTC

The coding gain and complexity of the additional coding tools are evaluated under the AHG5 common test condition (CTC) of the HEVC range extensions [9]. In all the tests, the anchor is HM12.0-RExt5.1 and the tested condition is “coding tool off”. Tables 1 to 7 summarize the test results for implicit RDPCM, explicit RDPCM, residual rotation, single context model for significance map coding in TS block, intra block copy, Rice parameter adaptation, and cross component de-correlation, respectively.

The results show that the coding gain provided by the additional range extension coding tools is not significant for the test sequences in the AHG5 CTC. This has been already confirmed when those coding tools were reviewed and adopted, since they were mainly for SCC. The one exception is cross component de-correlation, which provides significant coding gain even for camera-captured video sequences in RGB 4:4:4 format. The complexity of the coding tools measured in terms of encoding and decoding time was not significant for implicit RDPCM, residual rotation, single context model for significance map coding in TS block, Rice parameter adaptation. However, the encoding time could be reduced by 5~10% by disabling explicit RDPCM and cross component de-correlation. The test results also show that the encoding time could be significantly reduced (30% in all intra) by disabling intra block copy. Intra block copy is also known to increase the cost of practical decoder implementations, since the samples within the search range (currently two CTUs) should be kept in intra slices.

Table 1. Performance of implicit RDPCM off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 100% | 100% | 101% |

Table 2. Performance of explicit RDPCM off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 96% | 96% | 97% | 87% | 88% | 89% | 90% |
| Dec Time[%] | 99% | 100% | 100% | 100% | 100% | 100% | 100% |

Table 3. Performance of residual rotation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YCbCr 4:2:2 | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 100% | 100% |

Table 4. Performance of single context model for significance map coding in TS block off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 100% | 101% |

Table 5. Performance of intra block copy off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.5% | 0.2% | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 | 0.3% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.4% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 70% | 69% | 69% | 97% | 96% | 97% | 96% |
| Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

Table 6. Performance of Rice parameter adaptation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

Table 7. Performance of cross component de-correlation off vs. on (AHG5 CTC)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 25.6% | 19.5% | 13.7% | 18.0% | 14.0% | 15.1% | 11.7% |
| YCbCr 4:4:4 | 1.4% | 1.8% | 1.9% | 0.5% | 0.8% | 0.2% | 0.6% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 9.7% | 7.7% | 5.6% | 6.7% | 5.3% | 5.6% | 4.5% |
| Enc Time[%] | 93% | 92% | 92% | 96% | 94% | 96% | 93% |
| Dec Time[%] | 101% | 101% | 101% | 101% | 101% | 101% | 101% |

# Test results in AHG8 CTC

The coding gain and complexity of the additional coding tools are also evaluated under the AHG CTC of the HEVC range extensions [9]. In all the tests, the anchor is HM12.0-RExt5.1 and the tested condition is “coding tool off”. Tables 8 to 21 summarize the test results for implicit RDPCM, explicit RDPCM, residual rotation, single context model for significance map coding in TS block, intra block copy, Rice parameter adaptation, and cross component de-correlation, respectively, in both lossy and lossless coding configurations. The results show that the coding gain provided by the additional range extension coding tools is higher in AHG8 CTC which includes SC test sequences than their coding gain in AHG5 CTC without any SC sequences.

Table 8. Performance of implicit RDPCM off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 1.1% | 1.1% | 1.3% | 0.5% | 0.5% | 0.3% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.8% | 0.8% | 0.8% | 0.4% | 0.4% | 0.1% | 0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.9% | 0.8% | 0.8% | 0.5% | 0.3% | 0.0% | 0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB 4:4:4 SC (Optional) | 1.3% | 1.6% | 1.6% | 0.5% | 0.5% | 0.3% | 0.2% |
| YCbCr 4:4:4 SC (Optoinal) | 2.0% | 2.2% | 2.2% | 0.7% | 0.9% | -0.9% | -0.9% |
| Enc Time[%] | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 83% | 85% | 86% | 96% | 95% | 96% | 95% |

Table 9. Performance of implicit RDPCM off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 7.6% | 3.6% | 2.3% |
| Class B | 3.1% | 0.4% | 0.3% |
| RGB 4:4:4 SC | 11.2% | 7.2% | 6.4% |
| RGB 4:4:4 Animation | 8.9% | 2.4% | 1.8% |
| YCbCr 4:4:4 SC | 7.7% | 4.5% | 3.3% |
| YCbCr 4:4:4 Animation | 7.1% | 1.4% | 0.9% |
| RangeExt | 2.5% | 0.5% | 0.3% |
| RGB 4:4:4 SC (Optional) | 12.7% | 10.8% | 8.8% |
| YCbCr 4:4:4 SC (Optoinal) | 7.6% | 5.2% | 4.0% |
| Enc Time[%] | 99% | 97% | 97% |
| Dec Time[%] | 102% | 95% | 96% |

Table 10. Performance of explicit RDPCM off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 1.2% | 1.2% | 1.1% | 0.6% | 0.8% | 0.7% | 0.6% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| YCbCr 4:4:4 SC | 0.8% | 0.8% | 0.7% | 0.3% | 0.2% | 0.1% | 0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 5.7% | 6.5% | 6.7% | 6.1% | 6.8% | 9.3% | 9.1% |
| YCbCr 4:4:4 SC (Optoinal) | 3.0% | 3.0% | 2.7% | 2.8% | 2.8% | 3.5% | 3.7% |
| Enc Time[%] | 96% | 96% | 97% | 86% | 87% | 88% | 88% |
| Dec Time[%] | 84% | 86% | 87% | 95% | 96% | 96% | 95% |

Table 11. Performance of explicit RDPCM off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.5% | 4.0% | 6.0% |
| Class B | 0.0% | 0.5% | 0.6% |
| RGB 4:4:4 SC | 0.5% | 3.7% | 4.6% |
| RGB 4:4:4 Animation | 0.0% | 0.9% | 1.1% |
| YCbCr 4:4:4 SC | 0.8% | 3.1% | 4.7% |
| YCbCr 4:4:4 Animation | 0.0% | 0.6% | 0.8% |
| RangeExt | 0.0% | 0.2% | 0.3% |
| RGB 4:4:4 SC (Optional) | 1.9% | 3.0% | 4.9% |
| YCbCr 4:4:4 SC (Optoinal) | 2.8% | 3.9% | 7.4% |
| Enc Time[%] | 95% | 88% | 90% |
| Dec Time[%] | 96% | 95% | 94% |

Table 12. Performance of residual rotation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.5% | 0.5% | 0.5% | 0.3% | 0.3% | 0.2% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.3% | 0.4% | 0.4% | 0.3% | 0.3% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.4% | 0.5% | 0.5% | 0.3% | 0.3% | 0.2% | 0.2% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.2% | 0.3% | 0.5% | 0.2% | 0.1% | -0.9% | 0.1% |
| YCbCr 4:4:4 SC (Optoinal) | 0.4% | 0.5% | 0.5% | 0.3% | 0.5% | 0.1% | 0.7% |
| Enc Time[%] | 100% | 100% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 84% | 86% | 87% | 96% | 97% | 97% | 97% |

Table 13. Performance of residual rotation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.8% | 0.3% | 0.2% |
| Class B | 0.4% | 0.1% | 0.1% |
| RGB 4:4:4 SC | 0.5% | 0.7% | 0.5% |
| RGB 4:4:4 Animation | 0.6% | 0.2% | 0.1% |
| YCbCr 4:4:4 SC | 0.5% | 0.3% | 0.2% |
| YCbCr 4:4:4 Animation | 0.5% | 0.1% | 0.1% |
| RangeExt | 0.2% | 0.1% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.1% | 0.1% | 0.0% |
| YCbCr 4:4:4 SC (Optoinal) | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 97% | 97% | 97% |
| Dec Time[%] | 95% | 93% | 94% |

Table 14. Performance of single context model for significance map coding in TS off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.5% | 0.5% | 0.5% | 0.3% | 0.3% | 0.2% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.3% | 0.4% | 0.4% | 0.3% | 0.3% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.4% | 0.5% | 0.5% | 0.3% | 0.3% | 0.2% | 0.2% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.2% | 0.3% | 0.5% | 0.2% | 0.1% | -0.9% | 0.1% |
| YCbCr 4:4:4 SC (Optoinal) | 0.4% | 0.5% | 0.5% | 0.3% | 0.5% | 0.1% | 0.7% |
| Enc Time[%] | 100% | 100% | 99% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 84% | 86% | 87% | 96% | 97% | 97% | 97% |

Table 15. Performance of single context model for significance map coding in TS off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.7% | 0.6% | 0.5% |
| Class B | 0.2% | 0.1% | 0.1% |
| RGB 4:4:4 SC | 0.5% | 0.6% | 0.5% |
| RGB 4:4:4 Animation | 0.5% | 0.5% | 0.5% |
| YCbCr 4:4:4 SC | 0.5% | 0.7% | 0.5% |
| YCbCr 4:4:4 Animation | 0.6% | 0.4% | 0.5% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | -0.2% |
| YCbCr 4:4:4 SC (Optoinal) | 0.1% | 0.1% | 1.0% |
| Enc Time[%] | 102% | 101% | 100% |
| Dec Time[%] | 98% | 96% | 96% |

Table 16. Performance of intra block copy off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 11.4% | 9.8% | 8.2% | 8.4% | 7.3% | 4.9% | 4.2% |
| Class B | 0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 34.3% | 33.9% | 33.1% | 25.1% | 24.6% | 16.0% | 16.4% |
| RGB 4:4:4 Animation | 0.7% | 0.4% | 0.2% | 0.4% | 0.2% | 0.1% | 0.0% |
| YCbCr 4:4:4 SC | 33.0% | 32.2% | 30.8% | 23.5% | 23.4% | 14.8% | 15.7% |
| YCbCr 4:4:4 Animation | 0.6% | 0.5% | 0.3% | 0.3% | 0.2% | 0.1% | 0.1% |
| RangeExt | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 80.8% | 83.4% | 87.0% | 65.3% | 67.9% | 43.7% | 46.5% |
| YCbCr 4:4:4 SC (Optoinal) | 90.4% | 92.5% | 93.9% | 75.8% | 78.8% | 56.3% | 61.9% |
| Enc Time[%] | 77% | 76% | 76% | 98% | 97% | 98% | 97% |
| Dec Time[%] | 93% | 96% | 97% | 97% | 98% | 97% | 97% |

Table 17. Performance of intra block copy off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 3.8% | 2.1% | 0.9% |
| Class B | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 20.9% | 17.2% | 13.4% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 21.7% | 17.3% | 13.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 56.9% | 52.0% | 46.8% |
| YCbCr 4:4:4 SC (Optoinal) | 76.9% | 70.3% | 67.9% |
| Enc Time[%] | 71% | 94% | 94% |
| Dec Time[%] | 104% | 94% | 94% |

Table 18. Performance of Rice parameter adaptation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB 4:4:4 SC | 0.2% | 0.2% | 0.4% | 0.1% | 0.2% | 0.0% | 0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.1% | 0.2% | 0.3% | 0.3% | 0.3% | 0.0% | 0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 1.3% | 1.9% | 3.2% | 1.2% | 1.8% | 1.1% | 2.4% |
| YCbCr 4:4:4 SC (Optoinal) | 0.5% | 1.0% | 1.6% | 0.5% | 0.9% | 0.0% | 0.7% |
| Enc Time[%] | 100% | 100% | 100% | 99% | 99% | 99% | 99% |
| Dec Time[%] | 84% | 85% | 87% | 95% | 96% | 96% | 95% |

Table 19. Performance of Rice parameter adaptation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.8% | 0.3% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 4.9% | 4.6% | 4.2% |
| RGB 4:4:4 Animation | -0.1% | -0.2% | -0.2% |
| YCbCr 4:4:4 SC | 2.6% | 2.5% | 1.9% |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | 0.0% |
| RangeExt | 2.4% | 2.4% | 2.4% |
| RGB 4:4:4 SC (Optional) | 14.1% | 13.9% | 14.8% |
| YCbCr 4:4:4 SC (Optoinal) | 8.7% | 8.8% | 8.7% |
| Enc Time[%] | 99% | 98% | 97% |
| Dec Time[%] | 101% | 97% | 96% |

Table 20. Performance of cross component de-correlation off vs. on (AHG8 CTC, Lossy)

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 38.3% | 34.7% | 31.5% | 32.9% | 31.2% | 29.8% | 28.8% |
| RGB 4:4:4 Animation | 37.6% | 36.8% | 32.1% | 25.6% | 26.9% | 23.4% | 24.4% |
| YCbCr 4:4:4 SC | 4.9% | 4.9% | 4.5% | 3.8% | 4.0% | 2.7% | 3.3% |
| YCbCr 4:4:4 Animation | 2.6% | 3.2% | 3.5% | 1.0% | 1.5% | 0.3% | 1.0% |
| RangeExt | 0.9% | 1.0% | 1.0% | 0.4% | 0.5% | 0.2% | 0.3% |
| RGB 4:4:4 SC (Optional) | 39.4% | 41.1% | 41.1% | 39.1% | 42.3% | 37.6% | 41.9% |
| YCbCr 4:4:4 SC (Optoinal) | 8.2% | 7.3% | 5.9% | 6.5% | 5.9% | 7.5% | 7.5% |
| Enc Time[%] | 95% | 94% | 93% | 96% | 96% | 97% | 96% |
| Dec Time[%] | 86% | 88% | 89% | 97% | 97% | 98% | 97% |

Table 21. Performance of cross component de-correlation off vs. on (AHG8 CTC, Lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 20.8% | 14.2% | 11.9% |
| RGB 4:4:4 Animation | 14.9% | 2.9% | 1.8% |
| YCbCr 4:4:4 SC | 2.0% | 1.3% | 0.9% |
| YCbCr 4:4:4 Animation | 1.7% | 0.4% | 0.3% |
| RangeExt | 0.4% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 27.6% | 25.7% | 23.4% |
| YCbCr 4:4:4 SC (Optional) | 2.0% | 1.5% | 1.1% |
| Enc Time[%] | 91% | 92% | 93% |
| Dec Time[%] | 99% | 95% | 95% |

# Recommended profiling of range extension coding tools

The profiling of the range extension coding tools is recommended to follow a similar process to that used for HEVC v1. As shown in the test results, the coding gain of all the additional tools except cross component de-correlation is minor for natural video sequences even in 4:4:4 colour format.

There remains a market requirement for HEVC range extensions which fulfill the planned purpose of HEVC Amd 1, which was specified as follows in the Request for Amendment [10]:

“The purpose of this amendment of ISO/IEC 23008-2 is to provide the capability to support high fidelity video signals. In high end consumer and professional environment, larger sample accuracy and alternative chroma sampling structures such as 4:4:4 and 4:2:2 are used. This amendment is intended to extend the capability of ISO/IEC 23008-2 for those applications.”

Considering that the primary goal of HEVC range extensions is to support bit-depth higher than 10 bits and non-4:2:0 colour formats, it is suggested to include only cross component de-correlation in Main 4:4:4 10/12 to be included in Amd1, which reaches FDAM in April 2014. Instead, it is suggested to develop SC 4:4:4 Profiles, including SC coding tools, in coordination with the call for proposals for coding of screen content and medical visual content planned for January 2014. It is suggested that all of the coding tools in the draft RExt text should be included in the initial test model for this new activity

The recommended mapping between profiles and coding tools is summarized in Table 22.

Table 22. Mapping of coding tools onto Main profiles and initial TM for SC profiles

| Range extension coding tools | Main 12 | Main 4:2:2 |  | Main 4:4:4 |  | TM1 for SC 4:4:4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 | 12 | 10 | 12 | 8 | 10 |
| Implicit RDPCM |  |  |  |  |  |  |  |
| Explicit RDPCM |  |  |  |  |  |  |  |
| Residual rotation |  |  |  |  |  |  |  |
| Single ctx for TS |  |  |  |  |  |  |  |
| Intra block copy |  |  |  |  |  |  |  |
| Rice parm. adapt. |  |  |  |  |  |  |  |
| Cross comp. de-cor. |  |  |  |  |  |  |  |

# Conclusion

In this contribution, it is suggested that the decision process for profiling of range extension coding tools should follow a similar process to that used for HEVC v1.

Based on the coding gain and complexity trade-off for the additional range extension coding tools, evaluated under HEVC range extension common test condition, it is suggested that only cross component de-correlation is included in Main 4:4:4 10/12 profiles to be included in Amd 1 which reaches FDAM in April 2014. It is suggested that this proposed mapping between HEVC range extension profiles and coding tools is adopted into the next version of HEVC range extension text and reference software.

It is also suggested that SC 4:4:4 profiles should be developed, in coordination with the call for proposals for coding of screen content and medical visual content planned for January 2014. It would be appropriate to include all the coding tools considered in this proposal in the initial test model for this new activity.

# References

- M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “RCE2 subtest C.2: Extension of residual DPCM to lossy coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0052, Jul. 2013.

- M. Naccari, M. Mrak, “RCE2: Experimental results for Test C.1,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0074, Jul. 2013.

- D. He, J. Wang, G. Martin-Cocher, “Rotation of Residual Block for Transform Skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0093, Jun. 2012.

- T. Tsukuba, T. Yamamoto, “Constant coefficient context for intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0069, Jun. 2012.

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

- J. Sole, R. Joshi, M. Karczewicz, “RCE2 Test A1: Simplified update of the coefficient level Rice parameter,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0206, Oct. 2013.

- W. Pu, W.-S. Kim, J. Chen, J. Sole, M. Karczewicz, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0202, Oct. 2013.

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1006, Oct. 2013.

- MPEG Video and JCT-VC, “Request for ISO/IEC 23008-2:201X/Amd.1”, ISO/IEC document JTC1/SC29/WG11/N13344, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).