| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0107 |
| --- | --- |

| Title: | AHG5: Tools for lossless coding of medical image/video content in RExt profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Philipp Wojaczek, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Uwe-Erik Martin, Nikolaus Wirsz Siemens Healthcare Imaging and Therapy Systems Hartmannstr. 16 91052 Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This contribution analyses and discusses different coding tools currently under evaluation for incorporation into HEVC RExt profiles for compression of medical image/video content. Based on the analysis, especially the adoption of RDPCM for 12-bit and 16-bit monochrome profiles, for a potential 4.4:4 profile, and also for related higher profiles is recommended.

# Introduction and background

In [1], we proposed different future HEVC profiles in the context of the Range Extension work. The proposed future profiles are summarized in Table 1.

Table 1.  Proposed RExt profiles for monochrome and 4:4:4 content.

This contribution analyses the compression efficiency of different tools relevant for these profiles for coding of medical image/video content.

The sequences used were taken from [2]. Note that some of these sequences were adopted for the definition of test conditions in the context of RCE1 / AHG18.

# Lossless compression of medical image/video

We analysed the compression efficiency of different coding tools in HM-12.0+RExt-4.1 software for the lossless compression of medical image/video content, i.e.:

- intra residual differential pulse code modulation (“RDPCM intra”),

- inter residual differential pulse code modulation (“RDPCM inter”),

- residual rotation, and

- intra block copy.

The compression results are depicted in Figure 1, Figure 2, and Figure 3 for All-Intra, Random-Access, and Low-Delay B configurations, respectively.

Figure 1.  Bitrate savings of different tools in HM-12.0+RExt-4.1 for All-Intra configuration

Figure 2.  Bitrate savings of different tools in HM-12.0+RExt-4.1 for Random-Access configuration

Figure 3.  Bitrate savings of different tools in HM-12.0+RExt-4.1 for Random Low-delay B configuration

The analysis reveals that RDPCM contributes most to the total coding gain while other tools (i.e., residual rotation and intra block copy) do not give comparable gains.

# Recommendation

In the light of the preceding analysis of the different coding tools, we propose the adoption of RDPCM for 12-bit and 16-bit monochrome profiles and for a potential 4:4:4 profile due to their relative low complexity and their superior compression efficiency. In addition, RDPCM should also be incorporated in higher profiles (e.g., profiles with extended color space) in an onion shell fashion.

# References

- P. Amon, A. Hutter, U.-E. Martin, N. Wirsz, “Coding of medical mixed content and medical visual content with bit depth beyond 10 bits”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-O0173/m30935, Geneva, Switzerland, Oct./Nov. 2013.

- P. Amon, A. Hutter, U.-E. Martin, B. Heigl, “AHG22: Sequences with medical mixed content and medical visual content with bit depth beyond 10 bits for HEVC development”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-O0172/m30934, Geneva, Switzerland, Oct./Nov. 2013.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# Patent rights declaration(s)

Siemens may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).