| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0108 |
| --- | --- |

| Title: | RCE4: Test 1. Major-color-based screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xun Guo Yan Lu Shipeng Li No. 5 Danling Street, Beijing | Tel: Email: | +86-10-59172760 xunguo@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution presents the results on RCE4 Test 1, a major-color-based method for screen content coding. In specific, pixels in a CU are classified into major colors and escape pixels, and converted into index map. The major color table, index map and escape pixels are then transmitted. It is reported that, for lossy coding, 7.1%, 9.4% and 11.2% bit-rate saving is observed on SC YUV 444 sequences for AI-Main, AI-High and AI-Super-High, respectively. And average bit-saving of 9.2%, 8.2% and 7.2% is observed for SC YUV444 sequences for AI, RA and LD configurations, respectively.

# Introduction

In Geneva meeting, we proposed a major-color-based screen content coding method [1]. In this method, pixels of a CU are classified into major colors and escape pixels. Major colors are the most representative pixel values in a CU like a palette, which cover the majority pixels of the CU. And escape pixels are those do not belong to the major colors. For each pixel, a color index is transmitted to indicate whether it is a major-color pixel or not, and, if it is, which major color it belongs to. If it is an escape pixel, the pixel value is transmitted. A flag is signaled in CU level to indicate if this method is used. This contribution presents the results of the proposed method on top of the latest software HM-12.1+RExt-5.1.

# Algorithm description

## Color classification

A very simple but efficient histogram based algorithm is used to classify the pixels. In specific, the most significant N peak values in the histogram are selected as major colors. The pixel values that are close to a major color will be quantized to the major color. Other pixels which do not belong to any major color sets are escape pixels, which would also be quantized before coding. For lossless coding, no quantization is used.

## Predictive coding of color index

By using classification, pixels of a CU can be converted into color indices. After that, the major color number and values are coded. Then, the color indices are coded as follows.

- For each pixel line, a flag is signalled to indicate the coding mode. There are three modes: horizontal mode, vertical mode and normal mode.

  - If the mode is horizontal mode, the whole line shares the same color index. In this case, the color index is transmitted.

  - If the mode is vertical mode, the whole line is the same with the above line. In this case, nothing is transmitted. The current line copies the color indices of the above line.

  - If the mode is normal mode, a flag is signalled for each pixel position to indicate whether it is the same with one of the left and above pixels. If not, the index itself is transmitted.

- If the pixel is escape pixel, the pixel value is transmitted.

# Experimental results

The proposed method is implemented on top of HM-12.1+RExt-5.1. The experiments are conducted by following RCE4 test conditions defined in JCTVC-O1124 [2].

## Coding performance

Table 1. Results for lossy coding

Table 2. Results for lossless coding

## Complexity assessment

### Throughput

In parsing stage, no pixel reconstruction is needed. Therefore, the parsing of a pixel position is independent and worse case of the current throughput would not be reduced. In decoding stage, pixels can be reconstructed independently without referencing adjacent pixels. So the decoding process can be fully parallel.

### Number of operations

In parsing stage, all color indices and quantized escape pixels can be obtained. Therefore, in decoding stage, a LUT operation, which maps the color index into pixel value, is needed for a major-color pixel, and a shift operation for de-quantization is needed for an escape pixel.

## Subjective assessment

Subjective assessment is also conducted according to the requirement of RCE4 description. ClassF and SC 444 sequences are chosen for the assessment. Each QP point for AI, RA and LD is tested. In general, the proposed method has similar visual quality to the anchor. No special artifacts that may not appear in regular video coding are observed.

# Conclusions

This contribution reports the results of RCE4 Test 1 on major-color-based screen content coding method. The results show that the proposed method can achieve significant bit-rate saving. In addition, it has comparable visual quality to anchor and low decoding complexity.

# References

- X. Guo, B. Li, J. Xu, Y. Lu, S. Li, F. Wu, “AHG8: Major-color-based screen content coding,” JCTVC-O0182, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE4): Palette Coding for Screen Content,” JCTVC-O1124, Geneva, CH, 23 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

| … |  |
| --- | --- |

7.3.2.2 Sequence parameter set RBSP syntax

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( screen_coding_enabled_flag) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_scm_flag[ x0 ][ y0 ] ) { |  |
| scm_info_coding(x0, y0, nCbS, nCbS) |  |
| } else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag && !intra_scm_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] && !intra_scm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |

SCM info coding

| scm_info_coding(x0, y0, CbWidth, CbHeight) { |  |
| --- | --- |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( major_color_pred[ cIdx ] ) { |  |
| major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } |  |
| } |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 3 )?1:3; cIdx++) { |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ 0 ] | ae(v) |
| } |  |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] = color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i-1 ][ j ] |  |
| } |  |
| } |  |
| } else { //NOR_MODE |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] == NO_PREDICTION) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| if(color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][0] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ]) { |  |
| for(j = 0; j < CbWidth; j++) |  |
| quant_escape_value[ cIdx + cSubIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, intra_screen_coding_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

intra_screen_coding_enabled_flag equal to 1 specifies that intra screen coding may be used for intra blocks. intra_screen_coding_enabled_flag equal to 0 specifies that intra screen coding is not applied. When not present, the value of intra_screen_coding_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in screen coding mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in screen coding mode. When intra_scm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the above or left CU. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] specifies the prediction direction of the n-th major color for color component cIdx. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies current major color is the same as the corresponding major color in the above CU. When it equals to 0, current major color is the same as the corresponding major color in the left CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_mode[ x0 ][ y0 ][ i ] equal to HOR_MODE specifies that the line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][i] equal to VER_MODE specifies that the line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ i ] equal to NOR_MODE specifies that the line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current pixel in position ( i, j ). index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to INDEX_PREDICTION_ABOVE specifies that index value of current pixel is the same as the above one. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to 0 specifies that index value of current pixel is 0. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to NO_PREDICTION specifies that index value of current pixel cannot be derived from the above or the left pixel.

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape pixel value for color component cIdx in at the location ( i, j ). The variable bitEscapeQuant is define as Table

- escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] is set equal to

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ]<<(bitDepth – bitEscapeQuant)

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the pixel at the location( i, j ) for color component cIdx.

8.4.1 General decoding process for coding units coded in intra prediction mode

Depending on the values of pcm_flag[ xCb ][ yCb ], intra_scm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows:

For each i, j = 0..nCbS – 1

–  If color_index[0][i][j] is not equal to major_color_num[0],

SL [ xCb + i ][ yCb + j ] = major_color_value[0][ color_index[0][i][j] ]

–  Otherwise

SL [ xCb + i ][ yCb + j ] = escape_value [0][i][j]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

–  If color_index[1][i][j] is not equal to  major_color_num[1],

SCb [ xCb + i ][ yCb + j ] = major_color_value[1][ color_index[1][i][j]

–  Otherwise

SCb [ xCb + i ][ yCb + j ] = escape_value [1][i][j]

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

–  If color_index[2][i][j] is not equal to major_color_num[2],

SCr [ xCb + i ][ yCb + j ] = major_color_value[2][ color_index[2][i][j]

–  Otherwise

SCr [ xCb + i ][ yCb + j ] = escape_value [2][i][j]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xC ][ yC ] is equal to 0 intra_scm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xBS is set equal to xC + ( nS  >>  1 ) * ( blkIdx % 2 ).

- The variable yBS is set equal to yC + ( nS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xBS, yBS ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

Table 9-4 Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag |  | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9- | 0..2 | 3..5 | 6..8 |
|  | intra_scm_flag[ ][ ] | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |

Table 9-XX – Values of initValue for ctxIdx of intra_scm_flag

| Initialization variable | ctxIdx of intra_scm_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

Table 9-34 Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_mc_flag | FL | cMax = 1 |
|  | intra_scm_flag | FL | cMax = 1 |
|  | major_color_num | FL | cMax = 15 |
|  | major_color_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | major_color_pred_flag | FL | cMax = 1 |
|  | major_color_pred_dir | FL | cMax = 1 |
|  | line_mode | FL | cMax = 2 |
|  | index_pred_symbol | FL | cMax = 2 |
|  | quant_escape_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | color_index | FL | cMax = 16 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| zpred_mode_flag | 0 | na | na | na | na | na |
| intra_scm_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_num | bypass | bypass | bypass | bypass | na | na |
| major_color_value | bypass | bypass | bypass | bypass | bypass | bypass |
| major_color_pred_flag | 0,1 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_pred_dir | 0 | na | na | na | na | na |
| line_mode | 0,1 | 0,1,2 | na | na | na | na |
| index_pred_symbol | 0,1 | 0,1,2 | na | na | na | na |
| quant_escape_value | bypass | bypass | bypass | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 9-1 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_scm_flag[ x0 ][ y0 ] | intra_scm_flag[ xNbL ][ yNbL ] | intra_scm_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |