The proposed modifications are based on Range Extensions text specification: Draft 5 (JCTVC-O1005). The inserted parts are marked in yellow and deleted parts are marked in pink.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| multiple_sao_param_sets_enabled_flag | u(1) |
| } |  |
| } |  |
| ……. |  |
| } |  |

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| if(multiple_sao_param_sets_enabled_flag) |  |
| number_sao_param_sets | ue(v) |
| for(saoId=0; saoId < number_sao_param_sets; saoId++) { |  |
| slice_sao_luma_flag[saoId] | u(1) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_flag[saoId] | u(1) |
| } |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| ……. |  |
| } |  |

#### 7.3.8.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| for(saoId=0; saoId < number_sao_param_sets; saoId++) { |  |
| if( slice_sao_luma_flag[saoId] ¦ ¦ slice_sao_chroma_flag[saoId] ) |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| } |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| } |  |

#### 7.4.3.2 Sequence parameter set RBSP semantics

multiple_sao_param_sets_enabled_flag: equal to 1 specifies that the sample adaptive offset process can be applied to the reconstructed picture after the deblocking filter process more than once in each CTU. sample_adaptive_offset_enabled_flag equal to 0 specifies that the sample adaptive offset process is applied to the reconstructed picture no more than once after the deblocking filter process in each CTU.

7.4.7.1 General slice segment header semantics

number_sao_param_sets specifies the number of the SAO parameter sets decoded by the syntax structure sao( ) in each CTU. When not present, the value of number_sao_param_sets is inferred to be equal to 1.

slice_sao_luma_flag[saoId] equal to 1 specifies that SAO with process index saoId is enabled for the luma component in the current slice; slice_sao_luma_flag[saoId] equal to 0 specifies that SAO with process index saoId is disabled for the luma component in the current slice. When slice_sao_luma_flag[saoId]  is not present, it is inferred to be equal to 0.

slice_sao_chroma_flag[saoId] equal to 1 specifies that SAO with process index saoId is enabled for the chroma component in the current slice; slice_sao_chroma_flag[saoId] equal to 0 specifies that SAO with process index saoId is disabled for the chroma component in the current slice. When slice_sao_chroma_flag[saoId] is not present, it is inferred to be equal to 0.