| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0109 |
| --- | --- |

| Title: | AHG8: Sample adaptive offset with multiple parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable more than one SAO filtering operations performed on each CTU. The additional SAO filtering operation is performed on the output pixels processed by the previous SAO filtering operation in each CTU. The proposal re-uses the existing SAO tools and syntax structure to support the additional SAO filtering operations in each CTU, reportedly involving the very limited modifications to the current standard. It is reported to achieve 2.4%, 1.3%, and 1.0% Luma BD-rate savings for YCbCr 4:4:4 SC sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. The proposal further combined with the formerly proposed SAO slice header syntax reportedly achieves 2.6%, 1.6%, and 1.9% Luma BD-rate savings for YCbCr 4:4:4 SC sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

The sample adaptive offset method (SAO) in the current HEVC standard provides a rich set of the tools for filtering the reconstructed pixels, including the edge offset (EO) tools for correcting pixel values along different orientation and the band offset (BO) tools for correcting the pixel values corresponding to different intensity bands. However, the current standard can only support the syntax to select one SAO tool from the SAO tool set even though some other SAO tools are also found beneficial for processing the pixels from the current CTU.

# Proposed method

This contribution proposes to support more than one SAO filtering operations performed on each CTU. The additional SAO filtering operations in each CTU is performed on the output pixels processed by the previous SAO filtering operation. The additional SAO filtering operations just re-use the existing SAO tools. The existing SAO syntax is also re-used for coding the additional SAO parameters sets corresponding to the additional SAO filtering operations.

A new syntax flag is added to the SPS extension to enable the use of the additional SAO filtering operations. A new syntax element is added to the slice segment header to indicate the maximum number of the SAO filtering operations for each CTU in the current slice segment.

The proposed text modifications to the HEVC Range Extension text specification is provided in the attached file.

# Experimental results

The proposed method has been implemented using reference software HM-12.1+RExt-5.1 as the code base. The simulation is conducted according to the common test condition specified in [1]. The simulation results are summarized in Table 1 using the proposed algorithm with the maximum number of the SAO parameter sets equal to 5 in each CTU and summarized in Table 2 with the maximum number of the SAO parameter sets equal to 4 and 2 for the Intra slice and Inter slice, respectively.

Table 1: Summary of the BD-rate results by the proposal with the maximum number of the SAO parameter sets equal to 5 in each CTU versus the anchor results under the CTCs

Table 2: Summary of the BD-rate results by the proposal with the maximum number of the SAO parameter sets equal to 4 and 2 for the Intra slice and the Inter slice, respectively, in each CTU versus the anchor results under the CTCs

The software implementation of the proposed algorithm has made modifications to the default macro settings of the HM-12.1+RExt-5.1 software, listed below:

#define HM_CLEANUP_SAO                                               0

#define SAO_ENCODING_CHOICE                                         0

#define RExt__BACKWARDS_COMPATIBILITY_HM_TICKET_1149        0 ///< Maintain backwards compatibility with HM for ticket 1149 (allow the encoder to test not using SAO at all)

Our software has further integrated the three bugfixes reported in [2], which have not been updated for the previous SAO code base in HM-12.1+RExt-5.1. The BD-rate results of HM-12.1+RExt-5.1 using this modified macro setting is summarized in Table 3 for further reference.

Table 3: Summary of the BD-rate results of HM-12.1+RExt-5.1 using the modified macro setting versus the anchor results under the CTCs

In our former contribution [3], we propose to add a new set of syntax elements to the slice segment header such that the SAO coding tool can be effectively adapted to different sample bit depths, coding bitrates and sampling formats while making no significant modifications to the existing SAO method in the current HEVC. In Table 4, we summarize the BD-rate results under the AHG8 CTCs for the algorithm combining the proposed slice segment syntax in [3] and our proposed multi-parameter support with the maximum number of the SAO parameter sets equal to 4 and 2 for the Intra slice and Inter slice, respectively. Please note the runtime results in Table 4 are not accurate.

Table 4: Summary of the BD-rate results for our proposal combining combining the proposed slice segment header syntax and the multi-parameter support under the AHG8 CTCs

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, et al, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1006, Geneva, CH, 23 Oct. – 1 Nov. 2013.

- C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei, “SAO software cleanup and non-normative encoder-only bug-fixes,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0230, Vienna, AT, 25 July – 2 Aug. 2013.

- S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei , “AHG5: Modified SAO for range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0246, Vienna, AT, 25 July – 2 Aug. 2013.