| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0109 |
| --- | --- |

| Title: | AHG8: Sample adaptive offset with multiple parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable more than one SAO filtering operations performed on each CTU. The additional SAO filtering operation is performed on the output pixels processed by the previous SAO filtering operation in each CTU. The proposal re-uses the existing SAO tools and syntax structure to support the additional SAO filtering operations in each CTU, reportedly involving the very limited modifications to the current standard. It is reported to achieve 2.4%, 1.3%, and 1.0% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

The sample adaptive offset method in the current HEVC standard provides a rich set of the tools for filtering the reconstructed pixels, including the edge offset (EO) tools for correcting pixel values along different orientation and the band offset (BO) tools for correcting the pixel values corresponding to different intensity bands. However, the current standard can only support the syntax to select one SAO tool from the SAO tool set even though some other SAO tools are also found beneficial for processing the pixels from the current CTU.

# Proposed method

This contribution proposes to support more than one SAO filtering operations performed on each CTU. The additional SAO filtering operations in each CTU is performed on the output pixels processed by the previous SAO filtering operation. The additional SAO filtering operations just re-use the existing SAO tools. The existing SAO syntax is also re-used for coding the additional SAO parameters sets corresponding to the additional SAO filtering operations.

A new syntax flag is added to the SPS extension to enable the use of the additional SAO filtering operations. A new syntax element is added to the slice segment header to indicate the maximum number of the SAO filtering operations for each CTU in the current slice segment.

The proposed text modifications to the HEVC Range Extension text specification is provided in the attached file.

# Experimental results

The proposed method has been implemented using reference software HM-12.1+RExt-5.1 as the code base. The simulation is conducted according to the common test condition specified in [1]. The simulation results are summarized in Table 1 using the proposed algorithm with the maximum number of the SAO parameter sets equal to 5 in each CTU.

Table 1: Summary of the BD-rate results by the proposal versus the anchor results under the CTCs

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, et al, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1006, Geneva, CH, 23 Oct. – 1 Nov. 2013.