| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0111 |
| --- | --- |

| Title: | Non-RCE3: On initialization of the block vector predictor for Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Patrice Onno Tangi Poirier Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the block vector prediction scheme for the Intra Block Copy method. In the adopted Intra Block Copy prediction method, the block vector predictor is reset to (0, 0) for each CTB. In the proposed method, the vector predictor is initialized by a default value corresponding to a non-null value. In average, it is reported that the proposed initialization method (-2*N,0) gives -0.1% gain for the AI Main-tier and -0.1% for the LB High-tier. It is also reported that the proposed change has no impact on the Lossless configuration.

# Introduction

In the current Intra Block Copy method of HEVC Range Extensions [1], the displacement vector is now predicted since some important correlations exist between the current vector and its neighboring vectors.

During the last meeting, the method of JCTVC-O0122 [2] was adopted in order to predict the displacement block vector of the current CU by using the displacement vector coded of the previously IBC-coded CU. This method was considered as the best tradeoff between complexity and coding efficiency performance.

# Proposed method

The proposed modification consists in modifying the initialization value of the displacement vector predictor to clean the current design so that the predictor is really a true predictor and also to slightly improve the coding efficiency.

In the current Range Extension draft [1], the displacement vector predictor is reset to (0, 0) at each CTB. In the proposed method, the displacement vector is reset to (K, 0) where K is a variable depending of the size of the current Coding Unit.

In this document, the values of K is set to -2*N where N is the size of the current CU.

# Syntax change for the prediction

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the block vector bvIntra.

The variable nCbS is set equal to 1  <<  log2CbSize.

The variable InitBvIntra[0] = - 2* nCbS;

The variable InitBvIntra[1] = 0;

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

The block vector bvIntra is derived by the following ordered steps, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xCb ][ yCb ][ compIdx ] + InitBvIntra[ CompIdx ]

- Otherwise, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xCb ][ yCb ][ compIdx ] + BvpIntra[ compIdx ]	(8-1)

# Results

## Result for Experiment 1 where K = - 2N

The proposed method was implemented in the Range extension software version 5.1. The results were generated with the conditions as described in the Common Test Conditions [3] for the Range Extensions. Table 1 gives the results for Lossy configuration.

The proposed method gives an average gain (excluding the optional sequences) of -0.1% for the AI Main-tier and the Low-delay High-tier.

Table 1: Results for the lossy configuration.

Table 2 shows that there is no impact for the lossless configuration.

Table 2: Results for the lossless configuration.

# Conclusion

In this contribution, we propose to modify the initial value of the displacement vector predictor for Intra Block Copy method. Instead of being set to (0,0) which does not correspond to a real predictor, the initial value of the displacement vector is set to (-2N,0).

As described along this contribution, this small modification of the prediction scheme of the Intra Block Copy brings a cleaner design for the displacement vector prediction and provides a small coding efficiency gain.

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 5”, JCTVC-O1005, 15th meeting, Geneva, October/November 2013

- G. Laroche, T. Poirier and C. Gisquet, “AHG5: Vector prediction for Intra Block Copy”, JCTVC-O0122, 15th meeting, Geneva, October/November 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-O1006, 15th meeting, Geneva, October/November 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).