| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0112 |
| --- | --- |

| Title: | AHG8: Chroma interpolation filters for Lossless compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Christophe Gisquet Tangi Poirier Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a simplification of the interpolation filters by using shorter interpolation filters for the Luma component for the Lossless case. This contribution proposes to use the HEVC Chroma filters for both the Luma and Chroma components for the Lossless case. It is reported that the complexity and the memory bandwidth is significantly reduced without affecting the coding efficiency coding.  The coding efficiency gain is about -0.1% in RA and -0.2% in LB. It is also reported that the encoding time is reduced and represent 93% in RA and 92% in LB. The decoding time is also significantly reduced to 89% in RA and 86% in LB.

# Introduction

In the current design of HEVC Range Extensions [1], the interpolations filters are the same as the ones used for the HEVC version 1.  More precisely the interpolation filters for Luma and Chroma depend on the sub-sample position needed for the Luma and Chroma component.

The Luma interpolation filters are 2D separable DCT-based interpolation filters (DCT-IF). They enable to reach a precision of 1/4-sample. They are 8-tap long and are coded on 6 bits. They are defined as follows:

m_lumaFilter[LUMA_INTERPOLATION_FILTER_SUB_SAMPLE_POSITIONS][NTAPS_LUMA] =

{

{  0, 0,   0, 64,  0,   0, 0,  0 },  // position: 1/1

{ -1, 4, -10, 58, 17,  -5, 1,  0 },  // position: 1/4

{ -1, 4, -11, 40, 40, -11, 4, -1 },  // position: 2/4 or 1/2

{  0, 1,  -5, 17, 58, -10, 4, -1 }   // position: 3/4

};

The Chroma interpolation filters are 2D separable DCT-based interpolation filters (DCT-IF). They enable to reach a precision of 1/8-sample. They are 4-tap long and are coded on 6 bits. They are defined as follows:

m_chromaFilter[CHROMA_INTERPOLATION_FILTER_SUB_SAMPLE_POSITIONS][NTAPS_CHROMA]=

{

{  0, 64,  0,  0 },  // position: 1/1

{ -2, 58, 10, -2 },  // position: 1/8

{ -4, 54, 16, -2 },  // position: 2/8 or 1/4

{ -6, 46, 28, -4 },  // position: 3/8

{ -4, 36, 36, -4 },  // position: 4/8 or 1/2

{ -4, 28, 46, -6 },  // position: 5/8

{ -2, 16, 54, -4 },  // position: 6/8 or 3/4

{ -2, 10, 58, -2 }   // position: 7/8

};

# Proposed method

This contribution proposes to simplify the interpolation step for HEVC Range Extension by using shorter interpolations filters for the Lossless case for the Luma component. For lossless compression, the interpolation filters do not need to be as accurate as for the lossy case since the reference pictures correspond to the original picture.

In this proposal, instead of using the interpolation filters given in section 1, the Luma filters are replaced by the shorter Chroma filters for the corresponding sub-sample position.

The proposed Luma interpolation filters are thus the following ones:

m_LosslessLumaFilter[LUMA_SUB_SAMPLE_POSITIONS][NTAPS_LUMA] =

{

{  0, 64,  0,  0 },  // position: 1/1

{ -4, 54, 16, -2 },  // position: 1/4

{ -4, 36, 36, -4 },  // position: 2/4 or 1/2

{ -2, 16, 54, -4 }   // position: 3/4

};

The Chroma interpolation filters remain the same:

m_LosslessChromaFilter[CHROMA_SUB_SAMPLE_POSITIONS][NTAPS_CHROMA] =

{

{  0, 64,  0,  0 },  // position: 1/1

{ -2, 58, 10, -2 },  // position: 1/8

{ -4, 54, 16, -2 },  // position: 2/8 or 1/4

{ -6, 46, 28, -4 },  // position: 3/8

{ -4, 36, 36, -4 },  // position: 4/8 or 1/2

{ -4, 28, 46, -6 },  // position: 5/8

{ -2, 16, 54, -4 },  // position: 6/8 or 3/4

{ -2, 10, 58, -2 }   // position: 7/8

};

# Results

The following results have been generated by using the HM-12.1+RExt-5.1 reference software under the lossless configurations provided by the AHG8 [2]. The results show a significant reduction of the runtimes for both the encoder and the decoder. Furthermore, as illustrated by Table 1, the coding efficiency is even better when shorter filters are used for the Luma component compared to the anchors of HM-12.1+RExt-5.1 reference software.

Table 1: Results for the lossless configuration.

# Conclusion

In this contribution, we propose to use the same interpolation filters for the Luma and Chroma components for the lossless case. This modification enables to considerably reduce the memory bandwidth and brings some coding efficiency gains.

.

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 5”, JCTVC-O1005, 15th meeting, Geneva, October/November 2013

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-O1006, 15th meeting, Geneva, October/November 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).