| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0113 |
| --- | --- |

| Title: | Non-RCE4: Run coding for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The RCE4 studies several proposals related to palette-like coding methods. In particular, Test2 related to contribution JCTVC-O0218 evaluates a palette coding tool which uses 2 prediction modes: The “Run mode” and the “Copy above mode”. For both modes a run value is transmitted indicting the number of consecutive locations with the same palette index for the Run mode or the number of consecurive locations with the same palette index as the position in the above row. The run is coded with the same function as the absolute remaining coefficient in the implementation of JCTVC-O0218. In this contribution 2 modifications for the coding of the Run are proposed. The first one consists in adapting the Rice Golomb parameter used for the binarization of the run value. And the second one consists in avoiding sometimes the coding of the run value. It is reported that these modifications provide gains for all configuration. When the two methods are combined, the gains are, respectively for AI/RA/LDB configurations, 4.2%/4.0%/3.0% for SC classes compared to the JCTVC-O0218 implementation integrated in HM-12.1+RExt-5.1.

# Introduction

The palette mode coding presented in JCTVC-O0218 [1] uses 2 modes to code the palette index: the “Run mode” and the “Copy above mode”. For the Run mode, a palette index is transmitted followed by a run value, the “Palette run”, which indicates the number of following palette indexes with the same value. For the Copy above mode, the “Copy_run” is transmitted to indicate that, for the following positions (including the current one), the palette index is equal to the palette index of the one that is at the same location in the row above.

In JCTVC-O0218 implementation, the “Palette run” and the “Copy_run” are coded with the same Binarization process as the coeff_abs_level_remaining with the Rice Golomb parameter cRiceParam set equal to 3. But the “Palette run” and the “Copy_run” are not correlated. Moreover, the “Palette run” is often related to the palette index encoded. In this contribution we propose to modify the coding of the run value for both the Run mode and the Copy above mode.

# Proposed modifications

In order to exploit correlations between the run modes and their respective run values, and the correlation between the palette index and the run value for the Run mode we proposed 2 modifications. The first one consists in modifying the Rice Golomb parameter “cRiceParam_run” used for the coding of the run value and the second one consists in avoiding the coding of the run according to the palette value for the Run mode.

## Rice Golomb parameter

For the first modification, the cRiceParam_run for the binarization of the run value is set equal to 2 for the Copy above mode and for the Run mode with the previous palette index equal to 0. Otherwise the cRiceParam_run is set equal to 0. This can be summarized as the following:

If(!run_mode_flag || palette_index == 0)

cRiceParam_run = 2

else

cRiceParam_run = 0

## Limit run

The second modification only applies for the Run mode. The method consists in indicating the maximum palette index value (Limit_run) for which the run is coded. Thus, for all palette index values superior to this Limit_run, the run is not coded.

The syntax element Limit_run is decoded just after the palette_size_minus1 for all Palette CU. It is coded with a unary max code (CABAC bypass) with the maximum value equal to the Palette_size.

On encoder side, the Limit_run is determined a posteriori. It means that, during the selection between the Run mode and the Copy above mode, the rates of using or not the proposed method are estimated. At the end of this process, the best Limit_run value is determined and the syntax of the current palette CU is updated.

# Experimental results

For the proposed method, we have integrated the source code of JCTVC-O0218 in the HM-12.1+RExt-5.1. This is the Anchor of our following results.

## Rice Golomb parameter

The first modification gives gains for respectively AI/RA/LDB configurations of 3.2%/1.6%/2.1% for SC classes compared to JCTVC-O0218 implementation.

Table 1 Results of the adaption of the cRiceParam_run for the Lossy configurations compared to the Rext5.1+JCTVC-O0128.

For the Lossless coding the gain is higher for SC sequences with 4.1%/3.1%/2.7% gains for respectively AI/RA/LDB configurations.

Table 2 Results of the adaption of the cRiceParam_run for the Lossless configurations compared to the Rext5.1+JCTVC-O0128.

## Limit run

The first modification gives gains for respectively AI/RA/LDB configurations of 3.7%/1.9%/2.6% for SC classes compared to JCTVC-O0218 implementation.

Table 3 Results of the adaption of the limit run method for the Lossy configurations compared to the Rext5.1+JCTVC-O0128.

For the Lossless coding the gain is higher for SC sequences with 4.7%/2.3%/2.1% gains for respectively AI/RA/LDB configurations.

Table 4 Results of the adaption of the limit run method for the Lossless configurations compared to the Rext5.1+JCTVC-O0128.

## Rice Golomb parameter + Limit Run

When both modifications are combined the gains are respectively for AI/RA/LDB configurations: 4.2%/4.0%/3.0% for SC classes compared to JCTVC-O0218 implementation.

Table 5 Results of the 2 proposed methods for the Lossy configurations compared to the Rext5.1+JCTVC-O0128.

## Limit runon top of RCE4 Test 2

The first modification gives gains for respectively AI/RA/LDB configurations of 0.7%/0.8%/0.3% for SC classes compared to RCE4 Test 2.

Table 6 Results of the adaption of the limit run method for the Lossy configurations compared to the RCE4 Test 2.

The gain for Lossless coding is about the same.

# Draft texts

## Rice Golomb parameter

#### Binarization process for run

Input to this process is a request for a binarization for the syntax element run, the run_mode_flag, and the palette_index.

Output of this process is the binarization of the syntax element.

The variable cRiceParam_run and cLastRiceParam is derived as follows:

- If run_mode_flag is set equal to 0, cRiceParam_run is set equal to 2

- Otherwise, if the palette_index is set equal to 0, cRiceParam_run is set equal to 2

- Otherwise cRiceParam_run is set equal to 0

## Limit run

Palette mode JCTVC-O0218

Proposal

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if (palette_mode_flag) { |  |
| limit_run | ae(v) |
| palette_size_minus1[x0][y0] | ae(v) |
| for (cIdx = 0; cIdx < 3; cIdx ++) |  |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][ cIdx][i] | ae(v) |
| scanPos = 0 |  |
| while (scanPos < nCbS * nCbS) { |  |
| run_mode_flag | ae(v) |
| if (run_mode_flag) |  |
| { |  |
| palette_index | ae(v) |
| run=0 |  |
| if (palette_index<=limit_run) |  |
| { |  |
| run | ae(v) |
| } |  |
| else { |  |
| run | ae(v) |
| } |  |
| …… |  |
| else { |  |
| if( slice_type != I ) |  |
| ….. |  |
| } |  |

# Conclusion

This contribution proposed 2 modifications to improve the coding of the run of the Palette Mode Coding presented in JCTVC-O0128. According to the interesting gains without adding CABAC context, we recommend to consider these modifications if the Palette Mode coding is adopted for the Range Extensions.

# Reference

- L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm), “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).