| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0114 |
| --- | --- |

| Title: | Non-RCE4: Palette Prediction for Palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The RCE4 studies several proposals related to palette-like coding methods. In contribution JCTVC-O0218 the palette is transmitted for each CU. This contribution proposes to predict the current Palette using the last decoded Palette. The Palette predictor is reset for each CTB line. It is reported that these modification provides gains for respectively AI/RA/LDB configurations, 2.8%/1.4%/1.6% for SC classes, and up to 8.9%/7.1%/6.1% on optional classes compared to the JCTVC-O0218 implementation integrated in HM-12.1+RExt-5.1, and 1.2%/ 1.0%/ 0.5% for SC classes, and up to 4.4%/ 3.4%/ 3.4% on optional classes compared to RCE4 Test 2.

# Introduction

In the palette mode coding presented in JCTVC-O0218 [1], the palette is transmitted for each Palette CU. The palette size minus 1 (to avoid only one palette element) is transmitted followed by the elements in the palette. In this contribution we propose to predict the current palette using the palette used for the last decoded palette CU.

# Proposed Method

## Palette Predictor

In order to limit the impact of the palette prediction on the memory needed to store the palette predictor, only the palette of the last decoded Palette CU is used as the predictor. This does not modify the worst case as only the maximum number of elements in a palette need to be stored with the size of this last palette.

In addition, to offer palette estimation in parallel at CTB line level at encoder side, the palette is only reset at the beginning of each CTB line.

## Palette Prediction

For the current palette, N flags are extracted from the bitstream, indicating whether the element of the previous palette is copied into the current palette. N is equal to the last palette size. Please note the flags don’t use CABAC context. The number of palette elements copied from the previous palette NumElementFromPrev is set equal to the number of flags equal to 1.

In order to save bits on the coding of the palette size, the palette size is set equal to decoded size + NumElementFromPrev.

When the decoded size is superior to 0, the new palette elements are decoded from the bitstream and added to the palette as usual.

# Remarks on memory complexity

Please note that the use of neighboring palette CUs increased significantly the worst case memory complexity.  Indeed, the worst case for a palette in JCTVC-O0218 is 24 triplets, thus requiring 72 bytes of memory and the minimum CU size to which the palette mode applies is 8x8, thus:

- A CTB line memory requires 8*72 = 576 bytes per CTB – 36KB for 4K content;

- Use of a left predictor requires at least 576 bytes.

In our proposal, the use of the palette of the last decoded Palette CU requires only 72 bytes.

# Experimental results

## Anchor: JCTVC-OO218 in Rext5.1

For these simulation results, we use as reference and as the base of our implementation the JCTVC-O0218 code integrated in the HM-12.1+RExt-5.1.

The proposed palette prediction gives gains for respectively AI/RA/LDB configurations of 3.2%/1.6%/2.1% for SC classes compared to the reference. The gain is larger for optional screen content sequences with 8.9%/7.1%/6.1% for AI/RA/LDB configurations.

Table 1 Results of palette prediction for the Lossy configurations compared to the Rext5.1+JCTVC-O0128.

The proposed palette prediction gives gains also for Lossless coding with a gain for respectively AI/RA/LDB configurations of 3.0%/2.4%/2.6% for SC classes compared to the reference. The gain is larger for optional screen content sequences with 9.4%/8.2%/6.9% for AI/RA/LDB configurations.

Table 2 Results of palette prediction for the Lossless configurations compared to the Rext5.1+JCTVC-O0128.

## Anchor: RCE4 Test2

For these simulation results, we use as reference and as the base of our implementation the RCE4 Test 2 software. The RCE4 Test 2 uses a palette prediction from the left LCU. For this implementation we have only change the left palette CU by the last decoded palette CU. This reduces significantly the memory needed to store the palette predictor. Indeed, at least 8 palette predictors  need to be store in the RCE4 Test 2 instead of only one in our implementation.

The proposed palette prediction gives gains for respectively AI/RA/LDB configurations of 1.2%/ 1.0%/ 0.5% for SC classes compared to the reference. The gain is larger for optional screen content sequences with 4.4%/ 3.4%/ 3.4%.

Table 3 Results of palette prediction for the Lossy configurations compared to the RCE4 Test 2.

The proposed palette prediction gives gains also for Lossless coding with a gain for respectively AI/RA/LDB configurations of 0.6%/0.7%/1.3% for SC classes compared to the reference. The gain is larger for optional screen content sequences with 4.3%/4.6%/5.1% for AI/RA/LDB configurations.

Table 4 Results of palette prediction for the Lossless configurations compared to the RCE4 Test 2.

# Conclusion

This contribution proposes a palette prediction to improve the coding efficiency of the Palette Mode Coding. This prediction method needs a limited storage for the predictor because it uses only the palette of the last decoded Palette CU. This method obtains gains compared RCE4 Test 2 (reaches 4.4% for optional SC sequences) and divides by 8 the memory needed to store the predictor in RCE4 Test 2. According to the interesting gains and the limited impact on the memory, we recommend adopting the Palette mode with the proposed method for the palette prediction.

# Reference

- L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm), “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).