| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0116 |
| --- | --- |

| Title: | Non-RCE4: On Palette prediction modes coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The RCE4 studies several proposals related to palette-like coding methods. In particular, Test2 related to contribution JCTVC-O0218 evaluates a palette coding tool which used 2 prediction modes: The “Run mode” and the “Copy above mode”. These modes are systematically transmitted, but it is not always needed. In this contribution, it is proposed to avoid the signaling of the mode for some cases. It is reported that these modifications provide gains for respectively AI/RA/LDB configurations, ?.?%/?.?%/?.?% for SC classes, and up to ?.?%/?.?%/?.?% on optional classes compared to RCE4 Test 2.

# Introduction

The palette mode coding presented in JCTVC-O0218 [1] uses 2 modes to code the palette indexes: the “Run mode” and the “Copy above mode”. For the Run mode, a palette index is transmitted followed by a run value, the “Palette run”, which indicates the number of following palette indexes with the same value. For the Copy above mode, the “Copy_run” is transmitted to indicate that for the following positions, the palette index is equal to the palette index of the one that is at the same location in the row above.

The mode is transmitted systematically but it is not needed. Indeed, for the first line, it is not possible to copy the above line. Moreover, after a Copy above mode is it not needed to use a Copy above mode because the simple way is to add +1 to the last Copy_run.

In this contribution we propose to use these 2 simplifications to improve the coding efficiency of the Palette mode.

# Proposed modifications

## No Copy above mode for the first line

The first proposed simplification consists in avoiding the coding of the mode for the first line of palette indexes and set this mode to the Run mode.

## No Copy above mode after a Copy above mode

This second proposed simplification consists in avoiding the coding of the mode when the previous mode was a Copy above mode and set it to the Run mode.

# Experimental results

## No Copy above mode for the first line

## No Copy above mode after a Copy above mode

We use as reference the RCE4 test 2.

The proposed palette prediction gives gains for respectively AI/RA/LDB configurations of 0.4%/0.2%/0.2% for SC classes compared to the reference. The gain is larger for optional screen content sequences with 1.9%/1.7%/1.9% for AI/RA/LDB configurations.

Table 1 Results of second modification for the Lossy configurations compared to the RCE4 test 2.

# Conclusion

This contribution proposed 2 simplifications to improve the coding efficiency of the Palette Mode Coding of the RCE4 Test2. According to the interesting gains, we recommend to adopt the Palette mode with these modifications.

# Reference

- L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm), “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, 15th Meeting: Geneva, CH, 23 Oct – 1 Nov. 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).