| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0117 |
| --- | --- |

| Title: | Non-RCE4: Combined coding of run and index for RCE4 Test2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

In the scope of this meeting, RCE4 studies several proposals related to palette-like coding methods. In particular, Test2 related to contribution JCTVC-O0218 [1] evaluates a palette coding tool where runs of values and potentially indexes are encoded. These indexes currently use a fixed length code depending on the number of elements in the palette. It is asserted that, while it is possible to maximize the palette for a given codelength or use variable length codes, this depends on the encoder degree of optimization. It is therefore proposed in this contribution to reuse the unused codewords to encode couples of runs and indexes. It is reported that this method provides luma gains for respectively AI/RA/LDB configurations, on top of RCE4 Test2, of more than 0.4%/0.3%/0.3% for SC classes, and up to 1.6%/1.2%/1.0% on optional classes.

# Introduction

RCE4 Test2 evaluates a palette coding tool that can use up to 32 palette elements. The encoder transmits the number P of palette elements used in a CU. This is used to determine the length (log2(P)) of the fixed length code for coding the index values. Consequently, if P is not a power of 2, unused values ≥P of the code exist.

It is possible to imagine an encoder spending extra effort to produce a palette of P elements where P is a power of 2. However, it is easy to imagine content where, even in lossless mode, this is not achievable.

On the other hand, one can also mitigate the issue by using VLCs. This however makes the parsing substantially more complex compared to a FLC. In addition, this is highly dependent on the encoder ability to properly derive such code, may require additional, possibly counterproductive, in-band information and put additional constrains on the decoder. One last element to consider is that the better the prediction modes, the more equiprobable and random the index values, thusly diminishing the benefit of a VLC.

As a consequence, it seems simpler and interesting to instead repurpose the aforementioned unused values, as this may help compensate encoder algorithm inefficiencies.

# Proposal

What happens is that, in case of a left copy run, an index and a run need to be coded. Given a FLC of length R = log2(P), there are L = 2R-P values above P that are unused.

We have chosen to repurpose these unused values by having them represent instances of run for the palette index lvl=0, such that:

- For decoded index values lvl ≥ P, the run length is computed as lvl-P, while the level is set to 0;

- Otherwise, because runs of length < L for lvl=0 have been encoded following the previous point, the decoded run is incremented by L.

# Results

In order to better distinguish the results, we use the RCE4 Test2 as anchor:

# Conclusion

In light of the reduced complexity increase, the potential to compensate for encoder inefficiencies and the demonstrable gains compared to RCE4 Test2, we recommend further study of the technology in the scope of an RCE on palette coding.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).