| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0118 v4 |
| --- | --- |

| Title: | RExt HLS: Picture referencing across CRA pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ying Wang Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that the restriction “When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture” can hurt compression efficiency, especially for field coding picture structures.

The contribution therefore proposes to use NAL unit type 11 to indicate a new type of picture: the CRA trailing reference (CTR) picture, which may be associated with CRA and BLA pictures. The contribution further proposes that the restriction above is changed to “When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures with nal_unit_type not equal to CTR_NUT that are associated with the same IRAP picture” and that a new restriction is added: “Any CTR picture associated with a CRA or BLA picture shall precede any leading picture associated with the CRA or BLA picture in decoding order”

The contribution reports a -1.50% average bit-rate reduction on the four publicly available test sequences used within the MPEG AHG on study of interlaced coding in HEVC using QPs [22, 27, 32, 37].

Version 2 of this contribution contains source code patches for HM-12.1+RExt-5.0rc1 that are claimed by proponents to enable testing the compression efficiency effect of the restriction.

Version 3 of this document is identical to version 2, the only difference is that the presentation held on January 10 is attached.

Version 4 of this document contains a correction to the source code patches, small correction of the results accordingly, and additional results for lower QPs. The contribution reports an average BDR reduction of 1.46% for QPs [17, 22, 27, 32]

# Problem Statement

In the current HEVC Range Extensions draft specification [1] there are restrictions associated with IRAP picture types. One of those restrictions is the following:

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture.

As a result, all leading pictures must precede all trailing pictures of the associated CRA picture, thus it is impossible for pictures before the CRA in output order to use the pictures that are output after the CRA for reference.  This may reduce prediction efficiency.

Consider Figure 1 which shows a field coding picture structure. The output order is left to right and the decoding order is indicated by numbers inside the pictures. Let the picture immediately following the CRA picture be called the CTR picture. Due to the restriction above, the CTR picture needs to follow all leading pictures in decoding order and be coded as B8. This prohibits the dotted prediction arrows which mean that pictures B3 and B7 have no prediction of the same field parity. If the restriction would not apply however, the CTR picture could have been coded as B1 and the dotted prediction arrow would have been allowed.

Figure 1.  Example of picture structure in field coding

# Proposal

We propose to use a NAL unit type called CRA trailing reference (CTR_NUT) and change the restriction of IRAP pictures.

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures with nal_unit_type not equal to CTR_NUT that are associated with the same IRAP picture.

We also propose to add the following restriction:

Any CTR picture associated with an IRAP picture shall precede any leading picture associated with the IRAP picture in decoding order.

We propose to use NAL unit type 11 for CTR_NUT:

Table 1: Proposed change to NAL unit types

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 11 | CTR_NUT | Coded slice segment of a CTR picture slice_segment_layer_rbsp( ) | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |

A decoder/node can recognize this type and treat it as a special trailing picture. Figure 2 shows an example bitstream structure, the decoder/node can simply discard all NAL units between the CTR picture and the first non-CTR trailing picture of the associated IRAP picture.

| CRA | CTR | Leading pictures | Trailing pictures |
| --- | --- | --- | --- |

Figure 2. Example of bitstream structure with proposal

# Results

A compression efficiency test was done using HM-12.1+RExt-5.0rc1 [2] and the \cfg\misc\encoder_randomaccess_field_coding.cfg configuration. All four publicly available test sequences used within the MPEG AHG on study of interlaced coding in HEVC were used. Table 1 shows the results when comparing the proposal against the current software. QPs 22, 27, 32, and 37 were used. As shown in Table 1, the proposal reduces the average BD-rate by 1.5%.

Table 2: Luma BD-rate performance vs HM-12.1+RExt-5.0rc1 low tier (QP 22, 27, 32, 37)

| Sequence | BDR main tier | Bit-rate range |
| --- | --- | --- |
| MusicHD1_1088_300 | -2.28% | 4-47 Mbps |
| CheersHD2_1088_300 | -0.36% | 5-52 Mbps |
| CheersHD4_1088_300 | -0.56% | 9-103 Mbps |
| MadHD1_1088_300 | -2.80% | 1.5-14 Mbps |
| Average | -1.50% |  |

Table 3: Luma BD-rate performance vs HM-12.1+RExt-5.0rc1 high tier (QP 17, 22, 27, 32)

| Sequence | BDR high tier | Bit-rate tange |
| --- | --- | --- |
| MusicHD1_1088_300 | -2.02% | 8-122 Mbps |
| CheersHD2_1088_300 | -0.33% | 10-114 Mbps |
| CheersHD4_1088_300 | -0.51% | 21-207 Mbps |
| MadHD1_1088_300 | -2.97% | 3-38 Mbps |
| Average | -1.46% |  |

Table 2 and 3 indicates that the average bit-rate savings of 1.5% seems valid for both low and high bit-rates.

Attached to this contribution are two source code patches that can be applied to HM-12.1+RExt-5.0rc1 in order to compile the anchor and test codecs used for generating the results shown in Table 2. The same cfg\misc\encoder_randomaccess_field_coding.cfg configuration file should be used for both anchor and test; besides QP, only parameters in the “File I/O” section needs to be changed.

# References

[1] “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 5”, JCTVC-O1005_v4.doc, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.0rc1

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).