| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0119 |
| --- | --- |

| Title: | Non-RCE4: combination of palette coding improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, T. Poirier, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is about the combination of several modifications related to the Color Palette mode integrated on top of the original method presented in JCTVC-O0218. This contribution includes the technologies presented in contributions JCTVC-P0113, JCTVC-P0114, JCTVC-P0115 and JCTVC-P0116. It also includes 3 non-normative changes. It is reported that the results of the combinations provide gains, for respectively AI/RA/LDB configurations, of at least 13.6%/11.7%/9.1% for SC classes, and up to 45.8%/34.4%/31.2% on optional classes over HM-12.1+RExt-5.1. Additionally, gains over RCE4 Test2, for respectively AI/RA/LDB configurations, of at least 6.1%/6.1%/6.6% for SC classes, and up to 14.0%/13.9%/19.8% on optional classes are observed.

# Introduction

In the scope of this meeting, RCE4 studies several proposals related to palette-like coding methods. In particular, Test2 is related to contribution JCTVC-O0218 [1] with several additional tools such as extensive use of CABAC contexts, palette prediction, modified run coding, and escape coding for pixels for which the palette is unsuited. This contribution described and reports the results of an extension of JCTVC-O0218 with tools not making use of any additional CABAC context, and corresponding to several other contributions presented in this meeting.

# Proposed configuration

Starting from the integration of JCTVC-O0128 in HM12.0+RExt5.1, let us list the additional normative tools for this contribution:

- Mode coding (from JCTVC-P0116):

  - The “copy from above” mode cannot be selected twice successively, thus the prediction mode coding can be improved;

- A transition mode is added, predicting a run of indexes according to the previous location of a pixel having the same level as the left neighbour (from JCTVC-P0115);

- Modification of the run coding (from JCTVC-P0113):

  - Adaptation of the cRiceParam_run according to the mode and the palette index;

  - The limit_run method is enabled.

- Combined level+run coding repurposing the unused levels (from JCTVC-P0117);

- Palette element prediction according to the last decoded palette (from JCTVC-P0114) in the CTB line;

- In addition to the palette contains at most 32 elements (same number of elements than RCE4 Test 2). The palette size is coded with a truncated unary code and the palette size of 1 element is forbidden.

Non-normative changes to the palettization consist in:

- Adapting the error threshold (which determines when a pixel constitutes a new palette element) to the QP, therefore nicely scaling towards losslessness;

- Instead of taking the first palette element resulting in an error below said threshold, take the palette element resulting in the lowest error;

- Accumulating pixel component values for a given element, and determining the average component value to set the palette element.

# Results

The list of the previous section represents numerous changes compared to the initial contribution JCTVC-O0128, which makes the integration in RCE4 Test2 difficult. Furthermore, several of the above points were modified compared to JCTVC-O0128, increasing the integration difficulty. As a consequence, we have chosen to use as a reference HM-12.1+RExt-5.1 for the implementation. Decoding times are unreliable, and should not be considered here.

## Results with HM-12.1+RExt5.1 as reference

## Results with RCE4 Test2 as anchor

# Conclusion

In light of the alternative or additional tools and their corresponding gains, we recommend evaluating (individually and in combination) the additional tools listed in that contribution in a RCE to further investigate and refine the palette mode.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).