| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0120 |
| --- | --- |

| Title: | AHG9: Display hint SEI message for ROI layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message for SHVC with the aim of providing information on the intended display behaviour when enhancement layer is used for region of interest coding purposes.  The proposed SEI message indicates how the player should compose one or more enhancement layers with the base layer (e.g.  BL and EL pictures should be overlaid when displayed).

# Introduction

SHVC design includes several tools to efficiently support ROI coding, such as scaled reference layer offsets. This way, an enhancement layer could be coded corresponding to a certain region within the base layer making it very useful for ROI coding. In this use-case, if a decoder receives both BL and EL, the intended display behaviour would be to overlay the decoded EL picture over the (upsampled) BL picture.

However, scaled reference layer offsets could be used for purposes other than ROI coding. For example, when used in conjunction with tiles, a participants video feed could be enhanced in a multi-party video conferencing so when viewed in full-screen, EL is used to achieve full resolution. In these cases, the intended display behaviour is not always overlaying the BL and EL pictures on top of each other, but instead showing the EL independently or in separate window.

The current SHVC design does not have means to indicate the different intended display behaviour for ROI layers. In this contribution we propose an SEI message to indicate this preferred display operation for ROI layers as follows:

- The intended display behaviour is to combine (overlay) BL and EL decoded pictures.

- The BL and EL decoded pictures are allowed to be overlaid.

- The EL picture should not be combined with the BL picture during display.

# Proposed text

| display_composition_layers( payloadSize ) { | Descriptor |
| --- | --- |
| display_composition_id | ue(v) |
| display_composition_cancel_flag | u(1) |
| if( !display_composition_cancel_flag ) { |  |
| display_composition_persistence_flag | u(1) |
| num_display_composition_hints | ue(v) |
| for( i = 0; i < num_display_composition_hints; i++) { |  |
| display_composition_layer_id[ i ] | u(6) |
| display_composition_idc[ display_composition_layer_id[ i ] ] | u(2) |
| } |  |
| } |  |
| } |  |

This SEI message indicates the intended displaying behaviour, when two or more layers are output layers in an output layer set and either scaled reference layer offsets are specified between these layers or all of these layers have the same values of ScalabilityId[ i ][ j ] values for values of j not equal to 2.

display_composition_id contains an identifying number that may be used to identify the usage of the display composition layers SEI message. The value of display_composition_id shall be in the range of 0 to 232 − 2, inclusive.

Values of display_composition_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of display_composition_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all display composition layers SEI messages containing a value of display_composition_id in the range of 256 to 511, inclusive,  or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

display_composition_cancel_flag equal to 1 indicates that the display composition layers SEI message cancels the persistence of any previous display composition layers SEI message in output order. display_composition_cancel_flag equal to 0 indicates that display composition hint information follows.

display_composition_persistence_flag specifies the persistence of the display composition layers SEI message.

display_composition_persistence_flag equal to 0 specifies that the display composition layers SEI message applies to the current access unit only.

display_composition_persistence_flag equal to 1 specifies that the display composition layers SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in an access unit containing a display composition layers SEI message with the same value of display_composition_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

# num_display_composition_hints specifies the number of display composition hints present in the SEI message.

display_composition_layer_id[ i ] specifies the nuh_layer_id value for which display_composition_idc[ i ] is specified. A picture is associated with display_composition_layer_id[ i ], when the picture has nuh_layer_id equal to display_composition_layer_id[ i ] or when an access unit has no picture with nuh_layer_id equal to display_composition_layer_id[ i ], alt_output_layer_flag is equal to 1 and the picture is selected to be output from the list nonOutputLayerPictures as specified in subclause C.5.2.3. The value of display_composition_ layer_id[ i ] shall be among the layer_id_in_nuh[ j ] values when j is in the range of 1 to MaxLayersMinus1, inclusive, and shall be an output layer of at least one output layer set specified in the active VPS.

display_composition_idc[ display_composition_ layer_id[ i ] ] specifies the intended display behaviour of the associated picture as specified in Table XX. When not present, the value of display_composition_idc[ display_composition_ layer_id[ i ] ] is inferred to be equal to 0.

| display_composition_idc | Intended display behavior |
| --- | --- |
| 0 | The associated picture may or may not be overlaid over the decoded and output pictures with nuh_layer_id less than that of the associated picture |
| 1 | The associated picture should be overlaid over the decoded and output pictures with nuh_layer_id less than that of the associated picture |
| 2 | The associated picture should not be overlaid over the decoded and output pictures with nuh_layer_id less than that of the associated picture |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).