| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0122 |
| --- | --- |

| Title: | Backwards compatible enhancement of chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Döne Bugdayci Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a backwards compatible chroma format enhancement method, where base layer codes the 4:2:0 with HEVC version 1 and enhancement layer codes 4:4:4 or 4:2:2 U,V colour planes separately, using auxiliary pictures. It is argued that the proposed method includes minimal changes to HEVC as it reuses the existing mechanisms already present in HEVC version 1. The version 1 of this contribution includes detailed results of experiments that were agreed on previous meeting in Geneva, showing the performance of proposed methods against simulcast and single layer coding.

# Introduction

It has been noted that there are emerging consumer applications where using 4:4:4 chroma format shows significant performance improvements. These applications mostly involve coding of computer generated content (such as screen sharing in a video conferencing session or streaming the user interface of a mobile device to a TV), where subsampling chroma was reported to introduce visually annoying artifacts, and 4:4:4 coding was reported to be beneficial. However, it is expected that most of the commercially deployed video decoders will be capable of decoding 4:2:0 video only, so it is desired to introduce 4:2:2 and 4:4:4 chroma coding in a backwards compatible way.

This contribution proposes to code a 4:2:0 video using a legacy coder and additional auxiliary layers for the purpose of enhancing the chroma fidelity. The properties of the proposal are:

- Two new auxiliary picture types are proposed to be used with this method.

- The U and V color planes of desired chroma format (4:4:4 or 4:2:2) are coded separately in these new types of auxiliary pictures.

Figure 1-1 Illustration of the proposal where base layer is coded using legacy 4:2:0 encoder and U,V components of 4:4:4 chroma format are coded as separately auxiliary picture layers.

Figure 1-1illustrates the main idea of the proposal, where the 4:2:0 base layer is coded using HEVC version 1 and the auxiliary layers include the U and V components of 4:4:4 (U444,0  and V444,0) that are coded separately. The enhancement layer is reconstructed using the base layer luminance component and the enhancement layer chroma components (Y420,0 U444,0 and V444,0 ).

The proposed method requires the addition of two new AuxId to the existing ones as shown below.

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| … |  |  |
| 3 | AUX_CB | Monochrome picture representing a Cb sample array that enhances the Cb sample array of the associated primary picture |
| 4 | AUX_CR | Monochrome picture representing a Cr sample array that enhances the Cr sample array of the associated primary picture |
| … |  |  |

# Experimental Results

We have used the anti-aliasing filter [0.25 0.5 0.25] to generate the 4:2:0 subsampled versions of the 4:4:4 content. Full set of range extensions test set (with original 10-bit sequences), SC YUV 444 and Animation YUV 444 subsets of screen content test sets are used in the simulations. Results are generated for all tiers.

We have used SHM4.0 with AUXILIARY_PICTURES turned on. Comparisons are made against both simulcast and single layer coding cases where the bitrate of simulcast coding is sum of separate 4:2:0 and 4:4:4 codings whereas for test case it’s the sum of base and auxiliary layers. PSNR of the simulcast coding is taken as PSNR of 4:4:4 coding and for the test case Y PSNR is taken from base layer and U and V PSNR taken from auxiliary layers.

Tables 1-4 provide the summary of the comparison of test method with simulcast and single layer anchors, for range extension and screen content sequences. Detailed results are provided with the excel sheets and tables below.

Table 1 Anchor single layer, RExt test set

Table 2 Anchor simulcast, RExt test set

Table 3 Anchor single layer, SC test set

Table 4 Anchor simulcast, SC test set

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).