| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0122v2 |
| --- | --- |

| Title: | Backwards compatible enhancement of chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Döne Bugdayci Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a backwards compatible chroma format enhancement method, where base layer codes the 4:2:0 with HEVC version 1 and enhancement layer codes 4:4:4 or 4:2:2 U,V colour planes separately, using auxiliary pictures. It is argued that the proposed method includes minimal changes to HEVC as it reuses the existing mechanisms already present in HEVC version 1. The version 1 of this contribution includes detailed results of experiments that were agreed on previous meeting in Geneva, showing the performance of proposed methods against simulcast and single layer coding.

In version 2 of the contribution, the proposed specification text was updated as follows:

- Only one AuxId type (AUX_CHROMA) is taken into use for chroma enhancement pictures (whereas the previous version of contribution proposed to use two AuxId types). When the proposed chroma enhancement feature is used, it is required to have two layers mapped to AUX_CHROMA, where the smaller nuh_layer_id value corresponds to an enhanced Cb picture and greater nuh_layer_id value corresponds to an enhanced Cr picture.

- The chroma auxiliary pictures may or may not be monochrome pictures. Non-monochrome pictures may be used, when it is desired that chroma auxiliary pictures are compatible with the Main, Main 10 or Main Still Picture profiles.

In addition, coding efficiency results for the case where auxiliary chroma pictures are coded with 4:2:0 are added.

# Introduction

It has been noted that there are emerging consumer applications where using 4:4:4 chroma format shows significant performance improvements. These applications mostly involve coding of computer generated content (such as screen sharing in a video conferencing session or streaming the user interface of a mobile device to a TV), where subsampling chroma was reported to introduce visually annoying artifacts, and 4:4:4 coding was reported to be beneficial. However, it is expected that most of the commercially deployed video decoders will be capable of decoding 4:2:0 video only, so it is desired to introduce 4:2:2 and 4:4:4 chroma coding in a backwards compatible way.

This contribution proposes to code a 4:2:0 video using a legacy coder and additional auxiliary layers for the purpose of enhancing the chroma fidelity. The properties of the proposal are:

- Two new auxiliary picture types are proposed to be used with this method.

- The U and V color planes of desired chroma format (4:4:4 or 4:2:2) are coded separately in these new types of auxiliary pictures.

Figure 1-1 Illustration of the proposal where base layer is coded using legacy 4:2:0 encoder and U,V components of 4:4:4 chroma format are coded as separately auxiliary picture layers.

Figure 1-1illustrates the main idea of the proposal, where the 4:2:0 base layer is coded using HEVC version 1 and the auxiliary layers include the U and V components of 4:4:4 (U444,0  and V444,0) that are coded separately. The enhancement layer is reconstructed using the base layer luminance component and the enhancement layer chroma components (Y420,0 U444,0 and V444,0 ).

# Proposed specification text

The main proposed modifications to the specification text are indicated with red font below, while all proposed changes are indicated with change marks in the accompanying specification text.

Table Error: Reference source not found-1 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_CHROMA | Cb and Cr sample array enhancement |
| 4-127 |  | Reserved |
| 128-143 |  | Unspecified |
| 144-255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with AuxId in the range of 128 to 143, inclusive, is specified through means other than the AuxId value.

When AuxId[ lIdA ] is equal to AUX_CHROMA, it is a requirement of bitstream conformance that there shall be exactly one nuh_layer_id value lIdB for which AuxId[ lIdB ] is equal to 3 and ScalabilityId[ LayerIdxInVps[ lIdB ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ lIdA ] ][ j ] for all values of j in the range of 0 to 15, inclusive. Pictures with nuh_layer_id equal to Min( lIdA, lIdB ) enhance the Cb sample array of the associated primary picture. Pictures with nuh_layer_id equal to Max(( lIdA, lIdB ) enhance the Cr sample array of the associated primary picture.

It is a requirement of bitstream conformance that when an access unit includes an auxiliary picture with nuh_layer_id value lIdA such that AuxId[ lIdA ] is equal to AUX_CHROMA, the same access unit shall include an auxiliary picture with nuh_layer_id value lIdB such that AuxId[ lIdB ] is equal to AUX_CHROMA and ScalabilityId[ LayerIdxInVps[ lIdB ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ lIdA ] ][ j ] for all values of j in the range of 0 to 15, inclusive.

NOTE 2 – In other words, it is disallowed to enhance only one of the Cb sample array and the Cr sample array of the primary picture by a respective auxiliary picture. Instead, either no chroma sample array or both chroma sample arrays of the primary picture can be enhanced.

The luma sample array of an auxiliary picture or a monochrome auxiliary picture with a nuh_layer_id value lId such that AuxId[ lId ] is equal to AUX_CHROMA represents a Cb or Cr sample array enhancing the Cb or Cr sample array of the associated primary picture, respectively. When an auxiliary picture with a nuh_layer_id value lId such that AuxId[ lId ] is equal to AUX_CHROMA is not a monochrome picture, the semantics of Cb and Cr sample arrays of the auxiliary picture are unspecified.

NOTE 3 – Non-monochrome auxiliary pictures with a nuh_layer_id value lId such that AuxId[ lId ] is equal to AUX_CHROMA may be used to obtain combatibility with some profiles specified in Annex A.

# Experimental Results

We have used the anti-aliasing filter [0.25 0.5 0.25] to generate the 4:2:0 subsampled versions of the 4:4:4 content. Full set of range extensions test set (with original 10-bit sequences), SC YUV 444 and Animation YUV 444 subsets of screen content test sets are used in the simulations. Results are generated for all tiers.

We have used SHM4.0 with AUXILIARY_PICTURES turned on. Comparisons are made against both simulcast and single layer coding cases where the bitrate of simulcast coding is sum of separate 4:2:0 and 4:4:4 codings whereas for test case it’s the sum of base and auxiliary layers. PSNR of the simulcast coding is taken as PSNR of 4:4:4 coding and for the test case Y PSNR is taken from base layer and U and V PSNR taken from auxiliary layers.

## Auxiliary Pictures coded as monochrome

Tables 1-4 provide the summary of the comparison of test method with simulcast and single layer anchors, for range extension and screen content sequences. Detailed results are provided with the excel sheets and tables below.

Table 2 Anchor single layer, RExt test set

Table 3 Anchor simulcast, RExt test set

Table 4 Anchor single layer, SC test set

Table 5 Anchor simulcast, SC test set

## Auxiliary Pictures coded as 4:2:0

It was earlier discussed that auxiliary pictures could be coded as 4:2:0 to enable the use-case that 4:2:0 decoder without monochrome decoding capability can be reused. For this, we generated the high resolution 4:2:0 video where the luma corresponds to Cb or Cr and chroma components are set to 128 (for 8 bit video).

As noted below the coding efficiency is almost identical to results presented in Section 3.1.

Table 6 Anchor single layer, RExt test set

Table 7 Anchor single layer, SCC test set

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).