| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0123 |
| --- | --- |

| Title: | RExt HLS (AHG5 and 9): SEI message for alpha channel information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

Alpha channel signals are used in professional (studio) video coding applications and their usage may become popular also for frame composition at the receiver side. Briefly, the alpha channel complements the information contained in the main bitstream by providing, for each image pixel, its degree of transparency. Support for alpha channel in range extensions of HEVC is provided using auxiliary pictures which are transmitted along with primary coded pictures. HEVC will transmit auxiliary pictures as a scalable layer with a particular identifier to specify the type of auxiliary picture (e.g. alpha channel, depth map, chroma enhancement layer, etc.). According to this transmission modality, picture dependent data can be transmitted using SEI messages. This contribution proposes a SEI message to communicate some information related to alpha channels and their usage in frame composition.

# Introduction

Alpha channel signals are used in professional and studio video coding applications for object insertion, manipulation and editing in post-production. The alpha channel conveys information which complements the primary coded pictures by indicating for each pixel its degree of transparency or otherwise its degree of opacity. Alpha channels are video sequences consisting of monochrome frames whose pixels assume values in the range r = [αtransparent, αopaque] where αtransparent is typically equal to zero and αopaque is greater than αtransparent. The range extensions of the HEVC will carry alpha channel signals as auxiliary pictures which can be seen as additional metadata associated to the primary picture. At the 15th meeting in Geneva, the carriage mechanism for auxiliary picture proposed in JCTVC-O0041 was adopted. Briefly, in this mechanism auxiliary pictures are transmitted as scalable layers where the layer identifier specifies the type of auxiliary picture (e.g. depth map, alpha channel, chroma enhancement plane, etc.). According to JCTVC-O0041, all additional information which depends upon the auxiliary picture type can be then transmitted using a dedicated SEI message. Therefore, this contribution proposes the syntax and semantics for a SEI message used to transmit some data useful for frame composition and/or alpha channel post-processing. The next section details the application requirements and describes the syntax and semantics of the SEI message.

# Proposed alpha channel SEI message

When using the alpha channel for frame composition or other editing operations, it is desirable that the following information is communicated along with the compressed alpha channel signal:

- Alpha channel’s sample values: e.g. value for transparent samples, bit depth, etc.

- Alpha channel’s sample values for post-processing: e.g. whether the decoded alpha channel values should be clipped, filtered, etc.

- Usage instructions for alpha channel in frame composition: e.g. whether the received primary picture is a pre-multiplied alpha image.

The support to signal the aforementioned information is now detailed in the following subsections. All the syntax and semantics changes refer to JCTVC-O1005_v4 and are marked in yellow.

## Proposed alpha channel SEI message syntax

3	Definitions

...

3.x	alpha blending: A process not specified by this Recommendation | International Standard, in which an auxiliary coded picture is used in combination with a primary coded picture and with other data not specified by this Recommendation | International Standard in the display process. In an alpha blending process, the samples of an auxiliary coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding samples of the primary coded picture.

3.y	interpretation sample value A possibly-altered value corresponding to a decoded sample value of an auxiliary coded picture that may be generated for use in the display process. Interpretation sample values are not used in the decoding process and have no normative effect on the decoding process.

...

D.2.1	General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = 139 ) |  |
| alpha_channel_info( payloadSize ) |  |
| ... |  |
| } |  |

## Proposed alpha channel SEI message syntax

D.2.29	Alpha channel information SEI message syntax

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type | u(1) |
| } |  |
| } |  |

D.3.29	Alpha channel information SEI message semantics

This SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes.

alpha_channel_cancel_flag equal to 1 indicates that the alpha channel information SEI message cancels the persistence of any previous alpha channel information SEI message in output order. alpha_channel_cancel_flag equal to 0 indicates that alpha channel information follows.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for purposes to be specified in future versions of this Recommendation | International Standard. When not present, alpha_channel_use_idc shall be inferred equal to 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 shall be in the range 0 to 7 inclusive. alpha_channel_bit_depth_minus8 shall be equal to bit_depth_luma_minu8 of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample, and any auxiliary picture sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used without alteration as the interpretation sample value for the decoded auxiliary picture sample value. When not present, alpha_channel_incr_flag shall be inferred equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type syntax element. When not present, alpha_channel_clip_flag shall be inferred equal to 0.

alpha_channel_clip_type equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture sample that is greater than ( alpha_opaque_value – alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture sample, and any auxiliary picture sample that is less or equal than ( alpha_opaque_value – alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture sample. alpha_channel_clip_type equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture sample, and any auxiliary picture sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture sample.

NOTE X – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type shall be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain interpretation sample value for the auxiliary picture sample.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).