| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0124 |
| --- | --- |

| Title: | Non-SCE1: Color gamut scalability using modified weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Aminlou K. Ugur M. M. Hannuksela | Tel: Email: | +90 536 567 29997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

SCE1 tests two tools utilizing look-up tables for increasing the coding efficiency of SHVC for color gamut scalability. This contribution proposes an alternative method that is based on modified weighted prediction process for improving the coding efficiency of color gamut scalability. The proposal makes three changes to HEVC weighted prediction so that it is more suitable for inter-layer color gamut mapping: Firstly, the YUV space is divided into an NYxNCbxNCr region and for each region different parameters are signaled. Secondly, WP utilizes a matrix based mapping to derive the prediction pixel values (the luminance value of the prediction pixel is calculated using luminance and chrominance values of the reference pixel). As a third modification, second order polynomial equations are used for matrix based mapping, instead of linear equations. Experimental results show that the proposed method improves the coding efficiency by 8.6% and 6.4% on average for AI and RA cases respectively. In addition, results for several variations and simplifications are also included in the contribution.

As an update to the contribution, full results for polynomial based matrix mapping are provided. In addition more details on encoder algorithm and the syntax are provided.

# Introduction

SHVC WD4 includes a mechanism for color gamut scalability, to support coding EL with BT.2020 at UHD resolution and base layer with BT.709 at HD resolution. This is done by modifying the upsampling process, where the bit-shifts within upsampling process depend on the bit-depths of both EL and BL picture and using weighted prediction for inter-layer processing. In order to further improve the coding efficiency of the above use-case, SCE1 was formed and look-up table based approaches are tested within SCE1.

# CoGS using improved WP

In order to improve coding efficiency for CoGS we propose three modifications for weighted prediction process.

- Region based WP: Based on the observations in JCTVC-L0334 and JCTVC-M0197, dividing the YUV colorspace into regions and using different color space mapping function for each region improves the coding efficiency. This technique was also used in SCE1 responses and also included in our proposal, where the YUV region is divided into an NxNxN region and separate parameters are signaled separately for each region.

- Matrix based mapping: The WP process in HEVC does not utilize any cross-component correlation. In other words, the luminance of prediction pixel utilizes a linear gain-offset model that takes luminance of reference pixel (and not chrominance values) as input. As analyzed in JCTVC-L0334, utilizing a matrix based mapping more accurately reflects the BT.2020 to BT.709 mapping:
YCbCr2020=Gain(Y,Cb,Cr).YCbCr709+Offset(Y,Cb,Cr)						(1)

Gain(Y, Cb, Cr) is a 3x3 matrix and Offset(Y, Cb, Cr) is a 1x3 matrix. Encoder uses simple linear regression to derive the parameters of the matrices (using more complicated algorithms could further improve coding efficiency)

- Polynomial WP: In order to better capture the non-linear relationship between two color spaces, as an alternative to Equation 1, we use a second order polynomial equation as follows:

YCbCr2020=Gain1(Y,Cb,Cr).Y−Y0Cb−Cb0Cr−Cr07092+Gain2(Y,Cb,Cr).YCbCr709+Offset(Y,Cb,Cr)	(2)

Similar to Equation 1, Gain1(Y, Cb, Cr) and Gain2(Y, Cb, Cr) are 3x3 matrices and Offset(Y, Cb, Cr) is a 1x3 matrix. (Y0, Cb0, Cr0) is the central point of the corresponding region. For example, when there are 8x8x8 regions and BL data are 10bit, Y0 =((Y709>>7)<<7)+64. This is to reduce the dynamic range of polynomial part of Equation 2. The benefit of using polynomial equations is that one could reduce the number of regions as non-linear characteristics can already be captured with the polynomial equation.

The modified WP is only used for inter-layer processing (i.e. the temporal HEVC WP is not modified) and, similar to SHVC WD, operates on the output pixels after upsampling.

## Details of mapping process

The details of mapping process can be summarized as below:

- Divide color components (Y, Cb and Cr) ranges of the base layer picture into NY, NCb and NCr regions, respectively. Each range of Y, Cb, and Cr has the length of LY, LCbr, LCr, respectively. As a result, the YCbCr domain of the base layer is divided to NY×NCb×NCr regions. Each region is represented by index value of [iY, iCb, iCr].

- Decode/Find the gain-offset matrices of Gain and Offset for all regions at the encoder/decoder side.

- For all pixel of (Y709, Cr709, Cb709) in the base layer

  - Find the correspond region of [iY, iCb, iCr] using (3)

  - Map the pixel by the corresponding matrices, for example using (4)

iY=Y709LY iCb=Cb709LCb iCr=Cr709LCr								(3) YCbCr2020=Gain(iY,iCb,iCr).YCbCr709+Offset(iY,iCb,iCr)					(4)

## Gain-offset matrix estimation

At the encoder side, for each I-frame, the weight-offset matrices are found by least square error method for each region of [iY, iCb, iCr], and are signaled in the slice header. In the simulations, the matrices of I-frame are reused for all P/B frames in the Intra period. Let’s define set of S[iY, iCb, iCr] as (5) which includes all the pixel of the base layer frame which are in region of [iY, iCb, iCr].

| S[iY,iCb,iCr]=YUV709¦YUV709 in region of [iY,iCb,iCr] | (5) |
| --- | --- |

Then the coefficients for each color component in the regions of [iY, iCb, iCr] are found as below. The square error can be calculated for a color component, for example for Y component using (6),

| EY=Yb∈SYE−Y20202 | (6) |
| --- | --- |

where YE is the original values of the enhancement layer picture, and Y2020 is the calculated value of the target picture using the described mapping method. According to the described method, the value of Y2020 for each pixel is calculated using (7)

| Y2020=gYY×Y709+gYCb×Cb709+gYCr×Cr709+oY | (7) |
| --- | --- |

where gYX (X=Y, Cb, Cr) and oY are the coefficients of the first row of Gain[iY, iCb, iCr] and Offset[iY, iCb, iCr] matrices.

To have the optimum values of gYX and oY variables, the first derivation of EY with respect to each variable should be equal to zero. This forms a system of four equations and for variables, where the values of gYX and oY are found with a closed form solution. The value of other gij and oi variables is found for the other color components (Cb and Cr) using the similar manner.

## Gain-offset matrix coding

The coefficients of Gain and Offset matrices are real number that should be converted to signed integer values to be coded into the bitstream. Coefficients of Offset matrix are truncated to the signed integer values without any quantization. The coefficients on the main diagonal of matrix Gain (i.e. gYY, gCbCb, gCrCr) are subtracted by 1.0. Then all coefficients of matrix Gain are quantized to M bit (e.g 10 bit).

The Gain[iY, iCb, iCr] and Offset[iY, iCb, iCr] are coded into the enhancement layer bitstream as below. For each frame a “FramePresentFlag” is coded which indicates whether any parameters is code for this frame or not. In the value of this flag is zero, the RegionPresentFlag” of all regions are set to zero. Otherwise, parameters of some regions may be coded as below. For a given [iY, iCb, iCr], firstly the “RegionPresentFlag” is coded, indicating whether Gain and Offset matrices should be coded or not. If the flag is “0”, it means that Gain and Offset are not coded. In this case, Gain and Offset are set to fixed predefined matrices (e.g. Identity and zero, respectively) for I-frames, or they are copied from previously coded I-frames for P/B-frames. If the flag is coded as “1”, it means that the matrices are coded by signed variable length coding in the slice header.

# Experimental Results

In this section we provide results for different configurations. For all the configurations, the WP parameters are derived and signaled for the first picture of the Intra period (i.e. Intra-frame) and used for all the pictures within that Intra-period. More complex encoder algorithms could potentially improve the coding efficiency further.

Configuration 1: N=8, linear matrix mapping (YUV colorspace is divided into 8x8x8 regions)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.6% | -12.3% | -16.8% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs Ref) | -8.6% | -12.3% | -16.8% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs single layer) | 8.1% | 7.4% | -1.4% | 10.6% | 9.1% | -0.2% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -16.7% | -19.9% | -24.4% | -16.7% | -20.0% | -24.4% |
| Overall (Test EL+BL vs single EL+BL) | -29.1% | -29.1% | -36.2% | -27.4% | -28.0% | -35.6% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.4% | -10.0% | -14.3% | -6.3% | -10.0% | -14.1% |
| Overall (Test vs Ref) | -6.4% | -10.0% | -14.3% | -6.3% | -10.0% | -14.1% |
| Overall (Test vs single layer) | 19.2% | 20.5% | 8.5% | 20.9% | 21.9% | 9.8% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -11.8% | -15.0% | -19.3% | -11.7% | -15.0% | -19.0% |
| Overall (Test EL+BL vs single EL+BL) | -20.4% | -17.5% | -27.1% | -19.4% | -16.9% | -26.5% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 2: N=1, linear matrix mapping (Region based WP is disabled and only difference to HEVC WP is linear matrix based equations)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -3.0% | -3.0% | -4.4% | -3.2% | -3.2% | -4.4% |
| Overall (Test vs Ref) | -3.0% | -3.0% | -4.4% | -3.2% | -3.2% | -4.4% |
| Overall (Test vs single layer) | 15.0% | 18.9% | 13.0% | 17.2% | 20.6% | 14.3% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -6.1% | -6.0% | -7.5% | -6.5% | -6.3% | -7.5% |
| Overall (Test EL+BL vs single EL+BL) | -25.1% | -22.5% | -26.7% | -23.7% | -21.5% | -26.0% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -1.9% | -1.6% | -3.7% | -2.0% | -1.7% | -3.5% |
| Overall (Test vs Ref) | -1.9% | -1.6% | -3.7% | -2.0% | -1.7% | -3.5% |
| Overall (Test vs single layer) | 25.0% | 31.5% | 22.0% | 26.5% | 32.9% | 23.2% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -3.6% | -3.1% | -5.3% | -3.8% | -3.3% | -5.2% |
| Overall (Test EL+BL vs single EL+BL) | -17.1% | -11.5% | -18.1% | -16.3% | -10.9% | -17.6% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 3: N=8, no matrix based WP (Matrix based WP is disabled and only difference to HEVC WP is signaling different WP parameters for different regions)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -5.2% | -4.3% | -9.9% | -5.2% | -4.4% | -9.9% |
| Overall (Test vs Ref) | -5.2% | -4.3% | -9.9% | -5.2% | -4.4% | -9.9% |
| Overall (Test vs single layer) | 12.3% | 17.5% | 6.6% | 14.9% | 19.3% | 7.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -10.8% | -9.5% | -15.1% | -10.7% | -9.6% | -15.0% |
| Overall (Test EL+BL vs single EL+BL) | -26.4% | -22.8% | -31.0% | -24.6% | -21.7% | -30.2% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.3% | -4.4% | -9.6% | -4.2% | -4.6% | -9.3% |
| Overall (Test vs Ref) | -4.3% | -4.4% | -9.6% | -4.2% | -4.6% | -9.3% |
| Overall (Test vs single layer) | 22.0% | 28.1% | 14.5% | 23.7% | 29.3% | 15.8% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -8.2% | -7.8% | -13.1% | -8.1% | -8.1% | -12.8% |
| Overall (Test EL+BL vs single EL+BL) | -18.7% | -13.1% | -23.3% | -17.7% | -12.7% | -22.7% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 4: N=1, polynomial matrix mapping enabled (Region based WP is disabled and only difference to HEVC WP is polynomial matrix based equations)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.8% | -7.8% | -8.1% | -6.7% | -7.8% | -8.0% |
| Overall (Test vs Ref) | -6.8% | -7.8% | -8.1% | -6.7% | -7.8% | -8.0% |
| Overall (Test vs single layer) | 10.4% | 12.9% | 8.7% | 12.9% | 14.8% | 10.0% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -13.6% | -14.3% | -14.6% | -13.5% | -14.3% | -14.5% |
| Overall (Test EL+BL vs single EL+BL) | -28.1% | -26.1% | -30.5% | -26.4% | -25.0% | -29.8% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.6% | -4.7% | -6.0% | -4.5% | -4.6% | -5.6% |
| Overall (Test vs Ref) | -4.6% | -4.7% | -6.0% | -4.5% | -4.6% | -5.6% |
| Overall (Test vs single layer) | 21.5% | 27.5% | 19.1% | 23.3% | 29.0% | 20.5% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -8.8% | -8.5% | -9.8% | -8.7% | -8.5% | -9.4% |
| Overall (Test EL+BL vs single EL+BL) | -19.2% | -13.8% | -21.2% | -18.2% | -13.0% | -20.6% |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

The main advantage of the proposed method over 3D-LUTs is that it achieves additional coding efficiency gains with minor modifications to the existing Weighted Prediction process. In addition, the proposed method has the following complexity advantages:

- The proposed method uses a straightforward matrix multiplication and adding an offset value to map color gamut of base layer to color gamut of enhancement layer. However, 3D-LUT method has several sequential steps which are not computational friendly. It applies some pre-processing (known as CGS_LUMA_ENH) on YUV values. Then tetra interpolation (interpol_tetra_Lum and interpol_tetra_Chroma) is applied which is a complex process. This process potentially uses the values of eight nodes of the region corresponding to the pixel. Particularly, based on the YUV values, the appropriate interpolation path is selected by selecting the two appropriate nodes and the output values is calculated using three multiplications and adding an offset value. Finding appropriate nodes needs several (about 5) if conditions.

- Finding the optimum parameters is straightforward for the proposed method. It uses the least square error method based on the pixel values reside in each region. But in 3D-LUT method, the optimum YUV value of each node depends on the statistics of the pixels in all eight regions.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).