| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0125r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0125r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On VPS extension offset and VPS VUI offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution discusses some situations where insertion of emulation prevention bytes can cause problems with setting of the values of syntax elements vps_extension_offset and vps_vui_offset. To prevent such situation, it is proposed to take the last bit away from the two syntax elements and mandate the value of the bit to be equal to 1.

In the first revision of this contribution, it is identified that the example related to syntax element vps_extension_offset is not correct and thus the proposed syntax changes to solve the problem is not needed. Changes to the semantics of vps_extension_offset and changes to the syntax and semantics of vps_vui_offset are proposed.

# Introduction

The video parameter set (VPS) has two syntax elements that are present to enable bitstream parsers to quickly access the beginning of the VPS extension and the VPS VUI, without having to parse the entire rest of the syntax elements of the VPS that occur before these sections of the VPS, particularly without the need of parsing any ue(v) coded syntax element. The semantics of these two syntax elements are given below:

vps_extension_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the next set of fixed-length coded information starting from avc_base_layer_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

NOTE – VPS information for non-base layer or view starts from a byte-aligned position of the VPS NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by vps_extension_offset would then help to locate and access those essential information in the VPS NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the VPS that is essential for session negotiation and/or capability exchange.

vps_vui_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the set of fixed-length coded information starting from bit_rate_present_vps_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

The values of both the syntax elements include all emulation prevention bytes, when present, in the relevant sections in the VPS NAL unit.

## Emulation prevention bytes

Emulation prevention bytes are added in a bitstream, as specified in Annex B, in order that applications that use byte stream format (i.e. those applications that deliver a bitstream as a stream of bytes or bits) can unambiguously identify start codes. When a pattern occurs in the video bitstream that would emulate one of the fixed patterns, a byte (0x03) is added to ensure that the parsing can be unambiguous. This byte is called the emulation prevention byte.

The bit patterns in the video bitstream that are replaced, and the patterns that they are replaced with are given in Table 1.

Table 1: Patterns in the video stream for which emulation prevention bytes are added.

| Pattern that is replaced | Pattern replaced with |
| --- | --- |
| 0x000000 | 0x00000300 |
| 0x000001 | 0x00000301 |
| 0x000002 | 0x00000302 |
| 0x000003 | 0x00000303 |

# Problem Statement

The semantics of the two syntax elements vps_extension_offset and vps_vui_offset are such that the values of the syntax elements include the emulation prevention bytes inserted. However, the presence of emulation prevention bytes (EPBs) in the VPS is also based on the value of these syntax elements. Thus, there may be a cyclic dependence in the calculation of the value of the syntax element and presence of EPBs, which can result in either the wrong value filled for these syntax elements, or the byte stream format unable to parse the bytes correctly.

Figure 1 illustrates the value of the bits at the beginning of the VPS extension, where X denotes that the value of the bit could either be 0 or 1. The value of vps_vui_offset (equal to 1024) shown in Figure 1 is before the addition of EPBs. It is assumed that EPBs are not added in any location in the VPS after byte 4 even though a similar argument can be made even if more EPBs are present after byte 4. Since the bytes 1, 2, and 3 form the pattern 0x000000, the EPB (0x03) will be added between bytes 2 and 3 as shown in Figure 2. Since the EPB is now added, the value of vps_vui_offset will need to be incremented by 1, and the value now becomes 1025 (as shown in Figure 3). With this increment, however, the byte pattern formed by bytes 1, 2, and 3 is no longer 0x000000 and so the EPB is not needed. A parser would be unable to recognize the EPB in the byte stream, and thus be unable to remove this. This could result in a non-conformant bitstream.

Figure 1

Figure 2

Figure 3

Figure 4

# Proposal

It is proposed that the length of the syntax element vps_vui_offset be reduced by one bit to 15 bits, and the remaining 16th bit shall be set equal to 1. The semantics of vps_extension_offset are modified such that the value of vps_extension_offset specifies the byte offset of the first byte of the VPS extension from the beginning of the first byte succeeding vps_extension_offset. The semantics of vps_vui_offset are modified such that the value of vps_vui_offset specifies the byte offset of the first byte of the VPS VUI from the beginning of the VPS extension. These changes prevent the cyclic dependence of the value of the syntax elements on the presence of EPBs.

## Video parameter set semantics

vps_extension_offset specifies the byte offset, starting from the beginning of the VPS NAL unit byte following this syntax element, of the next set of fixed-length coded information starting from avc_base_layer_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear after this syntax element until avc_base_layer_flag, exclusive, in the VPS NAL unit are counted for purposes of byte offset identification.

NOTE – VPS information for non-base layer or view starts from a byte-aligned position of the VPS NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by vps_extension_offset would then help to locate and access those essential information in the VPS NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the VPS that is essential for session negotiation and/or capability exchange.

  - Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| vps_vui_offset | u(165) |
| vps_extn_bit_equal_to_one | u(1) |
| } |  |
| splitting_flag | u(1) |
| … |  |

vps_vui_offset specifies the byte offset, starting from the beginning of the VPS NAL unit avc_base_layer_flag, of the set of fixed-length coded information starting from bit_rate_present_vps_flagcross_layer_pic_type_aligned_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear from avc_base_layer_flag to cross_layer_pic_type_aligned_flag, exclusive, in the VPS NAL unit are counted for purposes of byte offset identification.

vps_extn_bit_equal_to_one shall be equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).