| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0126r1 |
| --- | --- |

| Title: | SEI message for Colour Mapping Information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Philippe Bordes, Edouard François Technicolor 975 avenue des champs blancs CS 17616-35576 Cesson-Sévigné Cedex, France | Tel: Email: | +33 (0)2 99 27 38 07 pierre.andrivon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution relates that colour mapping metadata guarantees smooth colour space transition with regards to upcoming HDTV and UHDTV services deployment. Additionally, it is claimed that colour mapping metadata helps preserving artistic intent of the content produced by studios while maintaining differentiation between TV sets manufacturers.

Some concerns were previously expressed on the complexity of a 3D LUT model. A simplified colour mapping model based on 1D LUT and 3x3 matrix is proposed in order to alleviate processing complexity and to leverage CE devices computational resources.

# Introduction

Consultations on colour metadata were recently launched for standards committees dealing with delivery to end consumers and feedback is to be expected for this meeting. Indeed, in the case that the colour encoding system defined in Rec.2020 would be considered for those standards for the upcoming HDTV and UHDTV services, critical colour issues should be cleanly managed.

First of all, it is a fact that current HDTV broadcast services use Rec.709 colour encoding system for which primaries were established from CRT displays technology. For home delivery, content production is mastered on Rec.709 reference displays then theoretically the content can be colour managed from the production via the distribution networks to the rendering. Additionally, CE and TV manufacturers propose in their rendering devices post-processing pre-rendering algorithms that leverage the maximum capabilities of the rendering devices technology; besides it enables differentiation of the rendered output pictures for the customers.

Rec.2020 today

Rec.2020 proposes a colour encoding system with primaries that are much closer to the spectral locus (visible colour boundaries). Hence, Rec.2020 gamut increases by 30% (CIE 1931) and 24% (CIE 1976) over Rec.709 gamut [4] (Figure 1).

Figure 1: Rec.709 (left) and Rec.2020 (right) gamut in CIE 1931– courtesy from BSkyB

Nowadays, only laser display technology (e.g. Mitsubishi LASERVue) allows reaching such saturated primaries and no reference display is yet compliant with Rec.2020. Migration to full Rec.2020 will take years. This is currently a major concern for producing wide colour gamut (WCG) content for broadcast. Intermediate colorimetry reference displays based on those used in Digital Cinema content mastering (DCI-P3) may support the advent of WCG content production for broadcast however for future-proof rationales it makes sense that the produced content would be expressed with Rec.2020 colour encoding system instead of redefining new colour encoding system every time a technology allows to spread primaries toward the spectral locus (leading to interoperability, maintenance and production cost issues).

Transition to Rec.2020 while keeping content creators intent

We can anticipate that a content will be colour graded (artistic process) and mastered on a particular WCG “Rec.709+” reference display before being encoded in Rec.2020 colour encoding system and finally it will be rendered on consumer WCG displays (Cf. Figure 2). This content may also be classically colour graded on a Rec.709 reference display to address widespread legacy devices (e.g. HDTV or UHD-1 phase 1 receivers…).

Considering a smooth transition of display technologies from Rec.709 to Rec.2020, colour mapping metadata may be an alternative to simulcast. Those metadata can be transmitted aside Rec.709 legacy content to enable a wide colour enhanced version of the content (mastered on a display between Rec.709 and Rec.2020). Besides, as those colour metadata are created during the content production process, artistic intent is preserved for the enhanced version, ensuring optionally “best possible” mapping for interested CE manufacturers that can add their own enhancement algorithms on top.

Figure 2: Preserving artistic intent
 Left side is studio colours (top-left: Rec.709 grading, bottom-left: WCG grading) -
 Right side is consumer WCG display (top-right: Rec.709WCG gamut mapping algorithm, bottom-right: Rec.709 with WCG colour mapping metadata)

Functionally, at the WCG receiver side, colour metadata are applied to the decoded video to map toward the production colour space while preserving artistic creation. Then, CE devices apply their own post-processing (e.g. gamut mapping) algorithm to remap the content on the display primaries (that may differ from the mastering display used at the production side) and enhance the content. Colour mapping metadata are optional and they could make particularly sense for the rendering of movies (either broadcast or stored on high quality Blu-Ray) where colour grading is an essential part of the artistic creation. The SEI message information concept enables to transmit side information at the nearest level of the video stream. Then, SEI colour mapping metadata is not prone to be pruned at every step of the distribution workflow.

UHDTV forward compatibility

If we consider forward compatibility for UHD-1 i.e. that UHD-1 phase 1 Rec.709 receivers should be able to render UHD-1 phase 2 Rec.2020 services, colour mapping metadata will enable transmitting legacy Rec.709 colour information aside with the main WCG content, guaranteeing a forward compatibility that will allow to transmit decent Rec.709 encoded colours of the content in comparison with the originally mastered Rec.709 legacy content.  Indeed, as mentioned upward, due to the important difference in gamut between the colour space use today in production for broadcast (Rec.709) and the targeted one for upcoming video services (Rec.2020) colour space and/or gamut mapping conversions (and implementations…) may be a critical source of approximations and deteriorations (saturation, clipping…) of the originally mastered content (Cf. Figure 3). Furthermore, it is envisioned by the authors that decent Rec.2020 to Rec.709 gamut mapping will be more difficult for legacy Rec.709 TV sets when production will use a wider Rec.2020 gamut subset over time, as without colour metadata the device manufacturer does not know which part of Rec.2020 gamut the content corresponds to. Thus, unlike Rec.709 end-to-end chain, the perceived quality of WCG rendered content may vary hugely depending on the distribution chain layout, systems and the receivers. Wrong colour displaying on legacy Rec.709 receivers for the launch of Rec.2020 services would send a wrong message to consumers.

Figure 3: Forward compatibility use case
Left side is studio colours (top-left: WCG grading, bottom-left: Rec.709 grading) -
 Right side is consumer Rec.709 display (top-right: WCGRec.709 gamut mapping algorithm, bottom-right: WCG with Rec.709 colour mapping metadata)

Link with XYZ MPEG AhG

Finally, in the MPEG Ad hoc Group on HDR and XYZ video coding, draft requirements for Full Gamut Content Distribution [5], mentions the interest of using colour metadata (such as 3x3 matrix with 1D or 3D LUT) to allow the display to perform an optimal signal processing to reproduce the colour at best quality. Besides, colour metadata may be the bridge between a XYZ derived colour space and the future broadcast colour space candidate Rec.2020.

# Proposal

During last JCT-VC meeting [2], some concerns were expressed on the complexity to implement in current CE devices the interpolation of the 3D LUT representation proposed as a colour mapping model.

Displays and CE devices are used to perform colour space conversions according to input formats. For instance, content is coded with Y’CbCr colour encoding system before encoding and distribution then at the receiver side, the renderer or CE devices classically convert the video signal in R’G’B’ that is a proper colour space to be displayed. After chroma interpolation (4:2:0 to 4:4:4) this operation is classically performed with a 3x3 matrix. Besides, displays generally control brightness, contrast and saturation by using one 1D LUT (optionally per component), a configurable 3x3 matrix and sometimes another 1D LUT (optionally per component). Thus, it is proposed as an alternative to the 3D LUT model, a simplified model based on a 1D LUT (optionally per component) and a 3x3 matrix in order to leverage displays computational resources.

## Proposed syntax

| colour_mapping_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_map_id | ue(v) |
| colour_map_model_id | ue(v) |
| colour_map_cancel_flag | u(1) |
| if( !colour_map_cancel_flag ) { |  |
| colour_map_video_signal_type_present_flag | u(1) |
| if(colour_map_video_signal_type_present_flag ) { |  |
| colour_map_video_format | u(3) |
| colour_map_video_full_range_flag | u(1) |
| colour_map_description_present_flag | u(1) |
| if (colour_map_description_present_flag) { |  |
| colour_map_primaries | u(8) |
| colour_map_transfer_characteristics | u(8) |
| colour_map_matrix_coeffs | u(8) |
| } |  |
| } |  |
| colour_map_persistence_flag | u(1) |
| colour_map_model_id | ue(v) |
| if( colour _map_model_id = = 0 ) { |  |
| colour_map_3d_lut_model_data ( ) |  |
| } else if( colour _map_model_id = = 1 ) { |  |
| colour_map_1d_lut_plus_matrix_model_data ( ) |  |
| } |  |
| } |  |
| } |  |

This SEI message provides information to enable remapping of the colour samples of the output decoded pictures for customization to particular applications or display environments. The remapped samples are expressed either in the luma and chroma or RGB colour space domain, and should be applied to the reconstructed luma and chroma, or the reconstructed RGB samples.

colour_map_id, colour_map_persistence_flag, colour_map_cancel_flag, semantic is same as tone_map_id, tone_map_cancel_flag and tone_map_repetition_period defined in Tone mapping information SEI message (specified in Annex D).

colour_map_model_id specifies the model utilized for the colour mapping data. Values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore all colour mapping SEI messages that contain a value of colour_map_model_id greater than 1, and bitstreams shall not contain such values.

NOTE 1 – A colour_map_model_id of 0 corresponds to a 3D LUT colour mapping function; a colour_map_model_id of 1 corresponds to a first 1D LUT per component, a gain matrix plus offset and a second 1D LUT per component applied on each reconstructed samples of the decoded pictures successively.

colour_map_video_signal_type_present_flag, colour_map_video_format, colour_map_video_full_range_flag, colour_map_description_present_flag, colour_map_primaries, colour_map_transfer_characteristics, colour_map_matrix_coeffs semantic is the same as the semantic of the syntax elements video_signal_type_present_flag, video_format, video_full_range_flag, colour_description_present_flag, colour_primaries, transfer_characteristics, matrix_coeffs in VUI (specified in Annex E) respectively. These syntax elements describe the remapped output decoded pictures video signal characteristics.

## Decoding process for 1D LUT plus matrix colour data model

The luma and chroma or RGB samples are transformed with the input 1D LUT per component, matrix and output 1D LUT per component in this order. When one transform is not present, it is inferred to be identity.

| colour_map_1d_lut_plus_matrix_model_data () { | Descriptor |
| --- | --- |
| colour_map_coded_data_bit_depth | u(8) |
| for( i = 0; i < 3; i++ ) { |  |
| input_1d_lut_flag[ i ] | u(1) |
| if ( input_1d_lut_flag[ i ] ) { |  |
| num_input_pivots[ i ] | u(16) |
| for( j = 0; j < num_input_pivots[ i ]; j++ ) { |  |
| coded_input_pivot_value[ i ][ j ] | u(v) |
| target_input_pivot_value[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| matrix_flag | u(1) |
| if (matrix_flag) { |  |
| log2_matrix_denom | ue(v) |
| for( i = 0; i < 3; i++ ) { |  |
| for( j = 0; j < 3; j++ ) { |  |
| matrix_coef[ i][j] | se(v) |
| } |  |
| } |  |
| } |  |
| for( i = 0; i < 3; i++ ) { |  |
| output_1d_lut_flag[ i ] | u(1) |
| if ( output_1d_lut_flag[ i ] ) { |  |
| num_output_pivots[ i ] | u(16) |
| for( j = 0; j < num_output_pivots[ i ]; j++ ) { |  |
| coded_output_pivot_value[ i ] [ j ] | u(v) |
| target_output_pivot_value[ i ] [ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

colour_map_coded_data_bit_depth specifies the bit depth for interpretation of the luma and chroma or RGB components of the associated pictures for purposes of interpretation of the colour mapping information SEI message. When colour_map_coded_data_bit_depth is not equal to the luma and chroma or RGB bit depth, these refer to the hypothetical result of a transcoding operation performed to convert the luma and chroma or RGB samples to the bit depth corresponding to the value of colour_map_coded_data_bit_depth.

The value of colour_map_coded_data_bit_depth shall be in the range of 8 to 14, inclusive. Values of colour_map_coded_data_bit_depth from 0 to 7 and from 15 to 255 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all colour mapping SEI messages that contain a colour_map_coded_data_bit_depth in the range of 0 to 7, inclusive, or in the range of 15 to 255, inclusive, and bitstreams shall not contain such values.

input_1d_lut_flag[ i ] equal to 1 specifies input 1D LUT data is present for the i-th component. input_1d_lut_flag[ i ] equal to 0 specifies input 1D LUT data is inferred to be identity.

output_1d_lut_flag[ i ] equal to 1 specifies output 1D LUT data is present for the i-th component. output_1d_lut_flag[ i ] equal to 0 specifies output 1D LUT data is inferred to be identity.

num_input_pivots[ i ], num_output_pivots[ i ] specifies the number of pivot points in the input and output piece-wise linear mapping function without counting the two default end points, respectively, for the i-th component.

coded_input_pivot_value[ i ][ j ], coded_output_pivot_value[ i ][ j ] specifies the value in the colour_map_coded_data_bit_depth corresponding to the j-th pivot point of the input and ouput piece-wise linear colour mapping function respectively, for the i-th component. The number of bits used for the representation of the coded_pivot_value is ( ( colour_map_coded_data_bit_depth + 7 ) >> 3 )  <<  3.

target_input_pivot_value[ i ][ j ], target_output_pivot_value[ i ][ j ] specifies the value in the reference colour_map_coded_data corresponding to the j-th pivot point of the input and ouput piece-wise linear colour mapping function respectively, for the i-th component. The number of bits used for the representation of the target_pivot_value is ( ( colour_map_coded_data_bit_depth + 7 )  >>  3 )  <<  3.

log2_matrix_denom specifies the base 2 logarithm of the denominator for all matrix coefficients factors. The value of luma_log2_matrix_denom shall be in the range of 0 to 9, inclusive.

matrix_coef[ i ][ j ] specifies the value of the matrix coefficients transform factors.
outSample[ i ] = Clip3( 0, ( 1 << colour_map_coded_data_bit_depth ) − 1, (matrix_coef[ i ][ 0 ] * inSample[ 0 ] + matrix_coef[ i ][ 1 ] * inSample[ 1 ] + matrix_coef[ i ][ 2 ] * inSample[ 2 ] + (1<< (log2_matrix_denom-1))) >> log2_matrix_denom)

## Decoding process for 3D LUT colour data model (optional)

| colour_map_3d_ lut_model_data ( ) { | Descriptor |
| --- | --- |
| nbp_code | u(3) |
| lut_bit_depth_minus8 | u(4) |
| coding_octant( 0, 0, 0, 0 ) |  |
| } |  |

| coding_octant ( layer, y,u,v) { | Descriptor |
| --- | --- |
| for( i = 0; i < 8 ; i++ ) { |  |
| n = getVertex(y, u, v, i) |  |
| if (!coded_flag[n]) { |  |
| encoded_vertex_flag[i] | u(1) |
| if ( encoded_vertex_flag[i] ) { |  |
| res_y[i] | ue(v) |
| res_u[i] | ue(v) |
| res_v[i] | ue(v) |
| } |  |
| coded_flag[n] = true |  |
| } |  |
| } |  |
| if ( layer < nbp_code ) { |  |
| split_octant_flag | u(1) |
| if ( split_octant_flag ) { |  |
| for( i = 0; i < 8 ; i++ ) { |  |
| coding_octant ( layer+1, y+dy[i],u+du[i],v+dv[i]) |  |
| } |  |
| } |  |
| } |  |
| } |  |

nbp_code indicates the three dimensional LUTs size nbp. nbp is equal to 1+(1<<(nbp_code-1)).

lut_bit_depth_minus8 specifies the bit depth of the LUTs samples LutBitDepth as follows:

LutBitDepth      = 8 + lut_bit_depth_minus8							(7 4)

encoded_vertex_flag[i] equal to 1 specifies that the residuals for the ith vertex of the octant(layer,y,u,v) are present. encoded_vertex_flag[i] equal to 0 specifies that the residuals for the ith vertex of the octant(layer,y,u,v) are not present and are inferred to be equal to zero.

res_y[i], res_u[i], res_v[i] are the difference of the luma, chroma1 and chroma2 components of the vertex (y+dy[i], u+du[i], v+dv[i]) with the predicted luma, chroma1 and chroma2 component values for this vertex respectively. The derivation of the predicted component values for this vertex and the decoding of the color LUTs is specified in the decoding process for color LUT in 8.4.

split_octant_flag specifies whether an octant is split into four octants with half size in all directions for the purpose of vertices residuals octant coding.

The decoding of the Colour Mapping SEI message allows to obtain three arrays LUTY[iL][iC1][iC2], LUTC1[iL][iC1][iC2], LUTC2[iL][iC1][iC2]. The array indices iL, iC1, iC2 specify the reconstructed picture color space components sub-sampled ranging from 0 to (1+1<<(nbp_code-1)).

The decoding of the residual vertices of an octant( layer, y,u,v) is a recursive process. Each octant is composed of 8 vertices associated with a flag (encoded_vertex_flag[i]) indicating whether the residual components values are encoded or all inferred to be zero. The component values of the LUTs are reconstructed by adding the residuals to the prediction predX[i]X=Y,C1,C2 of the components values of the ith vertex as follows:

LUTX[ y+dy[i] ][ u+du[i] ][ v+dv[i] ] = resX[i] + predX[i] 					(7 5)

where the values of dy[i], du[i], dv[i] are given in Table 1.

Once reconstructed, a vertex n is marked as reconstructed (coded_flag[n]=true). The predicted component predX[i] of the vertex (y+dy[i], u+du[i], v+dv[i]) with X=Y, C1, C2, is obtained using trilinear interpolation of the neighboring vertices of the upper layer as follows:

- If layer equal to 0, then predX[i] is given by Table 2.

- Otherwise the value of predX[i] is obtained as follows:

predX[i] = ( Ai,0 + Ai,1 + Ai,2 + Ai,3 + Ai,4 + Ai,5 + Ai,6 + Ai,7 + (1<<(shift3-1)) )  >>  shift3

where the values of (Ai,k)i=0,7 k=0,7 and shift3 are derived as follows:

shift3 = 3 * (nbp_code  – layer)

Ai,k = wi,k * LUTX[ yr+dylayer-1[k] ][ ur+dulayer-1[k] ][ vr+dvlayer-1[k] ]

Where the values of wi,k, yr, ur, vr are derived as follows:

yr = ( (y >> (nbp_code - layer)) << (nbp_code - layer) )

ur = ( (u >> (nbp_code - layer)) << (nbp_code - layer) )

vr = ( (v >> (nbp_code - layer)) << (nbp_code - layer) )

if ( yr == (nbp-1) )	yr = yr - ( (nbp-1) >> (layer-1) )

if ( ur == (nbp-1) )	ur = ur - ( (nbp-1) >> (layer-1) )

if ( vr == (nbp-1) )	vr = vr - ( (nbp-1) >> (layer-1) )

wi,k  = sy(i,k) * su(i,k) * sv(i,k)

where the values of sy(i,k), su(i,k), sv(i,k) are derived as follows:

sy(i,k) = ((nbp-1) >> (layer-1)) - ABS( (yr+dylayer-1[k]) – (y+dylayer[i]) )

su(i,k) = ((nbp-1) >> (layer-1)) - ABS( (ur+dulayer-1[k]) – (u+dulayer[i]) )

sv(i,k) = ((nbp-1) >> (layer-1)) - ABS( (vr+dvlayer-1[k]) – (v+dvlayer[i]) )

Table 1: values dy[i],du[i] and dv[i] in function of index I, for vertices belonging to layer = layer_id.

| i | dylayer_id[i] | dulayer_id[i] | dvlayer_id[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | (nbp-1) >> layer_id |
| 2 | 0 | (nbp-1) >> layer_id | 0 |
| 3 | 0 | (nbp-1) >> layer_id | (nbp-1) >> layer_id |
| 4 | (nbp-1) >> layer_id | 0 | 0 |
| 5 | (nbp-1) >> layer_id | 0 | (nbp-1) >> layer_id |
| 6 | (nbp-1) >> layer_id | (nbp-1) >> layer_id | 0 |
| 7 | (nbp-1) >> layer_id | (nbp-1) >> layer_id | (nbp-1) >> layer_id |

Table 2: prediction values used for the 3 components of the 8 vertices belonging to the first layer (layer_id=0)(max=(1<< (bit_depth_lut_minus8+8)-1)).

| i | predY[i] | predC1[i] | predC2[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | max |
| 2 | 0 | max | 0 |
| 3 | 0 | max | max |
| 4 | max | 0 | 0 |
| 5 | max | 0 | max |
| 6 | max | max | 0 |
| 7 | max | max | max |

# References

[1] P. Bordes, P. Andrivon and F. Hiron, “Colour Mapping SEI message”, JCTVC-N0180, 14th JCT-VC Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

[2] P. Bordes, “Colour Mapping SEI message”, JCTVC-O0363, 15th JCT-VC Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013.

[3] P. Bordes, Y. He, “Colour Mapping SEI message”, m31186, 106th MPEG Meeting: Geneva, CH, 29 Oct. – 1 Nov. 2013.

[4] M. Pedzisz, Beyond BT.709, TM-AVC0633, 38th CM-AVC & 54th TM-AVC Meetings: Geneva, CH, 11 – 12 Dec. 2013.

[5] W. Husak, “Draft Requirements for Full Gamut Content Distribution”, w13822, 105th MPEG Meeting: Vienna, AT, Aug. 2013.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).