| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0127r2 |
| --- | --- |

| Title: | AHG14: On a profile supporting Bit Depth and Color Gamut Scalability in HEVC Scalable extensions |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Philippe Bordes, Edouard François (Technicolor) Xavier Ducloux (TVN) Yan Ye (InterDigital) Alberto Duenas (NGCodec Inc) Andrew Segall (Sharp Labs) Kemal Ugur (Nokia) Xiang Li (Qualcomm) Elena Alshina (Samsung) | Email: | pierre.andrivon@technicolor.com xavier.ducloux@thomson-networks.com Yan.Ye@InterDigital.com alberto.duenas@ngcodec.com asegall@sharplabs.com Kemal.Ugur@nokia.com lxiang@qti.qualcomm.com elena_a.alshina@samsung.com |  |
| Source: | Technicolor, BSkyB, Thomson Video Networks, InterDigital Communications, NGCodec Inc., Sharp Labs, Nokia, Qualcomm, H2B2VS project, Samsung Ltd. |  |  |  |

_____________________________

# Abstract

This contribution requests to define a profile for the HEVC scalable extension supporting Color Gamut and Bit-depth Scalability (CGS). Such a scalability technology may be considered in applications standardization committees such as DVB CM-AVC/CM-UHDTV or ATSC3.0. Considering the foreseeable evolution of capture and display device capabilities and use cases, CGS appears to be one major feature to be supported in upcoming mainstream applications. Finally, this contribution also suggests to establish a Liaison Statement on Color Gamut Scalability requirements and performance between DVB CM-AVC/CM-UHDTV, SC 29/WG 11 and SG 16.

# Introduction

Color Gamut Scalability (CGS) use cases ([1]) and requirements ([2]) were formalized in May 2012 at the 100th MPEG meeting. CGS allows addressing the case where the original Enhancement Layer (EL) samples are color encoded considering different primaries (e.g. Rec.2020, [11]) than the Base Layer (BL) samples (e.g. Rec.709) (Cf. figure 1). Beyond the video signal representation in a given color encoding system (deterministic process treated by color management theory), content may also be color graded (artistic process) differently according to both the artistic intents and the gamut of the reference monitor of the targeted application (HDTV, Digital Cinema, UltraHD TV…).

Figure 1: Different gamut in u’v’ – courtesy from BBC R&D

# Applications

Nowadays, it is common to find cameras and displays technologies that are able to respectively capture or render content beyond Rec.709 primaries (figure 2). CGS technology helps to address legacy workflow and provides a smooth transition towards wide color gamut systems. Rec.2020 is a future-proof color encoding system that enables to leverage such deeper color. CGS is considered in priority for consumer distribution scenarios such as VoD, satellite, cable, and terrestrial broadcast.

Figure 2a: Gamut of different displays - Figure 2b: Sony F55 and F65 cameras gamut - courtesy from Sony

As an example, DVB CM-UHDTV is working on a multi-phases deployment of Ultra HD TV.

- Requirements for UHD-1 phase 1 video signal is being finalized. It is likely that both Rec.709 and Rec.2020 encoded signal will be supported in this first version of the specification to be released in early 2014. Besides it is not obvious that Rec.2020 color encoding system signaling will be supported for HDTV.

- For UHD-1 phase 2, scalability may be considered for forward compatibility with phase 1 CE devices, including temporal scalability, and colour gamut scalability between Rec.709 Base Layer and Rec.2020 Enhancement Layer.

NB: Forward compatibility here means that UHD-1 phase 1 CE devices should be able to render UHD-1 phase 2 services.

Here are different envisioned use cases, scenarios or applications for Color Gamut Scalability.

## Application 1:  UHD-1 TV over HDTV offer

The CGS trigger use case defined in [1] is a 2160p UltraHD TV service deployed over 1080p HDTV service. Indeed, in the case that HDTV video is represented with Rec.709 primaries whereas UHD-1 video may be represented with Rec.2020 color encoding system and possibly color graded with a reference monitor whose gamut is (a subset of) Rec.2020 gamut but beyond Rec.709 gamut. This use case is the basis of current SCE1 test conditions. Potential gains are given in [6]. In the case broadcast standardization committees would be interested in HEVC scalability extensions for HDTV (with Rec.709 gamut) BL and UHD-1 phase 2 (likely to be Rec.2020 only) EL, CGS might appear as an opportunity as well.

## Application 2:Forward compatibility between both UHD-1 phases

UHD-1 phase 1 is likely to support both Rec.709 and Rec.2020 . Besides, requirements of UHD-1 phase 2 are to be drafted soon where a support of only Rec.2020 color space is not excluded. In that case, CGS with 2160p Rec.709 BL and 2160p Rec.2020 EL may be desirable.

# Color Gamut Scalability concept

First works ([3]) related to CGS in the scalable extension of HEVC (SHVC) were presented in JCT-VC in October 2012. An Ad Hoc Group [4] and a Core Experiment [5] have been established to assess different technologies.

The main challenge of CGS is to define a pertinent Color Difference Predictor (CDP, an inter-layer prediction mechanism) by predicting the Enhancement Layer samples from the reconstructed Base Layer samples (Cf. figure 3).

Figure 3: Color Gamut Scalability principle – courtesy from Sharp

# Performance

Considering a scalable scenario with 1080p HDTV Rec.709 8-bit/10-bit as BL (Rec.709 color graded content) and 2160p UHDTV Rec.2020 10-bit as EL (DCI-P3 color graded, Rec.2020 color encoded content, [10]), the following performance (average on AI+RA configuration) is obtained:

|  | CGS vs Single Layer - Average all cases |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Avg YUV* | Source |
| No CGS pred | 31.9% | 34.7% | 32.7% | 32.4% | SHM3.0.1 |
| CGS pred. (WP) | 26.1% | 31.7% | 24.6% | 26.6% | JCTVC-O0194 |
| CGS pred. (3D LUT) | 16.6% | 20.6% | 8.1% | 16.0% | JCTVC-O0161 |

Table 1: CGS versus single layer (AI + RA)

* BDR_YUV is computed here as (6*BDR_Y + BDR_U + BDR_V) / 8.

The second vertical entry in Table 1 (CGS pred. with WP) corresponds to weighted prediction tool used as CDP while the third vertical entry (CGS pred. with 3D LUT) corresponds to 3D LUT used as CDP.

Besides, as stated in [7] and [8], most efficient CGS technologies achieve the same level of performance over simulcast (20%) as the regular scalable coding of regular content (that is, the EL and BL have the same color gamut but different spatial resolutions [8][9]).

It is recommended that CGS achieves high performance in order not to limit the possibilities of this technology to be deployed as the return on investment could be too low to justify its development. Besides, described applications are likely to be considered as mainstream then low CGS performance may hinder SHVC deployment.

# Suggested approach

It is suggested:

- To consider high performance inter-layer prediction tools guaranteeing sufficient coding efficiency compared to simulcast,

- To solicit DVB CM-AVC/CM-UHDTV stakeholders to study the opportunity of using Bit-Depth and Color Gamut Scalability in the deployment of next generation HDTV and UHDTV phases then communicate any requirements (minimum performance, use cases, applications, complexity, implementation constraints) through a liaison statement to MPEG and VCEG.

- To define a bit-depth and CGS profile  considering expressed constraints.

# Support from H2B2VS project

The following partners from the Celtic+ H2B2VS project [12] are fully supporting this contribution:

- Thomson Video Networks,

- Digitürk,

- Ecole Polytechnique Fédérale de Lausanne,

- Hispasat,

- Nagravision,

- Neusoft,

- Télécom ParisTech,

- TDF,

- Teleste,

- Turk Telekom/Argela,

- Vestel.

# References

[1] Use cases of the scalable enhancement of HEVC, A. Luthra et al, N12782, May 2012, Geneva, CH

[2] Requirements of Scalable Coding Extensions of HEVC, A. Luthra et al, N12783, May 2012, Geneva, CH

[3] Color Gamut Scalable Video Coding, L. Kerofsky et al, JCTVC-K0241, Oct. 2012, Shanghai, CN

[4] JCT-VC AHG report: Color gamut scalability (AHG14), JCTVC-O0014, Oct. 2013, Geneva, CH

[5] Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability, P. Bordes et al, JCTVC-O1101, Oct. 2013, Geneva, CH

[6] SCE4: Summary Report of Color Gamut and Bit Depth Scalability, A. Segall et al, JCTVC-O0034, Oct. 2013, Geneva, CH

[7] SCE4: Results on 5.3-test1 and 5.3-test2, P. Bordes et al, JCTVC-O0159, Oct. 2013, Geneva, CH

[8] Non-SCE4/AHG14: Combined bit-depth and color gamut conversion with 3D LUT for SHVC color gamut scalability, Y. He et al, JCTVC-O0161, Oct. 2013, Geneva CH

[9]: AHG-17: complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast, Elena Alshina, Alexander Alshin, JCTVC-M0086, April 2013, Incheon, KR

[10] AHG14: Wide Color Gamut Test Material Creation, P. Andrivon et al, JCTVC-N0163, Aug. 2013, Vienna, AT

[11] Parameter values for UHDTV systems for production and international programme exchange, ITU-R Recommendation BT.2020, August 2012.

[12] H2B2VS website : http://h2b2vs.epfl.ch/

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Thomson Video Networks may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NGCodec Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).