| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0128 |
| --- | --- |

| Title: | SCE1: Results on Core Experiment on Color Gamut and Bit-Depth Scalability, tests 1A & 1B |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Pierre Andrivon, Edouard François, Franck Hiron, 975 avenue des champs blancs CS 17616, 35576, Cesson-Sévigné Cedex, France | Tel: Email: | +33-2-99-27-32-42 philippe.bordes@technicolor.com, pierre.andrivon@technicolor.com |  |
| Source: | Technicolor |  |  |  |

_____________________________

# Abstract

This contribution reports the performance analysis of SCE1 on Color Gamut and Bit-Depth Scalability, based on the use of 3D color Look-Up Tables (CLUT) to perform inter-layer prediction. Results of tests 1.A and 1.B for use cases 1 and 2, described in SCE1 description (JCTVC-O1101) are provided.

Considering use case 1 (one single SPS, PPS inserted at the sequence start), it is reported that compared with the SCE1 anchor (SHM4.0 with Weighted-Prediction (WP) enabled on inter-layer prediction), the CLUT method achieves an average BD rate gain of {-2.2%, -5.1%, -4.1%} for Y, U, V in AI, and {-3.3%, -4.6%, -5.8%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer, and an average BD rate gain of  {-0.9%, -4.1%, -3.3%} for Y, U, V in AI, and {-2.6%, -4.2%, -5.8%} for Y, U, V in RA using 10-bit base layer and 10-bit enhancement layer.

Considering use case 2 (one SPS, PPS inserted at RAP periodicity of one second), it is reported that compared with the SCE1 anchor (SHM4.0 with Weighted-Prediction (WP) enabled on inter-layer prediction), the CLUT method achieves an average BD rate gain of {-5.9%, -8.0%, -10.0%} for Y, U, V in AI, and {-5.1%, -6.1%, -9.3%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer, and an average BD rate gain of {-5.6%, -7.8%, -9.9%} for Y, U, V in AI, and {-5.1%, -6.1%, -9.7%} for Y, U, V in RA using 10-bit base layer and 10-bit enhancement layer.

It is reported the complexity of the proposed method compared to anchors is slightly lower.

# Introduction

The combined bit-depth and color gamut conversion method with CLUT for SHVC color gamut scalability was proposed in JCTVC-O0159 [2], where CLUT was trained offline. This proposal reported the performance of CLUT with SCE1 test conditions [1]. The CLUT derivation process is integrated into the encoder based on SCE1 anchor software released on November 28th (SHM4.0_irap_2013_11_28.zip).

# 3D LUT derivation and coding

As specified in SCE1 description, the CLUTs are now derived online at the encoder. The corresponding flow chart of our SHVC encoder is shown in Figure 1.

Figure 1: SHVC encoder with CLUT online training.

The target picture for CLUT training is the down-sampled picture from the EL input and the input source is the BL reconstructed picture. The CLUT vertices derivation is based on three steps.

Step 1: the 8 vertices of every octant are computed independently for each octant. The color conversion is locally modelled as a linear function (1), and the parameters (ai, bi, ci, oi)i=0,..2 are derived using Least Mean Squares method (LMS).

(1)

Next, the vertex values are computed using the linear function (1).

Step 2: in case the LMS method fails (because of few or no input source YUV sample are falling in this octant), the step 1 is iterated for the parent octant recursively.

Step 3: finally, the vertices of neighboring octants (neighboring octants are sharing same vertices) are merged by weighting with the number of input source YUV samples in the octants.

We use the same entropy coding method as in [2] for CLUT coding, and the CLUT is signaled in the PPS extension, the same way as in [2].

# Test results

Two tests (A and B) have been conducted. Test conditions are 2x spatial scalability with the following contents:

Table 1: Test conditions

|  | Base layer | Enhancement layer |
| --- | --- | --- |
| Test A | 1920 x 1080, BT. 709, 8-bit | 3840 x 2160, BT.2020, 10-bit |
| Test B | 1920 x 1080, BT. 709, 10-bit | 3840 x 2160, BT.2020, 10-bit |

The common SHVC test conditions (QPs) have been used for AI and RA configurations, 2x scalability.

Table 2 : List of the tests (AI, RA configurations) and corresponding anchor software used.

|  | Use case 1 | Use case 2 |
| --- | --- | --- |
| Test A | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap_2013_11_28 |
| Test B | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap_2013_11_28 |

Use case 1:

In this use case, the CLUTs are derived from the first picture of the sequence. Only one single SPS, PPS is encoded in the bit stream at the beginning. This use case is actually not really realistic, since the target scenarios are off-line encoded applications such as hardware support video (DVD, blu-ray, VoD, broadcast movies). For these applications, video coding is optimized using more than the first frame, then the CLUT training. Therefore this use case is provided as an informative input.

Table 3: Results of use case 1, tests 1.A (first column) and 1.B (second column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0.9% | -4.1% | -3.3% | -2.2% | -5.1% | -4.1% |
| Overall (Test vs single layer) | 17.7% | 17.8% | 14.6% | 18.8% | 18.4% | 14.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -2.3% | -5.6% | -4.2% | -4.7% | -7.5% | -6.1% |
| Overall (Test EL+BL vs single EL+BL) | -21.5% | -21.8% | -24.0% | -21.0% | -21.6% | -24.1% |
| Overall (Ref EL+BL vs single EL+BL) | -22.4% | -19.5% | -22.8% | -20.7% | -18.4% | -22.0% |
| Enc Time[%] | 107.9% |  |  | 108.4% |  |  |
| Dec Time[%] | 101.7% |  |  | 102.5% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -2.6% | -4.2% | -5.8% | -3.3% | -4.6% | -5.8% |
| Overall (Test vs single layer) | 24.1% | 28.2% | 19.3% | 24.9% | 29.0% | 20.3% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -4.8% | -6.2% | -7.7% | -6.0% | -7.1% | -8.2% |
| Overall (Test EL+BL vs single EL+BL) | -16.6% | -13.4% | -19.3% | -16.4% | -13.2% | -19.0% |
| Overall (Ref EL+BL vs single EL+BL) | -15.2% | -9.9% | -14.3% | -14.3% | -9.3% | -14.0% |
| Enc Time[%] | 70.7% |  |  | 70.3% |  |  |
| Dec Time[%] | 105.4% |  |  | 102.9% |  |  |

Use case 2:

In this use case, the CLUTs are derived at every IRAP frame. In this case, a SPS, PPS is encoded in the at IRAP period in the bit stream. We used IRAP periodicity of 1 second approximately (see Table 4). This use case is adapted to realistic applications scenarios considered for CGS (e.g. broadcast).

Table 4: IRAP periodicity in number of frames, depending on the frame-rate.

| Frame rate (Hz) | IRAP period |
| --- | --- |
| 50 | 48 |
| 60 | 64 |

Table 5: Results of use case 2, tests 1.A (first column) and 1.B (second column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.6% | -7.8% | -9.9% | -5.9% | -8.0% | -10.0% |
| Overall (Test vs single layer) | 11.9% | 13.1% | 6.7% | 14.0% | 14.7% | 7.8% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -11.5% | -13.3% | -15.4% | -12.0% | -13.7% | -15.7% |
| Overall (Test EL+BL vs single EL+BL) | -26.3% | -25.7% | -30.4% | -24.9% | -24.7% | -29.9% |
| Overall (Ref EL+BL vs single EL+BL) | -22.4% | -19.5% | -22.8% | -20.7% | -18.4% | -22.0% |
| Enc Time[%] | 96.3% |  |  | 96.7% |  |  |
| Dec Time[%] | 96.2% |  |  | 97.8% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.1% | -6.1% | -9.7% | -5.1% | -6.1% | -9.3% |
| Overall (Test vs single layer) | 20.9% | 25.7% | 14.4% | 22.5% | 27.1% | 15.9% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.9% |
| EL only (Test vs Ref) | -9.6% | -10.2% | -13.9% | -9.7% | -10.2% | -13.5% |
| Overall (Test EL+BL vs single EL+BL) | -19.3% | -15.5% | -23.4% | -18.4% | -14.8% | -22.6% |
| Overall (Ref EL+BL vs single EL+BL) | -15.2% | -9.9% | -14.3% | -14.3% | -9.3% | -14.0% |
| Enc Time[%] | 67.4% |  |  | 67.0% |  |  |
| Dec Time[%] | 104.7% |  |  | 107.7% |  |  |

Considering this use case, it is reported that compared with the SCE1 anchor, the CLUT method achieves an average BD rate gain in comparison with simulcast of -24.9% for luma in AI, and -18.4% for luma in RA using 8-bit base layer and 10-bit enhancement layer, and of -26.3% for luma in AI, and -19.3% for luma in RA using 10-bit base layer and 10-bit enhancement layer.

# Complexity evaluation (comparison with “worst case”)

In the following, we define as “worst case” the case all PUs are using bi-prediction, one prediction is ILR and the other is temporal intra layer prediction (ref_BL, ref_X). The complexity evaluation (memory bandwidth and number of operations) is evaluated in comparison with the SHVC Enhancement Layer (EL) decoding worst case. All the figures are given relatively to an enhancement layer pattern block of size 4x4.

## Complexity evaluation of the anchor

For the anchor, all the PUs are processed in bi-prediction (ref_BL, ref_X), using Weighted-Prediction (WP) for building the ILR prediction (Figure 2).

Figure 2: Anchor workflow process for building ILR (inter-layer) prediction with Weighted Prediction (WP).

In case of using CLUT model, the color gamut conversion is performed before up-sampling and the WP stage is not present (Figure 3).

Figure 3: CLUT workflow process for building ILR (inter-layer) prediction with CLUT.

For both the anchor and CGS models, the intra-layer temporal prediction uses Motion Compensation (Figure 4).

Figure 4: intra-layer temporal prediction workflow process.

The memory usage of the up-sampling, motion-compensation and weighted-prediction stages is depicted in the Table 6.

Table 6: memory usage per block of the up-sampling, motion-compensation and weighted-prediction stages.

| 2x scalability | CLUT | Up-sampling | Motion Comp. | Weight. Pred. |
| --- | --- | --- | --- | --- |
| Input | 4Y, 1U, 1V (8b) | 4Y, 1U, 1V (8b) | 16Y, 4U, 4V (10b) | 16Y, 4U, 4V (10b) |
| Temporary | (16b) | 16Y, 4U, 4V (16b) | 16Y, 4U, 4V (16b) | (16b) |
| Output | 4Y, 1U, 1V (10b) | 16Y, 4U, 4V (10b) | 16Y, 4U, 4V (10b) | 16Y, 4U, 4V (10b) |

Up-sampling process:

The luma and chroma up-sampling filters are 8-taps and 4-taps respectively.

Horizontal up-sampling: (4Y,1U,1V) => (8Y, 2U, 2V)

nb_mult:	8x8 + 2x4 + 2x4	= 80

nb_add:		8x7 + 2x3 + 2x3	= 68

Vertical up-sampling: (8Y, 2U, 2V) => (16Y, 4U, 4V)

nb_mult:	16x8 + 4x4 + 4x4	= 160

nb_add:		16x8 + 4x4 + 4x4	= 160

nb_shift:	16x1 + 4x1 + 4x1	= 24

Total (Up-sampling) operations:	240 mult, 228 add, 24 shift.

Motion-compensation process:

The Luma and chroma motion compensation filters are 8-taps and 4-taps respectively.

Horizontal up-sampling: (16Y, 4U, 4V) => (16Y, 4U, 4V)

nb_mult:	16x8 + 4x4 + 4x4	= 160

nb_add:		16x8 + 4x4 + 4x4	= 160

Vertical up-sampling: (16Y, 4U, 4V) => (16Y, 4U, 4V)

nb_mult:	16x8 + 4x4 + 4x4	= 160

nb_add:		16x8 + 4x4 + 4x4	= 160

nb_shift:	16x1 + 4x1 + 4x1	= 24

Total (Motion compensation) operations:	320 mult, 320 add, 24 shift.

Weighted prediction process:

In the worst case (wc), the bi-prediction is built with one ref_BL (ILR) and one temporal (intra-layer) reference (ref_X). Then the ref_BL prediction part use the uni-prediction weighted prediction.

Pel weightUnidirY( Int w0, Pel P0, Int round, Int shift, Int offset)

out_Y = ( (w0*(P0 + IF_INTERNAL_OFFS) + round) >> shift ) + offset

(16Y, 4U, 4V) => (16Y, 4U, 4V)

nb_mult:	16x1 + 4x1 + 4x1	= 24

nb_add:		16x3 + 4x3 + 4x3	= 72

nb_shift:	16x1 + 4x1 + 4x1	= 24

Total:	24 mult, 72 add, 24 shift.

## Complexity evaluation of the  CLUTs process

### Memory consideration

Frame buffer memory:

This section studies the complexity impact in terms of memory of the additional step in inter-layer processing at the decoder of using CGS with CLUTs.

Figure 5: Frame buffer memory requirement for SHVC decoder (top) and SHVC-CGS decoder (bottom).

- BL reconstructed:				1920x1080 YUV frame buffer (8 or 10 bits)

- FullPelBaseRec(ref_idx):			3840x2160 YUV frame buffer (10 bits)

- Temporary buffer (EL reconstructed):	3840x2160 YUV frame buffer (16 bits)

The result of the color gamut conversion with CLUT can be temporarily stored in the buffer allocated to the ILR reference picture (FullPelBaseRec(ref_idx) buffer), prior to the upsampling process, as illustrated in Figure 5. Therefore, in the suggested implementation, no additional frame buffer memory is required.

In the decoder design, the color gamut conversion with CLUT may be advantageously merged with the up-sampling process based on linear operations, so it does not need to store the intermediate results (output of color gamut conversion) in global memory, and fetch it again for up-sampling.

CLUT memory:

In case of 9x9x9 Color LUT (3 components: Y,U,V), 8 bits precision, the amount of data to store is :

(9x9x9) x 3 x 8 = 17496 bits

### Number of operations

In this section, the number of operations is computed for the CLUT interpolation of 4 Y samples and 2 chroma samples (which will results after up-sampling into a 4x4 block). The input to the “Color gamut conversion with 3D LUT” module is the reconstructed BL (1920x1080) 8-bit (test-A) or 10-bit (test-B).  The output to the “Color gamut conversion with  3D LUT” module is the  10-bit frame buffer BL size (1920x1080).

TComLut3D::interpol_tetra_Lum()

TComLut3D::interpol_tetra_Chroma()

Inputs (4:2:0 case):

4 Y samples (in_Y[n], n=0,…3),

1 Chroma U sample (in_U)

1 Chroma V sample (in_V)

Outputs (4:2:0 case):

4 Y samples (out_Y[n], n=0,…3),

1 Chroma U sample (out _U)

1 Chroma V sample (out _V)

Luma sample operations:

The first and second operations are common for the 6 samples Y0, Y1, Y2, Y3, U, V. The third step is done for each component Y,U,V.

- First step: 3 shifts are used to determine which LUT vertices (ni, i=0,…8) to be used (i,j,k).

int		i = in_Y[n] >> shift_div;

int 	j = in_U >> shift_div;	// done only once for the 6 samples (4 luma and 2 chroma samples)

int 	k = in_V >> shift_div;	// done only once for the 6 samples

- Second step: the weights (er,eg,eb) are computed as a difference.

int 	er  = in_Y[n] & (m_dr-1);

int 	eg  = in_U & (m_dg-1);		// done only once for the 6 samples

int 	eb  = in_V & (m_db-1);		// done only once for the 6 samples

- Third step: 3 differences and 3 multiples, and finally one shift (shift_out).

if ((er >= eg) && (eg >= eb))	// done for each 4 Y samples

tmp_Y = ((n0->sY) << shift_div) + er * (n4->sY - n0->sY) + eg *(n6->sY - n4->sY)+ eb *(n7->sY - n6->sY);

else if((eb >= er) && (er > eg))

…

out_Y[n]   = (int)( (tmp_Y + offset) >> shift_out )

Additional operations for the Chroma samples:

Preliminary step:

int	tmp_Y = (in_Y[0] + in_Y[1] + in_Y[2] + in_Y[3] + 2) >> 2

- First step: 1 shift is used to determine which LUT vertices to be used (i,j,k).

int		i = tmp_Y >> shift_div;		// done only once for the first chroma samples

- Second step: one weights (er) is computed as a difference.

int 	er  = tmp_Y & (m_dr-1); 	// done only once for the first chroma samples

one masking operation.

- Third step: 3 differences and 3 multiples, and finally one shift (shift_out).

if ((er >= eg) && (eg >= eb)) {	// done for each 4 Y samples

tmp_U = ((n0->sU) << shift_div) + er * (n4->sU - n0->sU) + eg *(n6->sU - n4->sU)+ eb *(n7->sU - n6->sU);

tmp_V = ((n0->sV) << shift_div) + er * (n4->sV - n0->sV) + eg *(n6->sV - n4->sV)+ eb *(n7->sV - n6->sV);

}

else if((eb >= er) && (er > eg))

…

out_U   = (int)( (tmp_U + offset) >> shift_out )

out_V   = (int)( (tmp_V + offset) >> shift_out )

| Nb shifts |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Preliminary | First | Second | Third | Total |
| Y0 |  | 3 | 3 | 2 | 8 |
| Y1 |  | 1 | 1 | 2 | 4 |
| Y2 |  | 1 | 1 | 2 | 4 |
| Y3 |  | 1 | 1 | 2 | 4 |
| U | 1 | 1 |  | 2 | 4 |
| V |  |  |  | 2 | 2 |
|  |  |  |  |  | 26 |

| Nb sum |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Preliminary | First | Second | Third | Total |
| Y0 |  |  | 3 | 7 | 10 |
| Y1 |  |  | 1 | 7 | 8 |
| Y2 |  |  | 1 | 7 | 8 |
| Y3 |  |  | 1 | 7 | 8 |
| U | 4 |  |  | 7 | 11 |
| V |  |  |  | 7 | 7 |
|  |  |  |  |  | 52 |

| Nb mult |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Preliminary | First | Second | Third | Total |
| Y0 |  |  |  | 3 | 3 |
| Y1 |  |  |  | 3 | 3 |
| Y2 |  |  |  | 3 | 3 |
| Y3 |  |  |  | 3 | 3 |
| U |  |  |  | 3 | 3 |
| V |  |  |  | 3 | 3 |
|  |  |  |  |  | 18 |

Total (color conversion CLut) operations:	18 mult, 52 add, 26 shift.

## Complexity analysis

For the anchors, the Color gamut conversion process is performed using the Weighted-Prediction (WP), while for the tested (CLUT) implementation it is performed using LUTs interpolation. In Table 7 we provide the number of operations for these processes and also for the up-sampling and motion compensation stages (worst case bi-prediction).

In the last column of Table 7, we compare the relative number of operations of the Color gamut conversion process using CLUT compared to using WP.

The comparison of the overall number of operations using anchors workflow (Color gamut conversion performed with WP) and CLUT workflow (Color gamut conversion performed with CLUT), including the up-sampling and motion compensation processes, is provided in Table 8.

Table 7: Comparison of the number of operations for an enhancement 4x4 block of the processing steps, and relative ratio of CLUT color conversion vs Weighted Prediction (WP).

| nb total operations, 2x scalability | CLUT Color gamut conversion | Up-sampling | Motion comp. | Weighted pred. (WP) | CLUT conversion vs WP |
| --- | --- | --- | --- | --- | --- |
| Ref. |  | x | x | x |  |
| Tested | x | x | x |  |  |
| mult | 18 | 240 | 320 | 24 | -25% |
| add | 52 | 228 | 320 | 72 | -28% |
| shift | 26 | 24 | 24 | 24 | +8% |

Table 8: Comparison of the overall number of bi-prediction operations for an enhancement 4x4 block (color conversion + up-sampling, motion estimation) using CLUT vs anchors (up-sampling + WP, motion estimation) (16x16 PU size, 2Nx2N).

| nb total operations, 2x scalability | Anchor workflow (wc) | CLUT workflow | CLUT gain vs Anchor |
| --- | --- | --- | --- |
|  |  |  | wc |
| mult | 584 | 578 | -1% |
| add | 620 | 600 | -3% |
| shift | 72 | 74 | +3% |

# Conclusion

Considering use case 1 (one single SPS, PPS inserted at the sequence start), it is reported that compared with the SCE1 anchors, the CLUT method achieves an average BD rate gain of {-2.2%, -5.1%, -4.1%} for Y, U, V in AI, and {-3.3%, -4.6%, -5.8%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer.

Considering use case 2 (one SPS, PPS inserted at RAP periodicity of one second), it is reported that compared with the SCE1 anchors, the CLUT method achieves an average BD rate gain of {-5.9%, -8.0%, -10.0%} for Y, U, V in AI, and {-5.1%, -6.1%, -9.3%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer.

Considering the use case 2 the CLUT method achieves an average BD rate gain in comparison with simulcast of -24.9% for luma in AI, and -18.4% for luma in RA using 8-bit base layer and 10-bit enhancement layer, and of -26.3% for luma in AI, and -19.3% for luma in RA using 10-bit base layer and 10-bit enhancement layer.

It is reported the complexity of the proposed method compared to anchors is slightly lower.

# Patent rights declaration(s)

Technicolor Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- P. Bordes, Y. Ye, E. Alshina, X. Li, S.-H. Kim, A. Duenas, K. Ugur, K. Sato, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-O1101, Oct. 2013, CH.

- P. Bordes, P. Andrivon, F. Hiron, P. Salmon, R. Boitard, “SCE4: Results on 5.3-test1 and 5.3-test2”, JCTVC-O0159, Oct. 2013, CH.

- P. Bordes, P. Andrivon, F. Hiron, “AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results”, JCTVC-N0168, Jul. 2013.