| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0131 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0132 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Layer up-switching information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes an SEI message that signals the information that is asserted to assist layer up-switching. The SEI message signals, when the current layer is the highest layer being decoded, the locations in higher layers where layer up-switching can be performed.

# Introduction

In streaming scenarios, a streaming server or middle box may be serving users up until a certain layer. When a client request is made to switch up by one or more layers, or if the streaming server or middle box determines that if the network situation allows more layers to be transmitted, information of when the bitstream can switch to one or more higher layers would be useful in scheduling the up-switch.

For middle boxes to perform layer up-switching, it would not be sufficient to just look for the next IRAP picture in the highest layer to determine a layer up-switch point. Figure 1 shows an example where unless all the direct and indirect reference layers are initialized, even IRAP pictures could be cross-layer random access skipped (CL-RAS) pictures that may not be correctly decodable. In Figure 1, even though an IRAP picture occurs at EL2 in AU 7, the picture may not be correctly decodable due to inter-layer prediction relation to pictures in EL1.

Figure 1: A bitstream where there is a linear reference relationship between layer, EL1 refers to BL and EL2 refers to EL1. The first IRAP picture in EL2 is a CL-RAS picture as EL1 is no initialized until after AU 11.

Hence, it is not always sufficient to just look at the nal_unit_type of the pictures in a higher layer to determine whether layer up-switching can happen at a particular AU. Therefore, it would be useful to provide information in the bitstream to easily identify when layer up-switch can be performed.

# Proposal

## Layer up-switching information SEI message syntax

| layer_upswitching_information( payloadSize ) { | Descriptor |
| --- | --- |
| lups_sei_active_vps_id | u(4) |
| lups_num_layers_minus1 | ue(v) |
| for( i = 0; i <= lups_num_layers_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_sub_layers_minus1; j++ ) |  |
| lups_num_aus_to_switch_up[ i ][ j ] | ue(v) |
| } |  |

## Layer up-switching information SEI message semantics

When present, the layer up-switching information SEI message shall be associated with an IRAP picture. The layer containing the IRAP picture associated with the SEI message is referred to as the current layer. The IRAP picture associated with the SEI message is referred to as the current picture.

The layer up-switching information SEI message signals, when the current layer is the highest layer being decoded, the locations in higher layers where layer up-switching can be performed.

The nuh_layer_id of the NAL unit containing this SEI message, currLayerId, shall be equal to the nuh_layer_id of the IRAP picture that is associated with the SEI message.

lups_sei_active_vps_id identifies the active VPS referred to by the VCL NAL units of the IRAP picture associated with the layer up-switching information SEI message. The value of lups_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS.

lups_num_layers_minus1 plus 1 specifies the number of layers for which the SEI message describes layer up-switching information. The value of lups_num_layers_minus1 shall be in the range of 0 to vps_max_layers_minus1, inclusive.

lups_num_aus_to_switch_up[ i ][ j ] specifies a layer up-switching access unit to the (LayerIdxInVps[ currLayerId ] + i + 1)-th layer specified by the active VPS when HighestTid is equal to j. The layer up-switch access unit to the (LayerIdxInVps[ currLayerId ] + i + 1)-th layer is the lups_num_aus_to_switch_up[ i ][ j ]-th access unit that follows, in decoding order, the access unit containing the SEI message, excluding access units with TemporalId greater than j. The layer up-switch access unit shall contain an IRAP picture picA in the (LayerIdxInVps[ currLayerId ] + i + 1)-th layer. When the decoding process starts at the current access unit or a previous access unit in decoding order, and the value of LayerInitializedFlag[ currLayerId ] is equal to 1 before decoding the current picture, the picture picA and all pictures that are in the (LayerIdxInVps[ currLayerId ] + i + 1)-th layer and that succeed picA in decoding order, excluding the RASL pictures associated with the picture picA, can be correctly decoded.

The value of lups_num_aus_to_switch_up[ i ][ j ] shall be greater than or equal to the value of lups_num_aus_to_switch_up[ i ][ j - 1 ], for j in the range of 1 to vps_max_sub_layer_minus1, inclusive. For i in the range of 1 to lups_num_layers_minus1, inclusive, and k in the range of 0 to lups_num_layers_minus1 – 1, inclusive, the value of lups_num_aus_to_switch_up[ i ][ j ] shall be greater than or equal to the value of lups_num_aus_to_switch_up[ k ][ j ] when the (LayerIdxInVps[ currLayerId ] + k)-th layer is a direct or indirect reference layer of  the (LayerIdxInVps[ currLayerId ] + i)-th layer. The value of lups_num_aus_to_switch_up[ i ][ j ] shall be in the range of 0 to 216 – 1, inclusive.

NOTE – A value of 0 for lups_num_aus_to_switch_up[ i ][ j ] indicates that the i-th layer after the current layer is decodable starting from the current access unit.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).