| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0132_r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0133_r1 |
| --- | --- |

| Title: | On alt_output_layer_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang Hendry Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

At the 15th JCT-VC/6th JCT-3V meetings, the syntax element alt_output_layer_flag was introduced into the SHVC/MV-HEVC specifications. This document highlights several problems that are associated with this flag, including in output of pictures when there are multiple views, multiple target output layers or multiple resolutions across layers, and considerable complexity for the encoders to enable this flag. It is proposed that this flag be removed; if not removed, it is proposed that the identified problems to be fixed.

The first revision of this contribution adds snippets from JCTVC-O1008_v3 where the suggested removal of syntax element alt_output_layer_flag is applied.

# Introduction

The SHVC/MV-HEVC specifications allow one or more target output layers for a bitstream, and the target output layers for an output layer set are specified in the video parameter set (VPS) extension. A picture in an access unit is to be outputted if the picture belongs to a target output layer and has the associated PicOutputFlag equal to 1.

The syntax element alt_output_layer_flag is signalled in the VPS extension. The semantics of the flag are given below.

alt_output_layer_flag affects picture output as specified in subclause F.13. [Ed. (GT) semantics should be more specific.]

NOTE 1 – When alt_output_layer_flag is equal to 0, pictures that are not at the target output layers are not output. When alt_output_layer_flag equal to 1 and a picture at the a target output layer is not present in an access unit, a picture with highest nuh_layer_id among those pictures of the access unit for which PicOutputFlag is equal to 1 and which are not among the target output layers is output.

The purpose of this flag is to output a picture in an access unit from a layer that is not a target output layer, when a picture at a target output layer is not present in that access unit. When a picture is not present in a target output layer, a picture that does not belong to a target output layer and that has PicOutputFlag equal to 1 is output – such output behaviour would be henceforth referred to as the alternate output behaviour. The determination of whether and how a picture from a layer that is not a target output layer is output is determined as follows in Annex C:

The following applies for all decoded access units:

–	If an access unit does not contain a picture at a target output layer and alt_output_layer_flag is equal to 1, the following ordered steps apply:

–	The list nonOutputLayerPictures is the list of pictures of the access unit with PicOutputFlag equal to 1 and with nuh_layer_id values that are included in the TargetDecLayerIdList and that are not on target output layers.

–	The picture with the highest nuh_layer_id value among the list nonOutputLayerPictures is removed from the list nonOutputLayerPictures.

–	PicOutputFlag for each picture that is included in the list nonOutputLayerPictures is set equal to 0.

–	Otherwise, PicOutputFlag for pictures that are not included in a target output layer is set equal to 0.

# Problem Statement

The purpose of this flag has been to specify a conformant behaviour for certain cases when a target output layer picture is not present, e.g. in the so-called layer-wise startup decoding process. However, after further study on this feature, we found that it has several problems as discussed below.

## Burden on encoders

The flag specifies the cases where pictures from non-target output layers may be output. The alternate output behaviour could be triggered if the picture in a target output layer is not present for any reason; e.g. the picture may not be present in the original bitstream, the picture may have been a discardable picture, the picture may have been lost. This could provide undue burden on the encoders to determine several parameters in the bitstream.. The resultant effect desired is to have a conforming behaviour in all these cases.

Consider the example given in Figure 1. The bitstream contains 3 layers, and the output layer considered is only the EL2. To set alt_output_layer_flag equal to 1, the encoder has to ensure that the DPB size and so on work correctly when either of both of pictures from BL and EL1 in the AUs containing pictures A and B are output. Typically (when alt_output_layer_flag is equal to 0), for this output layer set, the sub-DPB size corresponding to the layers BL and EL1 could be determined by just considering the reference picture sets of the respective layers as these layers are not output. However, to enable this feature to be used, the encoder has to consider all the cases where the picture from BL or EL1 could be a candidate to output. Considering all the possible means by which pictures may not be present, including picture loss and discarding (particularly when the resultant bitstream is still conforming), the encoder would need to consider all these cases to determine the correct sub-DPB size to be assigned.

Figure 1 – Illustration of possible burden for encoder when alt_output_layer_flag is equal to 1

## More than one target output layer

Consider a bitstream where there is more than one target output layer. When a picture at any one of the target output layers is not present in an access unit, another picture in the same access unit that is the picture that has the highest nuh_layer_id among the layers that are not target output layers is output. Even if more than one picture at target output layers is not present in an access unit, only one picture at a layer that is not a target output layer is output. This behaviour seems to be unclear.

Figure 2 – Illustration for possible problems

Figure 2 highlights several problems with alt_output_layer_flag when there are more than one target output layer. The layers EL2 and EL3 are target output layers. The following access units are of relevance to the discussion:

- Access unit 1 – in this access unit, the picture in EL3 is not present and instead the picture in EL1 is chosen for output. In an output order conformant decoder that decodes the above bitstream, at most two pictures are output for each access unit – the first picture from EL2 and the second picture from EL3. However, in AU 1, the first picture that would be output is the picture from EL1, followed by the picture from EL2. The output of pictures in AU 1 is in the incorrect order as picture from EL2, when output, is always the first picture to be output within an access unit.

- Access unit 8 – in this access unit, neither picture in the target output layers is present. It is not clear whether both pictures in BL and EL1 shall be outputted for only EL1 shall be outputted twice. The behavior and mechanism regarding seems to be not clear.

## Multi-view coding

For multi-view case, it would not be good to replace a picture in a target output layer with a picture from another layer, since a different layer belongs to different views. For example, in a three view case wherein decoded pictures may be used for view synthesis of the views in the intermediate viewing positions. Outputting a different view than the target output view might need to completely messed-up view synthesis operation. In this case, it is much better just not to output any alternative layer.

In case of two views are needed for output, while a third view is used to replace a second view of these two when alt_output_layer_flag is equal to 1, the stereoscopic viewing experience will not be good, since the third view will in a different viewing position, thus corresponds to completely different 3D perception. The unexpected 3D perception might either be compensated by the render or viewers adjusted manually by the viewer with a specific UI. In any case, the user experience is unacceptable.

# Proposal

The feature based on alt_output_layer_flag has the following issues:

- It imposes a heavy burden for encoders to be able to utilize the feature, as described in Section 2.1.

- Decoder implementation can try to achieve the same behavior without the need of such flag, as it can choose to output any picture or perform any other course of action when a picture at a target output layer is not present in an access unit.

- When a picture with a different resolution from a lower layer is output in between pictures from the target output layer, all in the same coded video sequence, the sequence of output pictures, if displayed, would look odd without interpolation and so on. That means, some implementation-specific trick would be needed anyway for typical use cases such as spatial scalability to make the solution complete.

- When there is more than one target output layer, the behaviour of the flag could lead to wrong order of output of pictures within an access unit. One could signal alt_output_layer_flag for each output layer set, and restrict the value of the flag to be equal to 0 when the corresponding output layer set has more than one target output layer.

- When multiple views are coded, user experience would be affected when a picture from one view is output instead of a picture from another view. Thus, the alt_output_layer_flag should not be applied to multi-view scenarios.

Based on the above discussed problems, we propose that the alt_output_layer_flag be removed. If the group decides to keep the flag, we propose that the alternate output based on alt_output_layer_flag should not be applied to multi-view scenarios and the current text specification should be improved to clarify the behavior that can overcome the problems discussed above.

# Specification text after removal of alt_output_layer_flag

The following snippets of text that show the specification text (i.e., based on JCTVC-O01008_v3) with the proposed suggested removal, with additions highlighted in yellow, and removals strikethrough and in red font.

- ------------------------------------------------------ Start of snippets --------------------------------------------------

- C.3.1. General

The specifications in this subclause apply independently to each set of DPB parameters selected as specified in subclause Error: Reference source not found.

The decoded picture buffer consists of sub-DPBs, and each sub-DPB contains picture storage buffers for storage of decoded pictures of one layer only. Each of the picture storage buffers of a sub-DPB may contain a decoded picture that is marked as "used for reference" or is held for future output.

The following applies for all decoded access units:

–	If an access unit does not contain a picture at a target output layer and alt_output_layer_flag is equal to 1, the following ordered steps apply:

–	The list nonOutputLayerPictures is the list of pictures of the access unit with PicOutputFlag equal to 1 and with nuh_layer_id values that are included in the TargetDecLayerIdList and that are not on target output layers.

–	The picture with the highest nuh_layer_id value among the list nonOutputLayerPictures is removed from the list nonOutputLayerPictures.

–	PicOutputFlag for each picture that is included in the list nonOutputLayerPictures is set equal to 0.

–	Otherwise, PicOutputFlag for pictures that are not included in a target output layer is set equal to 0.

The processes specified in subclauses Error: Reference source not found, Error: Reference source not found and Error: Reference source not found are sequentially applied as specified below, and are applied independently for each layer, starting from the base layer, in increasing order of nuh_layer_id values of the layers in the bitstream. When these processes are applied for a particular layer, only the sub-DPB for the particular layer is affected. In the descriptions of these processes, the DPB refers to the sub-DPB for the particular layer, and the particular layer is referred to as the current layer.

….

- C.5.2.3 Picture decoding, marking, additional bumping, and storage

The processes specified in this subclause happen instantaneously when the last decoding unit of picture n is removed from the CPB. [Ed. (MH): This change might not comply with version 1, because version 1 decoders would mark and store the base-layer picture at the CPB removal time of the AU, which can be later than the CPB removal time of the base-layer picture.]

PicOutputFlag is updated as follows:

–	If the current access unit does not contain a picture at a target output layer and alt_output_layer_flag is equal to 1, the following ordered steps apply:

–	The list nonOutputLayerPictures is the list of pictures of the access unit with PicOutputFlag equal to 1 and with nuh_layer_id values that are included in the TargetDecLayerIdList and that are not on target output layers.

–	The picture with the highest nuh_layer_id value among the list nonOutputLayerPictures is removed from the list nonOutputLayerPictures.

–	PicOutputFlag for each picture that is included in the list nonOutputLayerPictures is set equal to 0.

–	Otherwise, PicOutputFlag for pictures that are not included in a target output layer is set equal to 0.

When the current picture has PicOutputFlag equal to 1, for each picture in the current layer in the sub-DPB that is marked as "needed for output" and follows the current picture in output order, the associated variable PicLatencyCount is set equal to PicLatencyCount + 1.

….

- F.7.3.2.1.1 Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| if( vps_max_layers_minus1 > 0 ) |  |
| alt_output_layer_flag | u(1) |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(8) |
| … |  |
| } |  |

- F.7.4.3.1.1 Video parameter set extension semantics

….

profile_level_tier_idx[ i ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive.

alt_output_layer_flag affects picture output as specified in subclause F.13. [Ed. (GT) semantics should be more specific.]

NOTE 1 – When alt_output_layer_flag is equal to 0, pictures that are not at the target output layers are not output. When alt_output_layer_flag equal to 1 and a picture at the a target output layer is not present in an access unit, a picture with highest nuh_layer_id among those pictures of the access unit for which PicOutputFlag is equal to 1 and which are not among the target output layers is output.

rep_format_idx_present_flag equal to 1 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are present. rep_format_idx_present_flag equal to 0 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are not present.

….

------------------------------------------------------- End of snippets ---------------------------------------------------

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).