| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0134 |
| --- | --- |

| Title: | Strawman SHVC level constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

The current SHM draft specification does not define level constraints for SHVC.  Strawman Scalable Main profile-specific level constraints are proposed.  All of the existing general level constraints would apply individually to each layer within the operation point corresponding to the output layer set conforming to the profile/tier/level.  In addition, cumulative constraints for all decoded layers are imposed for total picture size, total sample rate and total bitrate, equal to 2x their respective level limits for Main profile.

# Proposal

## Background

In the SHM draft, profile/tier/level conformance points apply to an output layer set.  An operation point is the sub-bitstream associated with the output layer set, and includes all layers which are to be decoded.

## Proposed concepts

The general tier and level constraints of version 1 (in A.4.1) are not changed, but apply to each layer represented included in the operation point individually.  For example, no layer may contain pictures which exceed the max picture size, or exceed the max number of tiles, etc., associated with the level.  Additionally, each target decoded layer in the target output layer set must individually obey all of the Main profile-specific tier and level constraints (in A.4.2), except for those related to bitrate.

Additional Scalable Main profile-specific tier and level constraints are imposed for the cumulative set of decoded layers in the operation point which conforms to Scalable Main profile.

- The cumulative total picture size of all decoded layers is proposed to equal to 2x the limit for Main profile

- The cumulative total sample rate of all decoded layers is proposed to equal to 2x the limit for Main profile

- A constraint on the cumulative total bitrate of all decoded layers is proposed to equal 2x the limit for Main profile

## Discussion

Levels are typically defined to correspond to expected use cases, so that decoders conforming to a particular level can handle the expected use case without needing to have a lot of unnecessary overhead in capability.

Selecting level constraints for the cumulative sample rate and bitrate at 2x the single layer rates allow both 2-layer SNR scalability and 3+ layer spatial scalability to use the same level as is used for single layer at the highest resolution. It also allows similar constraints as the 2-view case supported in the MV-HEVC Stereo Main profile.

However, this would over-provision the typical 2-layer spatial scalability case, which would only require 1.25x for 2:1 spatial scalability or 1.45x for 1.5:1 spatial scalability.

Consider the following examples

- Two-layer SNR scalable bitstream: layer 0 at 1920x1080, layer 1 at 1920x1080, both at 60 fps.  Two operation points defined, with OP-A0 including only layer 0, and OP-A1 including both layer 0 and layer 1.

- Three-layer spatial scalable bitstream: layer 0 at 640x480, layer 1 at 1280x720, layer 2 at 1920x1080, all at 60fps.  Three operation points defined, with OP-B0 including only layer 0, OP-B1 including layers 0 and 1, and OP-B2 including layers 0, 1, and 2.

The tables below indicate which level would be used for the typical operation points that would be defined for those use cases.  In both cases, the level_idc for the operation point is identical to the level_idc that would be used for a single layer bitstream at the same resolution as the highest resolution included in the operation point.

Example A: Two layer SNR scalable

| Operation point | Layers included | Resolutions/frame rates to be decoded | level_idc |
| --- | --- | --- | --- |
| OP-A0 | 0 | 1920x1080/60 | 4.1 |
| OP-A1 | 0, 1 | 1920x1080/60, 1920x1080/60 | 4.1 |

Example B: Three layer 2x spatial scalable

| Operation point | Layers included | Resolutions/frame rates to be decoded | level_idc |
| --- | --- | --- | --- |
| OP-B0 | 0 | 640x480/60 | 3 |
| OP-B1 | 0, 1 | 640x480/60, 1280x720/60 | 4 |
| OP-B2 | 0, 1, 2 | 640x480/60, 1280x720/60, 1920x1080/60 | 4.1 |

## Proposed specification text

H.11.2 General tier and level limits

Bitstreams containing output layer sets conforming to the Scalable Main profile shall obey the constraints of A.4.1, for each layer in the TargetDecLayerIdList associated with layerSetIdx, which is the layer set for an output layer set conforming to the Scalable Main profile:

H.11.3 Profile specific tier and level limits for the Scalable Main profile

Bitstreams containing output layer sets conforming to the Scalable Main profile shall obey the following constraints, with layerSetIdx being the layer set for an output layer set, and TargetDecLayerIdList being the target decoder layer identifier list for the output layer set, conforming to the Scalable Main profile:

- Each layer in the TargetDecLayerIdList shall obey the Bitstreams Profile-specific level limits a), b), c), d) g), h), i), and j) for the Main profile specified in A.4.2

- The value of TotalPicSizeInSamplesY shall be less than or equal to 2 * MaxLumaPs, where MaxLumaPs is specified in Error: Reference source not found, and where TotalPicSizeInSamplesY is derived as follows:

TotalPicSizeInSamplesY = 0

for (i = 0; i < 64; i++)

if layer i in TargetDecLayerIdList

TotalPicSizeInSamplesY += PicSizeInSamplesY of the layer with nuh_layer_id equal to i

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( TotalPicSizeInSamplesY  (2 * MaxLumaSr), fR ) for the value of TotalPicSizeInSamplesY of access unit n − 1, where MaxLumaSr is the value specified in Error: Reference source not found that applies to access unit n − 1.

- For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to 2 * CpbBrVclFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrVclFact	or bits/s.

- For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to 2 * CpbBrNalFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrNalFactor bits/s.

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).