| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0137 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0137 |
| --- | --- |

| Title: | REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process and profile-level-tier combinations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes the following three aspects. Aspect 3 is proposed only if aspect 1 is adopted.

- decoding_process_idc is included in the VPS for each layer. It specifies the decoding process (version 1, REXT, MV-HEVC, SHVC, 3D-HEVC) to be used for the layer and the constraints on sps_extension_type_flag[ i ] values for the layer.

- The use of the profile_tier_level( ) syntax structure for layers sets excluding the base layer is clarified as follows:

  - The independent layer with the smallest nuh_layer_id among independent layers in the layer set is considered to be the base layer in the decoding process except for the slice segment header decoding.

  - When the layer set does not contain layers with AuxId equal to 0, the profile_tier_level( ) syntax structure applies to a CVS in which AuxId for all the layers is considered to be equal to 0.

- The depth auxiliary picture type (AUX_DEPTH value of AuxId) is removed from MV-HEVC and the DepthFlag scalability dimension is used instead (scalability mask index equal to 0) with decoding_process_idc indicating either the version 1 or MV-HEVC decoding process for depth views.

It is asserted that aspects 1 and 2 provide the following functionality:

- Capability to indicate e.g. in session negotiation, which profile-level-tier combination and decoding process are used for independent layers and auxiliary picture layers, particularly differentiating between version 1 and REXT decoding processes.

- Capability to indicate which decoding process is used for layers that are not included in any output layer sets, e.g. when the total number of views in the bitstream exceeds profile limits.

# Introduction

## Problem Statement

It is asserted that it would be essential to be able to provide the functionality of indicating which decoding process (v1, REXT, MV-HEVC, SHVC, 3D-HEVC) is used for each layer for the following functionality:

- It should be known in session negotiation

  - which profile-tier-level combination applies to a set of auxiliary picture layers; and/or

  - which decoding process is used for particular auxiliary picture layers

This information lets the receiver to choose whether to receive a particular auxiliary picture layer or which one of the auxiliary picture layers offered as alternatives to choose. For example, a decoder may be able to process auxiliary picture layers decodable with version 1 decoding process only and hence desires not to receive e.g. REXT-coded auxiliary picture layers.

- In case of simulcast layers, it should be known in session negotiation which decoding process is used for each independent layer (to let the decoder to decide whether to receive or choose from layers provided as alternatives similarly to above for auxiliary picture layers).

- If the entire bitstream conforms to no profile and some layers fall outside of any specified output layer sets, decoders may still want to know which decoding process is used for those layers. For example a bitstream may include a greater number of views than allowed in any profile.

## Shortcomings in the Existing Indications

It is asserted that none of the existing indications are sufficient for the functionality due to the following reasons:

- A combination of profile, tier and level is associated with an output layer set, which has the following consequences:

  - Auxiliary picture layers might not be included in output layer sets, and hence the profile-tier-level triplet is not sufficient for indicating the decoding process used for auxiliary picture layers.

  - A profile-tier-level triplet may allow multiple decoding processes used for different layers. For example, it is expected that the 3D-HEVC profile would allow non-base views coded either with MV-HEVC tools or 3D-HEVC tools. Hence, the profile-tier-level is not sufficient to conclude which decoding process is used for a particular layer when the profile allows more than one decoding process to be selected.

- Scalability dimension (or equivalently ScalabilityId) is not sufficient for the functionality desired in this contribution, because it is possible that more than one decoding process is associated with the same ScalabilityId. For example, both MV-HEVC and 3D-HEVC use ViewOrderIdx (and ViewId) but have a different decoding process.

- direct_dependency_type[ i ][ j ] is not sufficient for the functionality desired in this contribution due to the following reasons:

  - It indicates only the inter-layer processing taking place between layers, and is not sufficient to differentiate e.g. version 1 and REXT codec independent layers.

  - It is current not having sufficiently fine granularity. For example, motion prediction can be indicated for both MV-HEVC and SHVC, but only SHVC requires the use of motion field mapping.

- sps_extension_type_flag[ i ] for values of i in the range of 0 to 6, inclusive, include which SPS extension structures (0 to 6, respectively) are included in the SPS. sps_extension_type_flag[ i ] is not sufficient for the functionality desired in this contribution due to the following reasons:

  - Even if SPSs were included in session negotiation, they are not assigned to particular layers. Hence, SPSs are not sufficient to indicate which decoding process is used for a particular layer in the session negotiation (or more generally, prior to activation of the SPSs).

  - sps_extension_type_flag[ i ] does not indicate the decoding process used for a layer. For example, MV-HEVC and SHVC share the same sps_extension_type_flag.

# Overview of the Proposal

The contribution proposes the following aspects. Aspect 3 is proposed only if aspect 1 is adopted.

- decoding_process_idc is included in the VPS for each layer. It specifies the decoding process to be used for the layer and the constraints on sps_extension_type_flag[ i ] values for the layer.

| decoding_process_idc[ layerId ] | Decoding process used for the layer with nuh_layer_id equal to layerId | Constraints of sps_extension_type_flag[ j ] values |
| --- | --- | --- |
| 0 | Subclause 8.1 | sps_extension_type_flag[ j ] shall be equal to 0 for all values of j in the range of 0 to 7, inclusive |
| 1 | Subclause 8.1 | sps_extension_type_flag[ 0 ] shall be equal to 1 sps_extension_type_flag[ j ] shall be equal to 0 for all values of j in the range of 1 to 7, inclusive |
| 2 | Subclause G.8.1 | sps_extension_type_flag[ 1 ] shall be equal to 1 sps_extension_type_flag[ j ] shall be equal to 0 for all values of j equal to 0 or in the range of 2 to 7, inclusive |
| 3 | Subclause H.8.1 | sps_extension_type_flag[ 1 ] shall be equal to 1 sps_extension_type_flag[ j ] shall be equal to 0 for all values of j equal to 0 or in the range of 2 to 7, inclusive |
| 4-15 | Reserved | Reserved |

The detailed text for proposal 1 is marked with "Miska Hannuksela" change marks in the accompanying specification text.

- The use of the profile_tier_level( ) syntax structure for layers sets excluding the base layer is clarified as follows:

  - The independent layer with the smallest nuh_layer_id among independent layers in the layer set is considered to be the base layer in the decoding process except for the slice segment header decoding.

Motivation: Independent layers with nuh_layer_id may contain syntax elements that depend on nuh_layer_id being greater than 0 or refer to PPS, SPS and VPS containing information needed for the decoding of the independent layer.

  - When the layer set does not contain layers with AuxId equal to 0, the profile_tier_level( ) syntax structure applies to a CVS in which AuxId for all the layers is considered to be equal to 0.

Motivation: A layer set including only auxiliary picture layers enables to specify the profile-tier-level combination for the auxiliary picture layers as if they were primary picture layers.

The detailed text for proposal 2 is marked with "(profile)" change marks in the accompanying specification text.

- AUX_DEPTH AuxId is removed from MV-HEVC and the DepthFlag scalability dimension is used instead (scalability mask index equal to 0) with decoding_process_idc indicating either the version 1 or MV-HEVC decoding process for depth views.

Motivation: As decoding_process_idc is sufficient for differentiating whether MV-HEVC or 3D-HEVC decoding process is used for a depth layer, it is more logical to use the same depth signalling for both MV-HEVC and 3D-HEVC.

The detailed text for proposal 3 is marked with "(depth)" change marks in the accompanying specification text.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).