| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0138v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0138v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: HRD parameters for bitstreams excluding CL-RAS pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Cross-layer random access skip (CL-RAS) pictures need not be decoded and hence it is asserted that HRD parameters without CL-RAS pictures would be beneficial. It is proposed to indicate that HRD parameters for a bitstream without CL-RAS pictures and without RASL pictures associated with the first IRAP picture of each layer using a buffering period SEI message included in a scalable nesting SEI message that applies to a layer set. No new syntax is proposed in the contribution.

Version 2 of the contribution responds to the comments expressed in the JCT-VC meeting, on 9th January, 2014 and in off-line review by some experts. The changes can be summarized as follows: use_alt_cpb_params_flag is included in the buffering period SEI message and is gated by "if payload_extension_present( )". The variable UseAltCpbParamsFlag is derived from use_alt_cpb_params_flag or from external means. When UseAltCpbParamsFlag is equal to 1, the alternative HRD parameters are chosen to be used from the buffering period SEI message.

# Introduction

Cross-layer random access skip (CL-RAS) pictures are pictures that cannot be correctly decoded when the decoding process starts from an IRAP access unit that does not contain IRAP pictures in all layers. CL-RAS pictures are not indicated in the bitstream but they are concluded during the decoding process: a CL-RAS picture is a picture with nuh_layer_id equal to layerId such that LayerInitializedFlag[ layerId ] is equal to 0.

Figure 1 shows an example how CL-RAS pictures are concluded when the decoding starts from AU x (in which case the CL-RAS pictures are the green pictures marked with "associated with AU x") or from AU z (in which case the CL-RAS pictures are the green pictures marked with "associated with "AU z"). When the decoding process starts from AU x or AU z, the respective CL-RAS pictures (the green pictures marked with "associated with AU x" or "AU z", respectively) can be removed from the bitstream, while the bitstream remains conforming.

Figure 1: Association of CL-RAS pictures with IRAP access units. AU y does not have any association CL-RAS pictures in this bitstream (copied from JCTVC-O0212/JCT3V-F0072).

Among other things, contribution JCTVC-O0212/JCT3V-F0072 proposed signalling of HRD parameters for bitstreams where CL-RAS pictures are not present. However, JCTVC-O0212/JCT3V-F0072 included a fairly large amount of additional syntax and semantics, which might have been one reason why the contribution was not accepted.

It is remarked that the indication of HRD parameters for bitstreams without CL-RAS pictures is the multi-layer equivalent of the indication of HRD parameters for bitstreams without RASL pictures for single-layer bitstreams. The HRD parameters for bitstreams without RASL pictures are indicated in the buffering period SEI message with the cpb_delay_offset, dpb_delay_offset, nal_initial_alt_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_offset[ i ], vcl_initial_alt_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_offset[ i ] syntax elements.

It is believed that it can be common to have more frequent IRAP picture interval in the base layer than in the enhancement layer for example to enable quick tune-in for broadcasts while obtaining better RD performance for the enhancement layer. Consequently, CL-RAS pictures may commonly occur and the number of CL-RAS pictures may be even greater than the number of RASL pictures. It is therefore believed that the signalling of HRD parameters for bitstreams without CL-RAS and RASL pictures is an important feature.

# Proposal

In this contribution, it is proposed to indicate that HRD parameters for a bitstream without CL-RAS pictures and without RASL pictures associated with the first IRAP picture of each layer using a buffering period SEI message included in a scalable nesting SEI message that applies to a layer set. The syntax elements cpb_delay_offset, dpb_delay_offset, nal_initial_alt_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_offset[ i ], vcl_initial_alt_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_offset[ i ] are interpreted for a bitstream that excludes both the CL-RAS picture and the RASL pictures (associated with the initial IRAP pictures of each layer). No new syntax is proposed in the contribution.

The detailed proposal is included as change marks in the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).