| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0140 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0140 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On non-HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution discusses two aligned designs for enabling non-HEVC-coded base layer:

- The decoded non-HEVC base layer pictures are provided by external means and their DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) are either provided by external means or included in the HEVC bitstream using a specific NAL unit. This design is the same as in JCTVC-O0166/JCT3V-F0060.

- The decoded non-HEVC base layer pictures are provided by externals means or by including non-HEVC NAL units within specific HEVC NAL units. Similarly to the first option the DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) of non-HEVC pictures are either provided by external means (when the pictures themselves are provided by external) or included in the HEVC NAL units together with the nested non-HEVC NAL units.

As the changes are asserted to be substantial and may require verification by both expert review and software implementation, the contribution has been submitted for discussion. The contribution follows up JCTVC-O0166/JCT3V-F0060.

# Introduction

It is asserted that in the case of hybrid codec scalability (the base layer being non-HEVC-coded):

- The decoded pictures of the non-HEVC-coded base layer have to be provided as reference for inter-layer prediction to the HEVC-extension (SHVC or MV-HEVC) decoder immediately after their decoding without considering the DPB delays specified for the base layer (e.g. in the AVC bitstream). Otherwise, the delay of the decoding system could be unreasonably high.

- Each decoded non-HEVC-coded base-layer picture has to be considered in the MV-HEVC/SHVC DPB similarly to a decoded HEVC-coded base-layer picture, as the output of decoded pictures is synchronized across layers (i.e. all pictures of the same access unit are output at the same time). It seems unreasonable to attempt to constrain the output times of non-HEVC-coded base-layer pictures (according to the base-layer specification, such as AVC) to be identical to the output times of the respective enhancement layer pictures. Hence, conceptually two separate copies of each decoded base-layer picture appear in the non-HEVC base-layer decoder/HRD and in the HEVC-extension decoder/HRD.

It is asserted that in order for the DPB of SHVC/MV-HEVC to operate correctly in the case of hybrid codec scalability, the base layer pictures shall be at least conceptually included in the DPB operation of the scalable bitstream and be assigned the following properties:

- NoOutputOfPriorPicsFlag

- PicOutputFlag

- PicOrderCntVal

- Reference picture set

It is asserted that these properties are needed for the base layer because the DPB for the base layer cannot operate independently of the other layers due to the following reasons:

- The base layer has output characteristics that may be related to the other layers, e.g.:

  - In some operation points, the base layer may be among the output layers, in some the base layer might not be among output layers.

  - The output of a base layer picture in a certain access unit should be synchronized with the output of the pictures of other layers in the same access.

  - The base layer pictures may be assigned or it may be desirable to assign the base layer information that is specific to the output operation, such as no_output_of_prior_pics_flag and pic_output_flag.

- The marking of base layer pictures as "unused for reference" shall not precede using the base layer picture as an inter-layer reference picture. (In H.264/AVC, a picture can be marked as "unused for reference" immediately after its decoding.)

- The marking of base layer pictures as "used for reference" and "unused for reference" affects the emptying of picture storage buffers and hence the total memory use of the DPB.

# Proposal

## Overview

The contribution proposes two aligned designs for discussion:

- The decoded non-HEVC base layer pictures are provided by external means and their DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) are either provided by external means or included in the HEVC bitstream using a specific NAL unit. This design is the same as in JCTVC-O0166/JCT3V-F0060.

- The decoded non-HEVC base layer pictures are provided by externals means or by including non-HEVC NAL units within specific HEVC NAL units. Similarly to the first option the DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) of non-HEVC pictures are either provided by external means (when the pictures themselves are provided by external) or included in the HEVC NAL units together with the nested non-HEVC NAL units.

It is noted that for the profile-tier-level signalling, the VPS is used as with conventional SHVC bitstreams. The profile-tier-level is therefore provided for each output layer set and includes consideration of the base layer.

## Option 1

Option 1 includes the following aspects:

- Removing avc_base_layer_flag from the VPS extension and including instead:

  - non_hevc_base_layer_flag specifying whether the base layer is HEVC-coded

  - base_layer_prop_present_in_bitstream_flag specifying whether the DPB related properties of non-HEVC-coded base layer are present in the bitstream (and encapsulated in specific NAL units as outlined below) or whether these properties are provided by external means

- A new NAL unit types, NON_HEVC_VCL, containing the DPB related properties (a part of the slice segment header as outlined below) of a non-HEVC-coded base-layer picture.

- The slice segment header is split into two parts, the generic part applicable to HEVC-coded and non-HEVC-coded slices and the HEVC-specific part. The generic part includes:

  - Slice address: first_slice_in_pic_flag and slice_segment_address

  - Output related syntax elements: no_output_of_prior_pics_flag, pic_output_flag

  - Generic syntax elements: slice_pic_parameter_set_id, dependent_slice_segment_flag, extra_slice_header_bits, slice_type, colour_plane_id

  - Picture order count related syntax elements

  - Reference picture set related syntax elements

- When non_hevc_base_layer_flag is equal to 1, the decoded pictures are provided by external means and the decoded picture is included in the DPB.

- If base_layer_prop_present_in_bitstream is equal to 0, syntax element and variable values (nal_unit_type, TemporalId, NoOuputOfPriorPicsFlag, PicOutputFlag, reference picture set) associated with the non-HEVC-coded base layer picture are provided by external means. Otherwise, the NON_HEVC_VCL NAL unit is decoded to obtain these syntax elements and variables.

The detailed proposal is provided with the accompanying document including the change-marked specification text ("JCTVC-P0140_JCT3V-G0140 option1 spectext.doc").

Note: as the purpose of the contribution is discussion, the specification text has not been updated to be on top of the latest specification text.

## Option 2

In the design of option 2, the following properties were considered desirable to be assigned for the base layer pictures:

- The 2-byte NAL unit header of HEVC NAL units, including nuh_layer_id and nuh_temporal_id_plus1, which can be used for bitstream extraction and adaptation.

- NAL unit types, which enable to indicate an IRAP picture of the base layer (versus a non-IRAP picture) and sub-layer non-reference pictures, which is asserted the make the bitstream adaptation performed in media-aware network elements (MANEs) more straightforward.

- first_slice_segment_in_pic_flag, which enables detection of the start of an access unit (when the NAL unit is not preceded by other NAL units that can start an AU, such as the AU delimiter NAL unit) or the start of a coded base layer picture.

Option 2 includes the following aspects:

- Removing avc_base_layer_flag from the VPS extension and including instead:

  - non_hevc_base_layer_flag specifying whether the base layer is HEVC-coded

  - base_layer_present_in_bitstream_flag specifying whether the non-HEVC-coded base layer is present in the bitstream (and encapsulated in specific NAL units as outlined below) or whether decoded base layer pictures are provided by external means

  - base_layer_format_idc specifying which specification the non-HEVC base layer present in the bitstream conforms to

- Four new NAL unit types:

  - NON_HEVC_IRAP for containing non-HEVC-coded IRAP pictures

  - NON_HEVC_N for containing non-HEVC-coded non-IRAP pictures that are sub-layer non-reference pictures

  - NON_HEVC_R for containing non-HEVC-coded non-IRAP pictures that are not sub-layer non-reference pictures

  - NON_HEVC_NVCL for containing non-HEVC non-VCL NAL units

It should be discussed whether one NON_HEVC_NVCL type is sufficient or whether two non-VCL types are needed, one from the value range that can start an access unit and another from the value range that can be in the "tail" of an access unit.

- The slice segment header is split into two parts, the generic part applicable to HEVC-coded and non-HEVC-coded slices and the HEVC-specific part. The generic part includes:

  - Slice address: first_slice_in_pic_flag and slice_segment_address

  - Output related syntax elements: no_output_of_prior_pics_flag, pic_output_flag

  - Generic syntax elements: slice_pic_parameter_set_id, dependent_slice_segment_flag, extra_slice_header_bits, slice_type, colour_plane_id

  - Picture order count related syntax elements

  - Reference picture set related syntax elements

- For non-HEVC-coded slices, the slice segment layer RBSP contains the slice segment header (as outlined above) and the slice as specified in the external specification. In the case of AVC base layer, the entire AVC-formatted coded slice NAL unit is included in the slice segment layer RBSP.

- When non_hevc_base_layer_flag is equal to 1 and base_layer_present_in_bitstream is equal to 0, the decoded pictures and associated syntax element and variable values (nal_unit_type, TemporalId, NoOuputOfPriorPicsFlag, PicOutputFlag, reference picture set) are provided by external means and the decoded picture is included in the DPB.

- When non_hevc_base_layer_flag is equal to 1 and base_layer_present_in_bitstream is equal to 1, the encapsulated base layer pictures are decoded according to the specification they conform to and the decoded pictures are included in the DPB. The associated syntax element and variable values are decoded from the generic part of the slice segment header.

- Each non-HEVC base layer format included in the same bitstream shall have its own mapping how it is encapsulated in the HEVC NAL units. For AVC-coded base layer, the mapping is provided in the proposed Annex I, which also specifies a sub-bitstream extraction process for generating a NAL unit stream that conforms to AVC.

The detailed proposal is provided with the accompanying document including the change-marked specification text ("JCTVC-P0140_JCT3V-G0140 option2 spectext.doc").

Note: as the purpose of the contribution is discussion, the specification text has not been updated to be on top of the latest specification text.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).