| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0145 |
| --- | --- |

| Title: | RCE3: Subtest B.3 - Intra block copy with NxN PU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of RCE3 Subtest B.3 on intra block copy with N×N PU. In the proposed technique, the intra block copy is extended to N×N PU for the smallest CU, so it is aligned with the other intra modes. The performance is reported to be an average BD-rate of -9.4%, -9.1%, -11.5%, -16.0%, respectively for RGB SC, YUV SC, optional RGB SC, optional YUV SC sequences in AI Super-High-Tier configurations.

# Technical Description

In the proposed technique, Intra Block Copy (Intra BC) is also applied to N×N PU for the smallest CU. Following current split modes for intra, each PU has a block vector (BV). For each PU, the prior PUs with in a CU cannot be used as the reference. Also the chroma blocks within a 4x4 block will share a BV instead which is derived from the one/multiple BVs.

# Results

In this section, the experimental results of the proposed method are presented, and BD-rates are provided for the lossy configuration (Table 1) and rate savings for the lossless configuration (Table 2) . The method is tested on the top of HM12.1_Rext5.1. AHG8 test conditions are used.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.8% | -2.5% | -2.5% | -2.6% | -2.4% | -2.4% | -2.3% | -2.1% | -2.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -8.6% | -8.5% | -8.5% | -9.2% | -9.2% | -9.1% | -9.5% | -9.4% | -9.4% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | -8.2% | -8.1% | -7.8% | -8.9% | -8.8% | -8.6% | -9.3% | -9.1% | -9.1% |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -10.5% | -10.1% | -10.1% | -11.5% | -11.0% | -11.2% | -12.0% | -11.5% | -11.7% |
| YCbCr 4:4:4 SC (Optional) | -12.6% | -12.3% | -12.3% | -14.8% | -14.3% | -14.3% | -16.7% | -16.0% | -16.3% |
| Enc Time[%] | 136% |  |  | 136% |  |  | 134% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.0% | -1.8% | -1.7% | -1.9% | -1.7% | -1.8% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -7.4% | -7.0% | -7.1% | -7.9% | -7.6% | -7.6% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -7.0% | -6.8% | -6.8% | -7.7% | -7.6% | -7.5% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -8.4% | -7.9% | -8.0% | -9.7% | -9.1% | -9.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -11.1% | -11.0% | -10.9% | -13.2% | -12.8% | -12.8% |  |  |  |
| Enc Time[%] | 93% |  |  | 95% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.9% | -1.0% | -1.1% | -0.8% | -1.0% |  |  |  |
| Class B | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -5.2% | -4.8% | -5.0% | -5.9% | -5.5% | -5.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC | -4.6% | -4.4% | -4.6% | -5.5% | -5.1% | -5.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -6.8% | -6.3% | -6.5% | -7.7% | -7.0% | -7.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -9.9% | -9.6% | -9.7% | -11.9% | -11.7% | -11.6% |  |  |  |
| Enc Time[%] | 95% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 2 Lossless results

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | -1.0% | -0.6% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -6.6% | -5.5% | -5.2% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -6.8% | -5.4% | -5.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | -10.2% | -8.6% | -8.2% |
| SC YUV 444 Optional | -13.8% | -12.8% | -13.3% |
| Enc Time[%] | 134% | 106% | 102% |
| Dec Time[%] | 91% | 101% | 103% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).