| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0149 |
| --- | --- |

| Title: | Non-RCE3: Block vector signaling for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document, two block vector signaling methods are proposed. The first method changes both the vector predictor and the coding of block vector difference, while the second one only changes the coding of block vector difference. Reported average BD-rates are -0.8% and -0.6% for RGB and YUV SC sequences in AI Main-Tier configurations, respectively. Results combining the methods with the N×N intra block copy method in RCE3 (JCTVC-P0145) are also reported. The average BD-rates are -1.3% and -1.6% for RGB and YUV SC sequences in AI Main-Tier configurations, respectively.

# Introduction

At the 15th JCT-VC meeting, it was adopted that the last decoded block vector (BV) is used as vector predictor for BV signaling in Intra Block Copy (Intra BC) [1]. The block vector difference (BVD) is coded by sharing the motion vector difference (MVD) coding method in HEVC.

In this document, two BV signaling methods are proposed to further improve the coding efficiency. The first method changes both the vector predictor and the coding of BVD, while the second one only changes the BVD coding.

# Proposed methods

## Method 1

In method 1, the left or above BV is selected as predictor. If one of them is unavailable, (-2*CUwidth, 0) is used instead. If both of them are unavailable, (-2*CUWidth, 0) and (-CUWidth, 0) are used instead.

The resulting BVD is coded as follows: first, one flag indicates whether the difference is zero. When BVD is not zero, exponential-Golomb codes with order 3 are used to encode the remaining absolute level of the BVD and a flag is used to code its sign.

In order to avoid extra line buffer to store the above BV, only the above BV inside current CTU is allowed to be used as the predictor.

## Method 2

In method 2, the vector predictor is not changed, but the resulting BVD is coded as in Method 1.

# Results

In this section, experimental results are presented for the proposed BV signaling methods. The methods are implemented in HM12.1_RExt5.1. AHG8 test conditions are be used. The performance is compared to the anchor for lossy and lossless configuration in terms of BD-rates and rate savings, respectively. Additional results of the proposed methods are shown when Intra BC is extended to N×N PU for the smallest CU as in RCE3 Subtest B. 3 [2].

- Results of method 1 for lossy (Table 1) and lossless (Table 2);

- Results of method 2 for lossy (Table 3) and lossless (Table 4);

- Results of method 1 for lossy (Table 5) and lossless (Table 6) with N×N PU Intra BC;

- Results of method 2 for lossy (Table 7) and lossless (Table 8) with N×N PU Intra BC;

Table 1 Lossy results of method 1

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.5% | -0.6% | -0.4% | -0.4% | -0.4% | -0.2% | -0.2% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.6% | -0.6% | -0.7% | -0.4% | -0.5% | -0.5% | -0.3% | -0.4% | -0.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.8% | -0.9% | -0.8% | -0.6% | -0.7% | -0.6% | -0.5% | -0.5% | -0.5% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.7% | -0.7% | -0.7% | -0.7% | -0.7% | -0.7% | -0.4% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC (Optional) | -1.3% | -1.3% | -1.3% | -1.1% | -1.1% | -1.2% | -1.0% | -0.9% | -1.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% | -0.4% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.7% | -0.7% | -0.7% | -0.8% | -0.7% | -0.8% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.1% | -1.3% | -1.0% | -1.0% | -1.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.3% | 0.1% | -0.1% | -0.3% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.7% | -0.6% | -0.4% | -0.5% | -0.4% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.6% | -1.7% | -1.7% | -1.2% | -1.2% | -1.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.8% | -1.6% | -1.5% | -1.3% | -1.1% | -1.1% |  |  |  |
| Enc Time[%] | 97% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

Table 2 Lossless results of method 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | -0.1% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.1% | 0.0% | -0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 1.8% | 1.6% | 0.7% |
| SC YUV 444 Optional | -0.5% | -0.3% | -0.7% |
| Enc Time[%] | 100% | 98% | 98% |
| Dec Time[%] | 101% | 101% | 101% |

Table 3 Lossy results of method 2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.5% | -0.3% | -0.3% | -0.4% | -0.3% | -0.2% | -0.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.6% | -0.6% | -0.5% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% |
| YCbCr 4:4:4 SC (Optional) | -0.8% | -0.7% | -0.8% | -0.7% | -0.6% | -0.7% | -0.6% | -0.6% | -0.6% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.6% | -0.7% | -0.7% | -0.6% | -0.6% | -0.6% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.8% | -0.1% | -0.1% | -0.4% | -0.2% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -0.3% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.5% | -0.3% | -0.5% | -0.3% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.6% | -0.5% | -0.6% | -0.6% | -0.5% | -0.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.9% | -0.8% | -0.6% | -0.8% | -1.0% | -0.8% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |

Table 4 Lossless results of method 2

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | 0.1% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.1% | 0.1% | -0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | -0.3% | -0.3% | -0.4% |
| SC YUV 444 Optional | -0.4% | -0.4% | -0.5% |
| Enc Time[%] | 100% | 99% | 99% |
| Dec Time[%] | 93% | 99% | 107% |

Table 5 Lossy results of method 1 with N×N PU Intra BC

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -0.9% | -0.8% | -0.7% | -0.7% | -0.7% | -0.6% | -0.6% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.3% | -1.3% | -1.5% | -1.1% | -1.2% | -1.2% | -0.9% | -1.0% | -0.9% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -1.6% | -1.8% | -1.7% | -1.4% | -1.4% | -1.5% | -1.1% | -1.1% | -1.1% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.6% | -1.6% | -1.7% | -1.7% | -1.7% | -1.7% | -1.3% | -1.0% | -0.8% |
| YCbCr 4:4:4 SC (Optional) | -1.8% | -1.8% | -1.9% | -1.6% | -1.7% | -1.7% | -1.5% | -1.4% | -1.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.8% | -0.7% | -0.6% | -0.7% | -0.7% |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -1.0% | -1.0% | -1.1% | -0.9% | -1.0% | -1.0% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.2% | -1.1% | -1.2% | -1.0% | -1.0% | -1.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% | -1.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.7% | -1.7% | -1.6% | -1.5% | -1.5% | -1.4% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -1.0% | -0.6% | -0.6% | -0.6% | -0.4% |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.8% | -0.9% | -0.9% | -0.7% | -0.8% | -0.8% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.2% | -1.1% | -1.1% | -1.1% | -1.0% | -1.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.4% | -1.3% | -1.3% | -1.2% | -1.2% | -1.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.8% | -1.8% | -1.8% | -2.3% | -2.3% | -2.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 6 Lossless results of method 1 with N×N PU Intra BC

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | 0.2% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.2% | -0.1% | -0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 2.2% | 1.9% | 0.8% |
| SC YUV 444 Optional | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 99% | 100% | 101% |
| Dec Time[%] | 101% | 101% | 98% |

Table 7 Lossy results of method 2 with N×N PU Intra BC

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.5% | -0.4% | -0.4% | -0.5% | -0.3% | -0.4% | -0.4% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.6% | -0.6% | -0.6% | -0.5% | -0.6% | -0.6% | -0.5% | -0.5% | -0.5% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.7% | -0.9% | -0.7% | -0.7% | -0.8% | -0.7% | -0.6% | -0.6% | -0.6% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.5% | -0.5% | -0.6% | -0.7% | -0.6% | -0.7% | -0.7% | -0.6% |
| YCbCr 4:4:4 SC (Optional) | -0.8% | -0.8% | -0.7% | -0.9% | -1.0% | -0.9% | -1.0% | -1.0% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.5% | -0.6% | -0.7% | -0.6% | -0.6% | -0.7% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.4% | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.7% | -0.8% | -0.7% | -0.7% | -0.8% | -0.6% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% | -0.4% |  |  |  |
| Class B | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC | -0.8% | -0.7% | -0.6% | -0.7% | -0.6% | -0.6% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.7% | -0.7% | -0.5% | -0.9% | -0.9% | -0.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.7% | -1.6% | -1.5% | -1.5% | -1.6% | -1.4% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

Table 8 Lossless results of method 2 with N×N PU Intra BC

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | -0.2% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.3% | -0.3% | -0.3% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.4% | -0.3% | -0.3% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | -0.6% | -0.5% | -0.8% |
| SC YUV 444 Optional | -0.9% | -0.9% | -1.0% |
| Enc Time[%] | 100% | 100% | 98% |
| Dec Time[%] | 95% | 99% | 106% |

# References

[1] G. Laroche, T. Poirier, and C. Gisquet, “AHG5: Motion prediction for intra block copy”, JCTVC-O0122, Geneva, CH, October 2013.

[2] C. Pang, J. Sole, L. Guo, and M. Karczewicz, “RCE3: Subtest B.3 – Intra block copy with NxN PU”, JCTVC-P0145, San José, US, January 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).