| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0150 |
| --- | --- |

| Title: | Non-RCE3: Block vector predictor initialization for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document, it is proposed to change the initial block vector predictor for intra block copy in each CTU. In the current design, the initial predictor is not an allowed value for the block vector. With this minor change, an average BD-rate of -0.1% for YUV SC sequences in AI configurations is obtained.

# Proposed method

At the 15th JCT-VC meeting, it was adopted that the last decoded block vector (BV) is used as vector predictor for BV signaling in Intra Block Copy (Intra BC) [1]. The BV difference is coded by sharing the motion vector difference (MVD) coding method in HEVC.

For the first BV in each CTU, zero BV, (0, 0), is used as predictor. The zero vector is not an allowed BV value.  It is proposed to use (-2*CUWidth, 0) as the vector predictor for the first BV in each CTU.

# Results

In this section, experimental results are presented for the proposed method. The method is implemented in HM12.1_RExt5.1. AHG8 test conditions are be used. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossy and lossless configuration in terms of BD-rates and rate savings, respectively.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |
| YCbCr 4:4:4 SC (Optional) | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.3% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 97% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.2% | 0.6% | 0.0% | -0.2% | 0.0% |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.1% | -0.2% | -0.2% | 0.3% | 0.3% | 0.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.1% | 0.1% | 0.2% | 0.1% | -0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |

Table 2 Lossless results

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 0.2% | 0.1% | 0.2% |
| SC YUV 444 Optional | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% | 94% | 95% |
| Dec Time[%] | 96% | 102% | 106% |

# References

[1] G. Laroche, T. Poirier, and C. Gisquet, “AHG5: Motion prediction for intra block copy”, JCTVC-O0122, Geneva, CH, October 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).