| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0151 |
| --- | --- |

| Title: | Non-RCE3: Fast encoder search for RCE3 Subtest B.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Intra Block Copy (Intra BC) was adopted as an intra coding tool in the HEVC Range Extensions draft specification. In RCE3 Subtest B.3, Intra BC is extended to N×N PU for the smallest CU. To reduce complexity from encoder search of this extension, a fast search method is proposed in this document. With the proposed method, the reported average BD-rate is -8.2% for YUV SC sequences in AI Super-High-Tier configurations with 10% encoder time increase.

# Introduction

At the 14th JCT-VC meeting, Intra Block Copy (Intra BC) [1] was adopted as a new intra coding tool in HEVC Range Extensions draft specification. Intra BC is an efficient technique to predict repetitive patterns in the frame and leads to large bit-rate savings for screen content.

In RCE3 Subtest B.3 (JCTVC-P0145), Intra BC is extended to N×N PU for the smallest CU [2]. To reduce encoder complexity involved in this extension, a fast search method is presented in this proposal

# Proposed Fast Encoder Search Method

A fast search method is described to accelerate the encoder search for RCE3 Subtest B.3 as follows:

- During the block vector (BV) search for the CU of size 8×8, the SADs of the 4 4×4 blocks are reused for the BV search for N×N PU;

- The Intra BC N×N mode is not checked if the one of following criteria is met:

- intra_cost < max (48, 32×Lambda)

- bUse1DSearchFor8x8 = true

- The BVs of the 4 4x4 blocks obtained from the BV search for the CU of size 8×8 are the same;

# Results

In this section, experimental results are presented for the proposed method on the top of HM12.1_RExt5.1. AHG8 test conditions are be used. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossy and lossless configuration in terms of BD-rates and rate savings, respectively.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.8% | -2.7% | -2.7% | -2.8% | -2.7% | -2.7% | -2.7% | -2.6% | -2.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -7.3% | -7.2% | -7.2% | -8.0% | -7.8% | -7.8% | -8.3% | -8.1% | -8.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -6.8% | -6.7% | -6.5% | -7.7% | -7.6% | -7.3% | -8.1% | -7.9% | -7.8% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -8.4% | -8.0% | -8.0% | -9.4% | -9.0% | -9.0% | -10.2% | -9.7% | -9.7% |
| YCbCr 4:4:4 SC (Optional) | -9.7% | -9.6% | -9.5% | -11.9% | -11.6% | -11.5% | -13.9% | -13.3% | -13.5% |
| Enc Time[%] | 109% |  |  | 108% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.0% | -2.2% | -2.1% | -2.0% | -2.1% | -2.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -6.3% | -6.0% | -6.1% | -6.7% | -6.4% | -6.4% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -5.7% | -5.6% | -5.7% | -6.5% | -6.3% | -6.4% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -6.5% | -6.1% | -6.2% | -7.6% | -7.0% | -7.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -8.4% | -8.2% | -8.4% | -10.2% | -9.9% | -10.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.9% | -1.4% | -0.8% | -1.1% | -1.1% | -1.1% |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -4.2% | -4.0% | -4.0% | -5.0% | -4.6% | -4.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -3.7% | -3.6% | -3.6% | -4.5% | -4.2% | -4.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -5.1% | -4.8% | -5.1% | -6.7% | -6.0% | -6.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -7.0% | -6.7% | -6.7% | -9.0% | -8.8% | -8.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 2 Lossless results of method 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | -1.2% | -0.6% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -5.3% | -4.4% | -4.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -5.6% | -4.5% | -4.3% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | -8.6% | -7.2% | -7.2% |
| SC YUV 444 Optional | -10.9% | -10.0% | -11.1% |
| Enc Time[%] | 115% | 102% | 103% |
| Dec Time[%] | 99% | 99% | 99% |

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

[2] C. Pang, J. Sole, L. Guo, and M. Karczewicz, “RCE3: Subtest B.3 – Intra block copy with NxN PU”, JCTVC-P0145, San José, US, January 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).