| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0152_r2 |
| --- | --- |

| Title: | Non-RCE4: Major color table (palette) merge from above and left CU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai1, Shan Liu1, Tzu-Der Chuang2, Yu-Wen Huang2, and Shawmin Lei1 1 2860 Junction Ave, San Jose, CA 95134, USA 2 No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | polin.lai@mediatek.com shan.liu@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents major color tables (or, palettes) merge methods, in which the major color tables (or, palettes) of the current CU are copied entirely from the major color tables (or, palettes) of its above or left CUs. When the major color tables are shared using the proposed merge methods, syntax corresponding to the current CU’s major color tables is omitted. Implementation example using RCE4 Test1 as anchor demonstrates 2.2%, 1.6%, and 1.0% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. Furthermore, when the proposed method is combined with the “major color table propagation” in JCTVC-P0096 using RCE4 Test1 as anchor, the corresponding BD-rate savings are 4.6%, 3.7%, and 2.4%. The proposed method can also be applied to RCE4 Test2. This document provides results on RCE4 Test1 just as an implementation example.

# Introduction

In the 15th JCT-VC meeting, two major color coding methods (or, palette coding methods) were proposed and are studied in the 16th JCT-VC meeting as RCE4 Test1 [1] and Test2 [2]. In both methods, each major color element in the major color tables (or, palettes) of the current CU can be predicted from the corresponding major color element in the major color tables (or, palettes) of its above or left CUs. Such element-wise copy scheme requires element-wise flags to indicate the copy operations.

In this document, methods of CU-wise major color tables merge are proposed, in which the entire major color tables of the current CU are copied directly from the major color tables of its above and left CUs. When the proposed merge methods are used for a given CU, syntax corresponding to its major color tables is omitted. For CUs with major color tables not being entirely copied from its neighboring CUs, tests were conducted with and without the element-wise copy scheme in [1] and [2] to evaluate tradeoff.

# Proposed method

For a CU coded with major color mode (or, palette mode), one flag major_color_merge is used to indicate whether its entire major color tables are copied, if so, another flag major_color_merge_dir is used to indicate it is copied from the major color tables in its above CU or its left CU. Since the entire major color tables are copied from previously coded ones, the syntax corresponding to the major color tables is omitted for CUs with major_color_merge = 1.

Note that for line-buffer consideration, in this document, if the above CU is from another CTU, “major color table merge above” is not allowed (considered as above CU not available).

For CUs that are coded with major color mode but with major_color_merge = 0, two methods were proposed as described below.

## Method 1: For non-merge-table, allow element-wise copy

In this first method, the aforementioned major color merge is directly added on top of the current RCE4 methods. That is, for major-color-mode CUs with major_color_merge = 0, the element-wise copy is used. The following pseudo-code summarizes the decoding process.

For a major-color-mode CU:

Parse major_color_merge

If (major_color_merge)

Parse major_color_merge_dir

If (major_color_merge_dir)

Copy above CU’s entire major color tables to the current CU

Else

Copy left CU’s entire major color tables to the current CU

Else

Decode each major color element for the current CU:

Can be copied from the corresponding element in above table, in left table (using flags), or new value

## Method 2: For non-merge-table, simply code all color values

In this second method, the element-wise copy is removed. That is, for major-color-mode CUs with major_color_merge = 0, all the major colors elements in the major color tables are directly encoded into the bitstream and have to be decoded. Thus in this method, there are only two scenarios for a CU coded with major color mode: Either its major color tables are entirely copied from the above or left CU, or all major colors are directly encoded/decoded. The following pseudo-code summarizes the decoding process.

For a major-color-mode CU:

Parse major_color_merge

If (major_color_merge)

Parse major_color_merge_dir

If (major_color_merge_dir)

Copy above CU’s entire major color tables to the current CU

Else

Copy left CU’s entire major color tables to the current CU

Else

Parse each major color element value for the current CU

# Simulation results

As an implementation example, the proposed methods were tested using RCE4 Test1 [1] as anchor. The encoder will evaluate three cases of major color mode coding: Copy the above major color tables, copy the left major color tables, use its own new tables. All the results were simulated according to the common test conditions defined in JCTVC-O1124 [3]. Please ignore the runtime comparison as the anchor and tests were obtained from different Linux cluster environments.

## Method 1: For non-merge-table, allow element-wise copy

Lossy and lossless coding results of the proposed method using RCE4 Test1 as the anchor are shown in Table 1, where 2.2%, 1.6%, and 1.0% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

## Method 2: For non-merge-table, simply code all color values

Lossy and lossless coding results of the proposed method using RCE4 Test1 as the anchor are shown in Table 2, where 2.2%, 1.7%, and 1.0% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

## Combination with the“major color table propagation” in JCTVC-P0096

In both RCE4 Test1 [1] and Test2 [2], if the above CU or left CU is not coded with major color mode, major color copy cannot be performed from the corresponding direction as there is no major color table for non-major-color-mode CU. In JCTVC-P0096 [4], a “major color table propagation” method is proposed in which for a non-major-color-mode CU, the major color tables of its left CU are used as its major color tables, and can be reference major color tables for following CUs to copy from.

Tests of combining major color merge and “major color table propagation” in JCTVC-P0096 [4] were also conducted. Lossy and lossless coding results of the proposed method using RCE4 Test2 as the anchor are shown in Table 3, where 4.6%, 3.7%, and 2.4% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Conclusions

This document presents major color tables (or, palettes) merge methods, in which the major color tables (or, palettes) of the current CU are copied entirely from the major color tables (or, palettes) of its above and left CUs. Results on RCE4 Test1 as an implementation example demonstrate 2.2%, 1.6%, and 1.0% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. Furthermore, when the proposed method is combined with the “major color table propagation” in JCTVC-P0096 using RCE4 Test1 as anchor, the corresponding BD-rate savings are 4.6%, 3.7%, and 2.4%.

# References

- “RCE4: Test1. Major-color-based screen content coding,” JCTVC-P0108, Jan. 2014.

- “RCE4: Results of Test 2 on Palette Mode for Screen Content Coding,” JCTVC-P0198, Jan. 2014.

- “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

- “Non-RCE4: Major color table propagation through non-palette CUs,” JCTVC-P0096, Jan. 2014.

Table 1. Major color table merge – Method 1
For non-merge-table: Allow element-wise copy

Anchor: RCE4 Test1 (P0108)

Lossy

Lossless

Table 2.  Major color table merge- Method 2
For non-merge-table: Simply code all color values

Anchor: RCE4 Test1 (P0108)

Lossy

Lossless

Table 3. Major color table merge – Method 1
+
Major color propagation JCTVC P0096

Anchor: RCE4 Test (P0108)

Lossy

Lossless

- Patent rights declaration(s)

MediaTek Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).