| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0153_r2 |
| --- | --- |

| Title: | Non-RCE4: Major color table (palette) sharing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai1, Shan Liu1, Tzu-Der Chuang2, Yu-Wen Huang2, and Shawmin Lei1 1 2860 Junction Ave, San Jose, CA 95134, USA 2 No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | polin.lai@mediatek.com shan.liu@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents major color tables (or, palettes) sharing methods, in which encoder and decoder store previously coded major color tables (or, palettes), and the current CU can either copy its entire major color tables from the stored ones, thus sharing, or use a its own new set of major color tables. When the major color tables are shared using the proposed sharing methods, syntax corresponding to the current CU’s major color tables is omitted. As an example, tests were conducted with only the most recently coded palettes from one previous CU stored for sharing purposes. Implementation example using RCE4 Test1 as anchor demonstrates 4.0%, 3.4%, and 2.2% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. The proposed method can also be applied to RCE4 Test2, and sharing using palettes from more than one previously coded CU can also be tested.

# Introduction

In the 15th JCT-VC meeting, two major color coding methods (or, palette coding methods) were proposed and are studied in the 16th JCT-VC meeting as RCE4 Test1 [1] and Test2 [2]. In both methods, each major color element in the major color tables (or, palettes) of the current CU can be predicted from the corresponding major color element in the major color tables (or, palettes) of its above or left CUs. Such element-wise copy scheme requires element-wise flags to indicate the copy operations. Furthermore, if above or left CU is not coded with major-color-mode, copy cannot be performed for the corresponding direction since there is no major color table for non-major-color-mode CUs.

To address these issues, in this document, methods of CU-wise major color tables sharing are proposed. To facilitate major color tables sharing, the most recently coded major color tables, for example from one previous CU, are stored. We denote this set as “last major color tables”. The current CU can either copy its entire major color tables from the stored “last major color tables” (i.e., sharing), or use a new set of major color tables. When the major color tables are entirely shared, syntax corresponding to the current CU’s major color tables is omitted. As for CUs with new major color tables, tests were conducted with and without the element-wise copy scheme as in RCE4 Test 1 [1] and Test 2 [2] to evaluate tradeoff.

# Proposed method

For a CU coded with major color mode (or, palette mode), one flag major_color_sharing is used to indicate whether its entire major color tables are copied from the stored “last major color tables”. Since the entire major color tables are copied from previously coded ones, the syntax corresponding to the major color tables is omitted for CUs with major_color_sharing = 1.

Note that as implementation example, in this document, only one set of the most recently coded major color tables, from one previous CU, is stored. Generic sharing methods, for example sharing using palettes from more than one previously coded CU can also be tested. Since there is only one stored set, simply the aforementioned major_color_sharing is sufficient for syntax signalling of the major color tables. There is no further syntax for identifying which stored sets (if there are more than one) to use for sharing. In comparison, RCE4 Test1 [1] and Test 2 [2], as well as “Major color table merge” in JCTVC P0152 [3], all require yet another flag to indicate the copy is from the above CU or the left CU. Furthermore, maintaining only one previously coded table set, as done in this document, imposes less memory requirements as compared to methods maintaining two sets of tables, one from the above CU and one from the left CU.

As for major-color-mode CUs with major_color_merge = 0, new major color tables will be used and the “last major color tables” have to be updated with this current CU’s new major color tables. As for singalling the new color tables, two methods were proposed as below.

## Method 1: For non-share-table, allow element-wise copy

In this first method, for major-color-mode CUs with major_color_merge = 0 (i.e., new major color tables), the element-wise copy as used in RCE4 Test 1 [1] and Test 2 [2] is utilized. The following pseudo-code summarizes process.

For a major-color-mode CU:

Parse major_color_sharing

If (major_color_sharing)

Copy entirely the stored “last major color tables” to the current CU

Else

Decode each major color element for the current CU:

Can be copied from the corresponding element in the “last major color tables”, or new value

Update the “last major color tables” with the current CU’s major color tables.

## Method 2: For non-share-table, simply code all color values

In this second method, for major-color-mode CUs with major_color_merge = 0 (i.e., new major color tables), all the major colors elements are directly encoded into the bitstream and have to be decoded. Thus in this method, there are only two scenarios for a CU coded with major color mode: Either its major color tables are entirely copied from the “last major color tables”, or all major color elements are directly encoded / decoded. The following pseudo-code summarizes the decoding process.

For a major-color-mode CU:

Parse major_color_merge

If (major_color_sharing)

Copy entirely the stored “last major color tables” to the current CU

Else

Parse each major color element value for the current CU

Update the “last major color tables” with the current CU’s major color tables.

# Simulation results

As an implementation example, the proposed methods were tested using RCE4 Test1 [1] as anchor. The encoder will evaluate two cases of major color mode coding: Sharing entirely with the stored “last major color tables”, or use its own new tables. All the results were simulated according to the common test conditions defined in JCTVC-O1124 [4]. Please ignore the runtime comparison as the anchor and tests were obtained from different Linux cluster environments.

## Method 1: For non-share-table, allow element-wise copy

Lossy and lossless coding results of the proposed method using RCE4 Test1 as the anchor are shown in Table 1, where 4.0%, 3.4%, and 2.2% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

## Method 2: For non-share-table, simply code all color values

Lossy and lossless coding results of the proposed method using RCE4 Test1 as the anchor are shown in Table 2, where 3.9%, 3.1%, and 2.2% BD-rate savings are shown for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Conclusions

This document presents major color tables (or, palettes) sharing methods. As a implementation example, only one set of “last major color tables” from one previous CU is stored, and can be shared with the current CU. Results on RCE4 Test1 as an implementation example demonstrate 4.0%, 3.4%, and 2.2% BD-rate savings for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# References

- “RCE4: Test1. Major-color-based screen content coding,” JCTVC-P0108, Jan. 2014.

- “RCE4: Results of Test 2 on Palette Mode for Screen Content Coding,” JCTVC-P0198, Jan. 2014.

- “Non-RCE4: Major color table (palette) merge from above and left CU,” JCTVC-P0152, Jan. 2014.

- “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

Table 1. Major color table sharing- Method 1
For non-share: Allowing element-wise copy

Anchor: RCE4 Test1

Lossy

Lossless

Table 2.  Major color table sharing- Method 2
For non-share: Simply code all color values

Anchor: RCE4 Test1

Lossy

Lossless

- Patent rights declaration(s)

MediaTek Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).