| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 16th Meeting: San Jose, US, 11 – 17 Jan. 2014 | Document: JCTVC-P0154 |
| --- | --- |

| Title: | AHG5 and AHG8: Alpha parameter coding methods for inter-component residual prediction in HEVC range extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Shawmin Lei2 1MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC-RExt [1], an alpha parameter is coded as a sign flag and an absolute value in the inter-component residual prediction (IRP) approach. This contribution proposes two methods to code the alpha parameter more efficiently. Method 1 is proposed to code the absolute value with one more context model. Method 2 additionally modifies the representation of alpha parameter for RGB content. It is reported that Method 1 achieves class-wise 0.1%~0.7% BD-rate gains on AHG8 test materials, and class-wise 0.1%~0.9% BD-rate gains on AHG5 RExt test materials. Furthermore, Method 2 achieves class-wise 0.1%~1.0% BD-rate gains on AHG8 test materials, and class-wise 0.1%~1.1% BD-rate gains on AHG5 RExt test materials.

# Introduction

In the current HEVC-RExt [1], alpha parameter for IRP [2] is signalled by syntax elements of res_scale_sign_flag and log2_res_scale_abs_plus1. The latter represents the absolute values of alpha and is binarized as a truncated unary code as shown in Table 1. log2_res_scale_abs_plus1 utilizes 3 context models in total. Two context models are used by the first two bins separately, while one context model is shared by the last two bins.

Table 1. Binarization for res_scale_abs_val and the corresponding alphas.

| Abosulte ResScaleVal | log2_res_scale_abs_plus1 [ c ] |
| --- | --- |
| 0 | 0(0) |
| 1 | 1(10) |
| 2 | 2(110) |
| 4 | 3(1110) |
| 8 | 4(1111) |

# Proposed method

It is observed that alpha being equal to 4 or 8 takes quite a large proportion, especially in RGB sequences. Since the current coding method does not match with such a distribution very well, two methods are proposed to code the alpha parameter more efficiently.

Method 1:

To code log2_res_scale_abs_plus1, four context models are used by the four bins separately.

Method 2:

As a further improvement, the representation and binarization method of alpha is revised. Alpha is represented by log2_res_scale_pred_4minus and log2_res_scale_neg as depicted in Table 2. The binarization for the two syntax elements and the corresponding alphas are demonstrated in Table. 3. Five context models are used by the five bins separately for log2_res_scale_pred_4minus. And log2_res_scale_neg is coded in the by-pass mode simply.

Method 1+Method2:

The proposed representation and binarization method is treated as an option. A flag in PPS extension is signalled to indicate whether the proposed or the original representation method is applied. In the encoder implementation, the new representation method is utilized for RGB videos and the original method is utilized for YUV videos.

Four context models are used by the four bins separately to code log2_res_scale_abs_plus1 when method 1 and method 2 are combined.

Table 2. Alpha parameter syntax in method 2

log2_res_scale_pred_4minus[ c ]

if( log2_res_scale_pred_plus1[ c ] != 5 )

log2_res_scale_neg[ c ]

Table 3. Binarization and corresponding alphas in method 2

| Alpha | log2_res_scale_pred_4minus[ c ] | log2_res_scale_neg [ c ] |
| --- | --- | --- |
| 0 | 0(0) | Na |
| 8 | 1(10) | Na |
| 4 | 2(110) | Na |
| 2 | 3(1110) | Na |
| 1 | 4(11110) | Na |
| -1 | 5(11111) | 0(00) |
| -2 | 5(11111) | 1(01) |
| -4 | 5(11111) | 2(10) |
| -8 | 5(11111) | 3(11) |

# Experimental Results

All the results were simulated according to the common test conditions defined in JCTVC-O1006 [3]. Runtime measurements are not reliable.

Method 1:

Detailed results are provided in the following pages. It is reported that Method 1 achieves class-wise 0.1%~0.7% BD-rate gains on AHG8 test materials, and class-wise 0.1%~0.9% BD-rate gains on AHG5 RExt test materials.

Method 1+Method2:

When Method 2 is applied on top of Method 1 for RGB content, improvements in coding efficiency are reported. Detailed results are provided in the following pages. It is reported that the combined method achieves class-wise 0.1%~1.0% BD-rate gains on AHG8 test materials, and class-wise 0.1%~1.1% BD-rate gains on AHG5 RExt test materials.

# Conclusions

In this document, methods are proposed to improve the alpha parameter coding for IRP. Experimental results demonstrate class-wise 0.1%~1.0% BD-rate gains on AHG8 test materials, and class-wise 0.1%~1.1% BD-rate gains on AHG5 RExt test materials.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 5,” JCTVC-O1005, Geneva, Switzerland, October 2013

- Wei Pu, Woo-Shik Kim, Jianle Chen, Joel Sole, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4,” JCTVC-O0154, Geneva, Switzerland, October 2013.

- “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, Geneva, Switzerland, October 2013.

AHG 8 Results of Method 1

AHG 5 Results of Method 1

AHG 8 Results of Method 1+Method 2

AHG 5 Results of Method 1+Method 2