| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0155 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0144 |
| --- | --- |

| Title: | On Sequence Parameter Set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document a modification is proposed to syntax and semantics of SPS. It is proposed that the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] be signalled in SPS only when nuh_layer_id is equal to 0. Corresponding change is also proposed to the semantics. It is asserted that this aligns the signalling of DPB parameters in SPS with the current sub-DPB operation based on VPS signalled DPB parameters for output layer sets.

# Introduction

In JCTVC-O1008 [1] and JCT3V-F1004 [2], if a CVS conforming to one or more of the profiles specified in Annex G or H is decoded by applying the decoding process specified in clauses 2−10, Annex F, and Annex G or H, the DPB parameters max_vps_num_reorder_pics[][], max_vps_latency_increase_plus1[][], and max_vps_dec_pic_buffering_minus1[][][] are used for the operation of DPB.

Only if a CVS conforming to one or more of the profiles specified in Annex A is decoded by applying the decoding process specified in clauses 2−10, the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] are used for the operation of DPB.

As a result in this document it is proposed that the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] be signalled in SPS only when nuh_layer_id is equal to 0.

Correspoding change to the semantics of sps_sub_layer_ordering_info_present_flag , sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] is proposed.

# Proposal

A modification is proposed to syntax and semantics of SPS. It is proposed that the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] be signalled in SPS only when nuh_layer_id is equal to 0. Corresponding change is also proposed to the semantics.

Proposed changes related to syntax, and semantics for the proposal are highlighted below compared to JCTVC-O1008 [1].

- F.7.3.2.2 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 ) { |  |
| update_rep_format_flag | u(1) |
| if( update_rep_format_flag ) |  |
| sps_rep_format_idx | u(8) |
| } else { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| if( nuh_layer_id = = 0 ) { |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| … |  |
| } |  |

sps_sub_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering_minus1[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase_plus1[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers. sps_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ], and sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ] apply to all sub-layers. When not present sps_sub_layer_ordering_info_present_flag is inferred to be equal to 0.

sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ]. When not present sps_max_dec_pic_buffering_minus1[ i ] is inferred to be equal to max_vps_dec_pic_buffering_minus1[ TargetOptLayerSetIdx ][ currLayerId ][ i ] of the active VPS, where currLayerId is the nuh_layer_id of the layer for which this SPS is active.

- sps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures with PicOutputFlag equal to 1 that can precede any picture with PicOutputFlag equal to 1 in the CVS in decoding order and follow that picture with PicOutputFlag equal to 1 in output order when HighestTid is equal to i. The value of sps_max_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. The value of sps_max_num_reorder_pics[ i ] shall be less than or equal to vps_max_num_reorder_pics[ i ] for each value of i. When sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ]. When not present sps_max_num_reorder_pics [ i ] is inferred to be equal to max_vps_num_reorder_pics[ TargetOptLayerSetIdx ][ i ] of the active VPS.

sps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of SpsMaxLatencyPictures[ i ], which specifies the maximum number of pictures with PicOutputFlag equal to 1 that can precede any picture with PicOutputFlag equal to 1 in the CVS in output order and follow that picture with PicOutputFlag equal to 1 in decoding order when HighestTid is equal to i.

- When sps_max_latency_increase_plus1[ i ] is not equal to 0, the value of SpsMaxLatencyPictures[ i ] is specified as follows:

- SpsMaxLatencyPictures[ i ] = sps_max_num_reorder_pics[ i ] + 	(7-1)		sps_max_latency_increase_plus1[ i ] − 1

- sps_max_latency_increase_plus1[ i ] − 1

- When sps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

- The value of sps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ i ] is not equal to 0, the value of sps_max_latency_increase_plus1[ i ] shall not be equal to 0 and shall be less than or equal to vps_max_latency_increase_plus1[ i ] for each value of i. When sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ]. When not present sps_max_latency_increase_plus1[ i ] is inferred to be equal to max_vps_latency_increase_plus1[ TargetOptLayerSetIdx ][ i ] of the active VPS.

# Conclusion

This document proposed that the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] be signalled in SPS only when nuh_layer_id is equal to 0. Corresponding change is also proposed to the semantics. We propose to adopt the proposal in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 4,” JCTVC-O1008, Geneva, October 2013.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 6,” JCT3V-F1004, Geneva, October 2013.